| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change proposal for UL Multi-BS MIMO section (15.5.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics Rong Ran, Wooram Shin ETRI | wbong@lge.com sunny_rr@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot 30a |  |
| Abstract | Text change proposal for Multi-BS MIMO section |  |
| Purpose | For member’s review and adoption into P802.16m_D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change proposal for UL Multi-BS MIMO section (15.5.2)

Wookbong Lee, Bin-Chul Ihm

LG Electronics

Rong Ran, Wooram Shin

ETRI

- Introduction

In this contribution, we modify current 802.16m draft standard (IEEE P802.16m/D2). Many texts in uplink single BS precoding with Multi-BS Coordination is not required since most of operation can be done by ABS without AMS notice.

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

- Proposed text change for 802.16m Multi-BS MIMO Amendment Text

Instruction to editor;

Black text: existing D2 text

Red and strike out text: for deletion

Blue and underline text: for addition

--------------------------------------------------------Text Start -----------------------------------------------------

15.5.2 UL Multi-BS MIMO

15.5.2.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques applicable with UL MIMO modes 2 and 4 with adaptive codebook precoding, with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

15.5.2.1.1 DL Signaling

Table 922 shows the control parameters for UL multi-BS MIMO.

Table 922- Control parameters for UL multi-BS MIMO

| Parameter | Description | Value | Control channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| UCCE | Uplink codebook coordination enable | 0b0: disable 0b1: enable | MAC control message | Indicates operation of PMI combination |
| UTRU | Uplink target resource unit | Cell and user cyclic shift separation [TBD] | Sounding commend A-MAP | Indicates AMSs to send sounding signals using all subcarriers within sounding allocation |
| PMImin | PMI minimizing interference |  | MAC control message AAI_MultiBS_PMI_COM message | Enable when UCCE=0b1 [Preferred PMI minimizing inter-cell interference for the neighboring ABS] |
| PCR | PMI combination ratio |  | AAI_MultiBS_PMI_COM message |  |

15.5.2.1.2 UL Signaling

Table 923 shows the feedback information for UL multi-BS MIMO.

Table 923-Feedback information for UL multi-BS MIMO

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is a member ID assigned to this ABS |
| Periodic feedback | Uplink sounding | Uplink sounding on the subband indicated by the ABS in sounding command A-MAP IE |
| Event driven feedback | RSSI difference level | Report RSSIdiff for ABS selecting AMS which should send sounding signals |

15.5.2.1.32 Operation procedure

Single BS precoding with Multi-BS Coordination is performed by combining two PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells. One of PMIs maximizes the transmission power of the serving cell; another minimizes interference generated to the neighboring cell.

The operation procedure of PMI combination follows the steps below:

1) PMI combination may be triggered by an ABS in an unsolicited manner. Based on channel measurements, ABSs may send UCCE (UL codebook coordination enable) in a MAC control message to an AMS indicating operation with PMI combination.

2) Once an AMS receives this MAC control message,, it shall send a periodic feedback header with the requested information. This information may include a Temp_BSID (a member ID) and the corresponding RSSI difference level.

AMSs shall calculate RSSI difference between two RSSIs received from the serving and neighboring ABSs. If the RSSI difference value of an AMS is smaller than a threshold T, the AMS can select the neighboring ABS which suffers the most interference from it via the following criterion:

(291)

where i is the index of the AMS, p is the index of the serving ABS and q is the index of the neighboring ABS. indicates the RSSI measured by the AMS i from the received preamble transmitted by the ABS p.

For determining the level of RSSI difference denoted as RSSIdiff, the following procedure shall be performed by the AMS i after selecting the neighboring ABS:

RSSIdiff=b1    if (RSSIq,i - RSSIp,i) > T0

RSSIdiff=b2    if (RSSIq,i - RSSIp,i) ≤ T0    (292)

Where T0 is larger than T. b1 and b2 are two quantized values of RSSI difference. Therefore, only one feedback bit is necessary.

3) After receiving feedback from multiple AMSs, the serving ABS should select several AMSs according to RSSI difference level and the number of AMSs supported by the sounding channel. The selected AMSs should be indicated by the serving ABS to send sounding signals using all subcarriers within sounding allocation via UTRU (unit target resource unit) in a sounding command A-MAP. The UL sounding channel for multiple AMSs of multiple cells can be multiplexed through cell cyclic shift separation and user cyclic shift separation indicated in a sounding command A-MAP IE.

41) PMI combination may be triggered by an ABS in an unsolicited manner. By using UL sounding signals, the serving and neighboring ABSs shallmay measure channels of selected AMSs. Upon the channel measurements, the serving and neighboring ABSs shallmay determine PMIPMI maximizing transmission power and PMIMinPMImin minimizing interference, respectively. The neighboring ABS shallmay inform the serving ABS of PMIMinPMImin through the backhaul network.

Two PMIs of the predefined codebook {wi} should be selected the following procedure:

PM=argmaxiHsWi2

PMI=argmaxiHsWi2

PMIMin=argmaxiHIWi2              					(293)

where i is the PMI.  and  denote information of channels from the AMS to its serving ABS and the neighboring ABS, respectively.

52) The serving ABS shall inform the PMI in A-MAP IE, and PMIMinPMImin and PCR (PMI combination ratio) in a MAC control messageAAI_MultiBS_PMI_COM message to the AMS.

The following procedure should be performed by the AMS to determine a PCR (PMI combination ratio), (K1, K2, K3)) based on two fixed RSSI difference criteria

(RSSIp,i - RSSIq,i) ≤ T1, α=K1

T1 ≤ (RSSIp,i - RSSIq,i) ≤ T2, α=K2

(RSSIp,i - RSSIq,i) > T3, α=K3                                   (294)

The transmitted precoder  combining two precoders (WPMI, WPMIMin) should be generated as follows

W=α⋅WPMI+(1−α)⋅WPMIMinα⋅WPMI+(1−α)⋅WPMIMin, 0≤α≤1

W=PCR⋅WPMI+(1−PCR)⋅WPMIMinPCR⋅WPMI+(1−PCR)⋅WPMIMin, 0≤PCR≤1         (295)

(296)

--------------------------------------------------------Text End -----------------------------------------------------