| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Extended Header Length indication (15.2.2.2) |  |
| Date Submitted | 2009-11-5 |  |
| Source(s) | Sungjin Lee, Youngbin Chang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. | Steve.lee@samsung.com +82-31-279-5248 |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a:Announcement” |  |
| Abstract | The contribution proposes a extended header length indication in the MAC header |  |
| Purpose | To be discussed and adopted by TGm for 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Extended Header Length Indication (15.2.2.2)

Sungjin Lee, Youngbin Chang, Jungje Son, Rakesh Taori

Samsung Electronics Co., Ltd.

- Introduction

In the current D2, there is no information carried in the MAC header to indicate how the total length of extended headers. This information is essential for fast processing or payload since the receiver side know exactly know the length of each part of a MPDU consist of GMH, Extended header and Payload. However, payload processing is delayed until parsing all EHs contents to figure when EHs are finished and payload starts based on current D2.

- Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

[Change the section 15.2.2.2 Extended Header formats in page 18, line 65 as follows:]

15.2.2.2 Extended header formats

The inclusion of Extended Header is indicated by EH bit in MAC Header. If the EH bit is set to ‘1’ a MAC header includes at least one extended header. When a extended header is present, 1 byte length Extended Header Length field is present aheading all Extended Headers. This Extended header length field indicates what the total length of all extended headers is.

Figure xx. Extended Header part format

| EH Length | Extended header 1 | Extended Header 2 | Extended Header 3 |
| --- | --- | --- | --- |

The Extended header format is defined in Table 655— and will be used unless specified otherwise.

-------------------------------  Text End  ---------------------------------------------------