| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Extended Header Length indication (15.2.2.2) |  |
| Date Submitted | 2009-11-5 |  |
| Source(s) | Sungjin Lee, Youngbin Chang, Jungje Son, Rakesh Taori, Anil Agiwal Samsung Electronics Co., Ltd. | steve.lee@samsung.com, anilag@samsung.com +82-31-279-5248 |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a:Announcement” |  |
| Abstract | The contribution proposes a extended header length indication in the MAC header |  |
| Purpose | To be discussed and adopted by TGm for 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Extended Header Length Indication (15.2.2.2)
Sungjin Lee, Youngbin Chang, Jungje Son, Rakesh Taori, Anil Agiwal

Samsung Electronics Co., Ltd.

- Introduction

In the current D2, there is no information carried in the MAC header to indicate how the total length of extended headers. This information is essential for fast processing or payload since the receiver side know exactly know the length of each part of a MPDU consist of GMH, Extended header and Payload. However, payload processing is delayed until parsing all EHs contents to figure when EHs are finished and payload starts based on current D2.

- Text to change in AWD

# Proposed text

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

[Change 1: Modify the section 15.2.2.2, page 18 (line 60-65), page 19 (line 1-26) as shown below]

---------------------------------------------------Text Start---------------------------------------------

15.2.2.2 Extended header formats

The inclusion of Extended Header group is indicated by EH bit in MAC Header. The extended header group (see figure xxx), when used, shall always appear immediately after the MAC header. Extended header group shall not be encrypted. The fields of the Eextended header group format is are defined in Table 655— and will be used unless specified otherwise.

Figure xxx Extended Header Group Format

Table 655 – Extended Header Group Format Fields

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended Headers { |  |  |
| Last | 1 | Last Extended Header indication: 0 = one or more extended header follows the current extended header unless specified otherwise; 1 = this extended header is the last extended header unless specified otherwise |
| EH Group Length |  | Extended header Length |
| Type | TBD | Type of extended header |
| Body Contents | Variable | Type dependent content |
| } |  |  |
| Extended header Group Length | 8 | The Extended header Group Length field indicates the total length of the extended header group, including all the extended headers and the Extended header Group length byte. |
| Extended header Type | 3 | Type of extended header as defined in Table 656. |
| Extended header Body | variable | The size of the extended header is determined by extended header type as specified in Table 656. The extended header including the extended header type is byte aligned. |

[Change 2: Delete ‘Last’ field from table 661, 662, 664, 665, 666 and 667]

---------------------------------------------------Text End---------------------------------------------