| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to DL Persistent A-MAP IE (15.3.6.5.2.8) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a: P802.16m/D2 |  |
| Abstract | This contribution proposes some modifications on Section 15.3.6.5.2.8 ( DL PA A-MAP IE ) |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to DL Persistent A-MAP IE (15.3.6.5.2.8)

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

In the previous session #63, it was agreed that the DL Persistent A-MAP IE [1] was defined to include the modulation constellation information of the paired user in DL MU-MIMO when 2 streams are transmitted in order to support Maximum Likelihood Demodulation (MLD).

However, the definition manner is different between the DL individual Persistent A-MAP IE (Table 812) and the DL composite Persistent A-MAP IE (Table 813) .

In this contribution, we would like to propose to modify the DL composite Persistent A-MAP IE to follow the same manner of the DL individual Persistent A-MAP.

# Reference

[1]	IEEE C802.16m-09/1891r5, Proposed Changes to the DL and UL Persistent Allocation A-MAP IEs in the IEEE 802.16m /D1

[2]	IEEE P802.16m/D2, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, October 2009

# Proposed text

- Editorial notes:

- Current text in IEEE P802.16m/D2

Remove text from IEEE P802.16m/D2

- Add new text to IEEE P802.16m/D2

-------------------------------  Start text proposal  ---------------------------------------------------

- Section 15.3.6.5.2.7 Group Configuration A-MAP IE, from page 383, line 47 to page 364, line38

Table 813 - DL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mp | 2 | Modulation constellation of the paired user 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| } else{ |  |  |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPS 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 streams with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| } |  |  |

-------------------------------  End text proposal  --------------------------------------------------