| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Direct Handover from AAI only ABS to WirelessMAN-OFDMA BS (15.2.6.4.2.4) |  |
| Date Submitted | 2009-11-16 |  |
| Source(s) | Yang Liu, Nan Li, Mary Chion, Yuqin Chen, Lei Zhang，Hongyun Qu ZTE Corporation Kelvin Chou,Yih-Shen Chen, I-Kang Fu, Paul Cheng MediaTek Inc. | E-mail: liu.yang8@zte.com.cn kelvin.chou@mediatek.com |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.6.4.2.4” |  |
| Abstract | This contribution proposes the texts for handover from AAI only ABS to WirelessMAN-OFDMA BS |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handover from AAI only ABS to WirelessMAN-OFDMA BS (15.2.6.4.2.4)

Yang Liu, Nan Li, Mary Chion, Yuqin Chen, Lei Zhang, Hongyun Qu

ZTE Corporation

Kelvin Chou,Yih-Shen Chen, I-Kang Fu, Paul Cheng

MediaTek Inc.

# 1. Introduction

In current P802.16m/D2, the procedure for direct HO is found as follows.

“An AMS served by a WirelessMAN-OFDMA BS may discover an AAI only ABS and decide to directly HO to this ABS. In this case, the AMS performs WirelessMAN-OFDMA Reference System HO procedure to the serving WirelessMAN-OFDMA BS per Section 6.3.21.2, and then performs WirelessMAN-OFDMA Advance System network re-entry procedure to the target ABS”

However, the revere direction HO for the scenario above is missing.

In this proposal, the procedure of handover from AAI only ABS to WirelessMAN-OFDMA BS is described.

# 2. Text Proposal

======================== Start of Proposed Text =====================

15.2.6.4.2.4 Handover from AAI only ABS to WirelessMAN-OFDMA BS

An AMS served by an AAI only ABS may discover and handover to a WirelessMAN-OFDMA BS. The existence of neighbor WirelessMAN-OFDMA BS is indicated by the AAI-NBR-ADV message from the serving AAI only ABS. The AMS scans neighbor WirelessMAN-OFDMA BS based on the indication information. After the target WirelessMAN-OFDMA BS is determined, the AMS leaves WirelessMAN-OFDMA Advance System per Section 15.2.6.3, and starts WirelessMAN-OFDMA System network re-entry procedure to the target BS. Specifically, In the AAI_HO-CMD sent in MZone for the HO procedure, HO reentry mode shall be 0 (unless it is a MC HO) and no dedicated ranging code is assigned for the network reentry in WirelessMAN-OFDMA BS.

The context mapping follows the procedure in 15.2.6.4.2.3.

============================== End of Proposed Text ===============