| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text change for deleting implicit QoS parameter switching (15.3.7.3.1) |  |
| Date Submitted | 2009-11-5 |  |
| Source(s) | Sungjin Lee, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. | Steve.lee@samsung.com +82-31-279-5845 |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a: Announcement” |  |
| Abstract | The contribution proposes to delete implicit switching of QoS parameter in the draft |  |
| Purpose | To be discussed and adopted by TGm for 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text change for

deleting implicit QoS parameter switching (15.3.7.3.1)

Sungjin Lee, Jungje Son, Rakesh Taori

Samsung Electronics Co., Ltd.

- Introduction

In the current D2, implicit switching of QoS parameter set is adopted conceptually but has no text describe how it works. It is lack of how to trigger the switching and how to synchronize the triggering time between ABS and AMS to be in the same state at the same time.

In this contribution, it is proposed to delete implicit QoS parameter switching related text from D2.

- Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

[Delete implicit adaptation from the table 704 in page 92 line 21 as follows]

| Adaptation method | TBD | - Implicit adaptation - Explicit adaptation sustained - Explicit adaptation, one time only |
| --- | --- | --- |

[Change the section 15.3.7.3.1 as follows in page 160 line 38 as follows]

There are two three adaptation methods:

- Implicit: upon detecting of certain traffic condition with respect to a pre-negotiation SF QoS parameter set, ABS automatically changes GPI and/or Grant size; or switches between GPI_primary/ Grant_Size_primary and GPI_secondary/Grant_Size_secondary if secondary SF QoS parameter set is defined.

[Delete implicit adaptation from the table 719 in page 161 line 37 as follows]

| Adaptation method (unsigned int) | - Implicit adaptation - Explicit adaptation, sustained - Explicit adaptation, one time only |
| --- | --- |

[Delete implicit adaptation from the table 720 in page 162 line 11 as follows]

| Adaptation Method = Implicit adaptation or Explicit adaptation - sustained or Explicit adaptation - one time only GrantSize_primary != x bytes (the newly defined bandwidth request header size | ertPS | - UGI = Primary GPI - and map equally all the other common QoS parameters between ertPS and aGP service |
| --- | --- | --- |
| Adaptation Method = Implicit adaptation or Explicit adaptation - sustained or Explicit adaptation - one time only GrantSize_primary = x bytes (the newly defined bandwidth request header size) | rtPS | - UPI = Primary GPI - and map equally all the other common QoS parameters between rtPS and aGP service |

-------------------------------  Text End  ---------------------------------------------------