| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification for Amendment Working Document on Channel Coding and HARQ(15.3.11.1.2) |  |
| Date Submitted | 2009-11-09 |  |
| Source(s) | Chien-Yu Kao, Tsung-Pao Chen, Jen-Yuan Hsu, Yu-Tao Hsieh, Pang-An Ting ITRI Hsi-Pin Ma, Yeong-Luh Ueng, Jen-Ming Wu, Yuan-Hao Huang NTHU Tony Chen TERADYNE | Voice: E-mail: ythsieh@itri.org.tw |
| Re: | P802.16m/D2 |  |
| Abstract | This contribution proposes one modification for the burst size in Section 15.3.11.1.2 |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification for Amendment Working Document on Channel Coding and HARQ

Chien-Yu Kao, Tsung-Pao Chen, Jen-Yuan Hsu, Yu-Tao Hsieh, Pang-An Ting

- ITRI

Hsi-Pin Ma, Yeong-Luh Ueng, Jen-Ming Wu, Yuan-Hao Huang

NTHU

Tony Chen

TERADYNE

- Introduction

In contribution C802.16itu-09_0031r1, it is mentioned that the code rate value greater than unity is possible due to rate matching. In this contribution, we would like to modify the burst sizein Section 15.3.11.1.2 of the AWD [1] to make the code rate value less than unity.

- Text Proposal

We suggest modifying the burst size (starting from line 26 of page 531) as follows.

---------------------------------------------------------Start of the Text--------------------------------------------------------

- 15.3.11.1.2 Burst partition

Only the burst sizes NDB listed in Table 908 are supported in the PHY layer. These sizes include the addition of CRC (per burst and per FEC block) when applicable. Other sizes require padding to the next burst size. When the burst size including 16 burst CRC bits exceeds the maximum FEC block size, NFB_MAX, the burst is partitioned into KFB FEC blocks.

- Table 908─Burst size

| idx | NDB (byte) | KFB | idx | NDB (byte) | KFB | idx | NDB (byte) | KFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1 | 23 | 90 | 1 | 45 | 1200 | 2 |
| 2 | 8 | 1 | 24 | 100 | 1 | 46 | 1416 | 3 |
| 3 | 9 | 1 | 25 | 114 | 1 | 47 | 1584 | 3 |
| 4 | 10 | 1 | 26 | 128 | 1 | 48 | 1800 | 3 |
| 5 | 11 | 1 | 27 | 144 | 1 | 49 | 1888 | 4 |
| 6 | 12 | 1 | 28 | 164 | 1 | 50 | 2112 | 4 |
| 7 | 13 | 1 | 29 | 180 | 1 | 51 | 2400 | 4 |
| 8 | 15 | 1 | 30 | 204 | 1 | 52 | 2640 | 5 |
| 9 | 17 | 1 | 31 | 232 | 1 | 53 | 3000 | 5 |
| 10 | 19 | 1 | 32 | 264 | 1 | 54 | 3600 | 6 |
| 11 | 22 | 1 | 33 | 296 | 1 | 55 | 4200 | 7 |
| 12 | 25 | 1 | 34 | 328 | 1 | 56 | 4800 | 8 |
| 13 | 27 | 1 | 35 | 368 | 1 | 57 | 5400 | 9 |
| 14 | 31 | 1 | 36 | 416 | 1 | 58 | 6000 | 10 |
| 15 | 36 | 1 | 37 | 472 | 1 | 59 | 6600 | 11 |
| 16 | 40 | 1 | 38 | 528 | 1 | 60 | 7200 | 12 |
| 17 | 44 | 1 | 39 | 600 | 1 | 61 | 7800 | 13 |
| 18 | 50 | 1 | 40 | 656 | 2 | 62 | 8400 | 14 |
| 19 | 57 | 1 | 41 | 736 | 2 | 63 | 9600 | 16 |
| 20 | 64 | 1 | 42 | 832 | 2 | 64 | 10800 | 18 |
| 21 | 71 | 1 | 43 | 944 | 2 | 65 | 12000 | 20 |
| 22 | 80 | 1 | 44 | 1056 | 2 | 66 | 14400 | 24 |

- The burst size index is calculated as

- , where

is a 5 bits index in A-MAP IE, and  is calculated based on the allocation size as shown in Table 909. The byte number of coded bitsthat shall be transmitted for allFEC blocks is defined by Equation (xx1):

(xx1)

Where:

is the modulation order defined in Table 910.

is equal to the product of MIMO rank that are allocated for the burst and the number of subframes when long TTI is used for the burst.

should be updated by Equation (xx2):

(xx2)

Where:

is the largest possible burst size in Table 908 such that.

The allocation size is defined as the number of LRUs multiplied by the MIMO rank that are allocated for the burst. In the case of long TTI, the number of LRUs is calculated as the number of subframes in TTI multiplied by the number of LRUs per subframe. For allocaton size 185~192, ISizeOffset = 31 is invalid.

---------------------------------------------------------End of the Text---------------------------------------------------------

- References

[1] IEEE 802.16m-09/D2, “IEEE 802.16m Amendment Working Document.”