| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to Compact MAC Header for the IEEE 802.16m (15.2.2.1.2/ 15.2.12.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim, JinSam Kwak LG Electronics Joey Chou, Xiangying Yang, Muthaiah Venkatachalam, Shantidev Mohanty, Roshni Srinivasan, Intel | E-mail: chegal@lge.com, ksryu@lge.com, E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | The contribution proposes texts for the Compact MAC header to be included in the 802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to Compact MAC Header for the IEEE 802.16m (15.2.2.1.2/ 15.2.12.2)

Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim, JinSam Kwak

LG Electronics

Joey Chou, Xiangying Yang, Muthaiah Venkatachalam, Shantidev Mohanty, Roshni Srinivasan

Intel

# Introduction

According to 16m/D2, a FPEH is always included in a MPDU which contains the payload for a single transport connection regardless of fragmentation, packing, and ARQ operation in order to ensure the HARQ MPDU reordering. 1 bytes Compact MAC header is defined for MPDUs with connections using persistent allocation or group resource allocation and it will contain the FPEH (minimum size is 2 bytes) to ensure the reordering of MPDUs using CMH. The others fields except the SN (sequence number) in FPEH are hardly used for MPDUs using CMH because fragmentation, packing, and ARQ don’t happen in packets using CMH. Thus, we propose new 2-bytes Compact MAC header as shown in figure 1.

Figure 1. Compact MAC header format

- Flow ID (Flow identifier)

  - Is used to identify multiple flows using CMH in an AMS and to indicate whether the CMH or AGMH is present in the front of MPDUs of the service flow .

  - During service connection setup procedure, an ABS determines whether the CMH or AGMH is used in MPDUs of the service. If the MAC Header type included in DSA message is set to 1, the Compact MAC header will present at the start of MPDUs of the connection. Otherwise, an AGMH will be present at the start of MPDUs of the connection.

# Performance analysis between GMH and CMH

# Comparison of Packet Sizes

  - Bandwidth: 10MHz (48 LRUs)

  - GMH +FPEH: 4 bytes

  - New CMH including SN: 2 bytes

  - Common Overhead: 9 bytes

    - ROHC: 3~ 4 bytes (maximum: 4bytes)

    - Security overhead (PN): 3 bytes

    - CRC: 2 bytes

  - Voice Codec: 12.2kbps AMR codec

Table 1. HARQ burst Size required for MPDUs using GMH and CMH

| AMR |  | Using GMH + FPEH |  | Using 2 bytes CMH |  |
| --- | --- | --- | --- | --- | --- |
| Codec Rate | Payload size | MPDU (bytes) | HARQ burst Size (bytes) | MPDU (bytes) | HARQ burst Size (bytes) |
| 12.2 kbps | 33 bytes (Active) | 46 | 50 | 44 | 44 |
| SID | 7Bytes(Inactive) | 20 | 22 | 18 | 19 |

# Comparison of VoIP capacity for each MCS

Table 2. Number of required LRUs for each MCS (44 bytes vs. 50 bytes, 12.2kbps)

| Nominal MCS | Required LRUs |  |
| --- | --- | --- |
|  | 44 bytes | 50 bytes |
| QPSK 31/256 | 16 | 18 |
| QPSK 48/256 | 10 | 12 |
| QPSK 71/256 | 7 | 8 |
| QPSK 101/256 | 5 | 6 |
| QPSK 135/256 | 4 | 4 |
| QPSK 171/256 | 3 | 4 |
| 16QAM102/256 | 3 | 3 |
| 16QAM128/256 | 2 | 3 |
| 16QAM155/256 | 2 | 2 |
| 16QAM185/256 | 2 | 2 |
| 64QAM135/256 | 2 | 2 |
| 64QAM157/256 | 1 | 2 |
| 64QAM181/256 | 1 | 1 |
| 64QAM205/256 | 1 | 1 |
| 64QAM225/256 | 1 | 1 |
| 64QAM237/256 | 1 | 1 |

Table 3. Comparison of VoIP capacity

| MCS | Number of VoIP users in 10MHz |  | Increasing rate of VoIP capacity by CMH (%) |
| --- | --- | --- | --- |
|  | 44 bytes | 50 bytes |  |
| QPSK 31/256 | 3 | 2.666667 | 12.5 |
| QPSK 48/256 | 4.8 | 4 | 20 |
| QPSK 71/256 | 6.857143 | 6 | 14.28571 |
| QPSK 101/256 | 9.6 | 8 | 20 |
| QPSK 171/256 | 16 | 12 | 33.33333 |
| 16QAM128/256 | 24 | 16 | 50 |
| 64QAM157/256 | 48 | 24 | 100 |

# Performance Analysis

Figure 2 shows the CDF of geometry SNR with Baseline Scenario in EMD. Assuming the scheduler in ABS reflects the reported CQI directly, we can simply obtain the average number of resources to be used by an AMS.

With 44byts of HARQ burst, 2.66 LRUs are required while 3.04 LRUs are required with 50 bytes of HARQ burst. We can obtain over 12% of resource saving gain by using CMH in AMR 12.2kbps.

Figure 2. CDF of geometry SINR to be used in this analysis

# References

- IEEE 802.16m-07/002r4, “802.16m System Requirements.”

- IEEE 802.16m-08/003r9, “IEEE 802.16m System Description Document”

- IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January 2009.

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

- IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

----------------------------------------------------- Start of the text ----------------------------------------------------------

[Remedy 1. Modify the Section “15.2.2.1.2 Compact MAC header” as follows:]

15.2.2.1.2 Compact MAC header (CMH)

The CMH is defined to support applications, such as VoIP, which uses small data packets and no ARQ. Extended header may be piggybacked on the CMH, if allowed by its length field. With the exception of extended headers, the CMH shall not require any other headers. The CMH is identified by the specific FID that is provisioned statically, or created dynamically via AAI_DSA-REQ/RSP.

The CMH format is defined in Table 654.

Table 654. CMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compact MAC Header () { | - |  |
| Flow ID | 4 | Flow Identifier |
| EH | 1 | Extended header presence indicator; When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 7 | This field indicates the length in bytes of MAC PDU including the CMH and extended header if present. |
| SN | 4 | MAC PDU payload sequence number increments by one for each MAC PDU (modulo 16). |
| } |  |  |

---------------------------------------------------------- End of the text ----------------------------------------------------------

----------------------------------------------------- Start of the text ----------------------------------------------------------

[Remedy 2. Modify the Section “15.2.12.2 Service Flow Management” as follows:]

15.2.12.2 Service Flow Management

In addition to the legacy scheduling services described in 6.3.5, AAI supports adaptation of service flow (SF) QoS parameters.One or more QoS parameter set(s) may be defined during the initial service negotiation, e.g., a mandatory primary SF QoS parameter set, and an optional secondary SF QoS parameter set, etc. Each SF QoS parameter set defines a set of QoS parameters. If multiple SF QoS parameter sets are defined, each of them corresponds to a specific traffic characteristic for the user data mapped to the same service flow. When QoS requirement/traffic characteristics for UL traffic changes, the ABS may autonomously perform adaptation by either changing the SF QoS parameters or switching among multiple SF QoS parameter sets. The AMS may also request the ABS to perform adaptation using explicit signaling. The ABS then allocates resource according to the adapted SF QoS parameters.

The value of FID field specifies the FID assigned by the ABS to a service flow of an AMS with a non-null AdmittedQosParamSet or ActiveQosParamSet. The 4-bit value of this field is used in BRs and in MAC PDU headers (AGMH or CMH). This field shall be present in a ABS-initiated DSA-REQ or DSC-REQ message related to establishing an admitted or active service flow. This field shall also be present in DSA-RSP and DSC-RSP messages related to the successful establishment of an admitted or active service flow.

AAI_DSA-REQ/RSP shall include the MAC Header type field to indicate which MAC header format is to be used for such service flow. When MAC Header type = 0, the given service flow shall use AGMH. When MAC Header type = 1, the given service flow shall use CMH.

If a service flow has been successfully admitted or activated (i.e., has an assigned FID) the SFID shall NOT be used for subsequent DSx message signaling as FID is the primary handle for a service flow.

---------------------------------------------------------- End of the text ----------------------------------------------------------

----------------------------------------------------- Start of the text ----------------------------------------------------------

[Remedy 3. Insert a following underlined row (“Field: MAC Header Type) into Table739]

Table 739—Service flow/Convergence Sublayer parameters

| Field | Size(bits) | Description |
| --- | --- | --- |
| MAC Header Type | 1 | Indicates whether AGMH or CMH is presented at the start of MPDUs of the service flow 0 = AGMH (Advanced Generic MAC Header) 1 = CMH (Compact MAC header) |

---------------------------------------------------------- End of the text ----------------------------------------------------------