| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS MAP (15.3.6.2.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak LG Electronics | chegal@lge.com, sixs@lge.com, emptylie@lge.com, ksryu@lge.com, ronnykim@lge.com, samji@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes texts for the E-MBS MAP to be included in the 802.16m Draft. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS MAP (15.3.6.2.3)

Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak

- LG Electronics

# Introduction

According to 16m/D2, the size of E-MBS MAP is of variable because an E-MBS-DATA IE contains resource information for multiple MBS bursts.

This contribution proposes the E-MBS Control Header that carries information to decode the E-MBS MAP (e.g. the allocation size of E-MBS MAP, MCS).

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Text #1 ---------------------------------------------------------------------------

15.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries some configuration information for enhanced multicast broadcast service for one EMBS Zone.  The E-MBS MAP is transmitted in the first several resources of the E-MBS region in the beginning of the MSI.

Table 788: E-MBS MAP Structure

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| EMBS-Zone-ID | 7 | The EMBS Zone ID to which this EMBS MAP apply |
| MSI Length (m) | [TBD] | The length of an MSI is 2m+3 superframes |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

The size of E-MBS MAP is indicated by the E-MBS Control Header. An E-MBS Control Header is the same physical structure as non-user specific A-MAP defined in 15.3.6.4.1 and is transmitted at the first several resources of the E-MBS region in the beginning of a MSI. The E-MBS MAP follows the E-MBS Control Header.

- Table 789 show the E-MBS Control header format.

- Table 789. E-MBS Control Header format

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS Control Header () { | - | - |
| Allocation size of E-MBS MAP | [7] | Number of LRUs which are used for E-MBS MAP transmission. |
| ISizeOffset | [5] | ISizeOffset |
| } |  |  |

--------------Proposed Text #1 ---------------------------------------------------------------------------

--------------Proposed Text #2 ---------------------------------------------------------------------------

15.3.6.3.3 Enhanced- Multicast Broadcast Services MAP (E-MBS MAP)

The E-MBS Control Header is encoded with a code rate of [1/12] using TBCC described in 15.3.11.2.

The encoded sequence of E-MBS Control Header is modulated using QPSK.

A 16-bit CRC is generated based on the contents of the E-MBS MAP.  Following randomization, the resulting sequence of bits shall be encoded by the CTC described in <<15.3.11. 1. >>

--------------Proposed Text #2 ---------------------------------------------------------------------------