| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Carrier Change Procedure for single carrier supported AMS (15.2.8.2.11) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Inuk Jung, Youngsoo Yuk, Ronny Kim LG Electronics Kelvin Chou MediaTek Inc. Yuqin Chen ZTE Corporation | Voice : +82-31-450-1876 E-mail: mehappy3@lge.com, ksryu@lge.com, cooper@lge.com, sixs@lge.com, ronnykim@lge.com kelvin.chou@mediatek.com chen.yuqin@zte.com.cn |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Carrier Change Procedure for single carrier supported AMS (15.2.8.2.11)

Eunjong Lee, Kiseon Ryu, Inuk Jung, Youngsoo Yuk and Yongho Kim

LG Electronics

Kelvin Chou

MediaTek Inc.

Yuqin Chen

ZTE Corporation

- Introduction

According to the current AWD, a single carrier supported AMS should perform inter-frequency HO procedure when changing its carrier between two carriers managed by a common MAC. However, this may lead to unnecessary procedure, such as network reentry. Therefore, this contribution proposes a carrier change procedure through AAI_CM-CMD message that a single carrier supported AMS can simply change its carrier to one of the fully configured carriers managed by a common MAC at multi-carrier supported ABS.

- Discussions

Multi-carrier operation of the current AWD describes the primary carrier change procedure for multi-carrier supported AMS. Since this procedure has been designed for multi-carrier supported AMS, a single carrier supported AMS cannot correctly decode the AAI_CM-CMD message. Under the multi-carrier operation, however, the single carrier supported AMS can be also supposed to change its carrier to one of the fully configured carriers managed by a common MAC at multi-carrier supported ABS, but current multi-carrier operation doesn’t support the carrier change for single carrier supported AMS. According to the current AWD, they should perform inter-frequency HO to change their carrier between two carriers managed by a common MAC. This would lead to unnecessary procedure (e.g., network reentry procedure).

Therefore, we’d like to define carrier change procedure for single carrier supported AMS which is performed in case of action code = ‘11’ of AAI_CM-CMD message. A single carrier supported AMS is able to communicate with only one carrier and cannot decode current AAI_CM-CMD message because they don’t use any concept for multi-carrier operation, such as assigned carriers, logical carrier index or primary/secondary carrier. Therefore, in order for single carrier supported AMS to perform the carrier change procedure, it is necessary to design the AAI_CM-CMD message for single carrier supported AMS. So, in this contribution, we design the AAI_CM-CMD message for carrier change and propose some descriptions for the carrier change procedure for single carrier supported AMS.

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

[Modify the text and the Figure at line 36 on page 97 as follows:]

15.2.8 Multi-carrier operation

15.2.8.2.11.4 Carrier Change Procedure for single carrier supported AMS

The ABS may instruct the single carrier supported AMS, through the AAI_CM-CMD MAC control message on the current carrier, to change its carrier to one of the fully configured carriers within the same ABS for load balancing purpose, carriers' varying channel quality or other reasons. When a single carrier supported AMS receives the AAI_CM-CMD MAC control message indicating action code = ‘11’, the AMS transmits the AAI_CM-IND MAC control message before action time. Then AMS performs hardware reconfiguration (e.g. RF center frequency) and switches to target carrier. If CDMA_RNG_FLAG in AAI_CM-CMD message is set to ‘1’, the AMS shall perform the dedicated ranging procedure with target carrier after action time. The AMS may transmit a bandwidth request to notify the successful change to the target carrier to the ABS. If the AMS has UL data to be transmitted, it follows the normal bandwidth request procedure. If the AMS does not have any UL data, it transmits a bandwidth request with zero bandwidth request size. The ABS should transmit data and control signal after a bandwidth request is received from the AMS through the target carrier. Given that a common MAC manages both serving and target carriers, network re-entry procedures at the target carrier is not required. The ABS may provide the system information of the target carrier that is different from the serving carrier via the serving carrier. The ABS may direct an AMS to change the carrier without scanning.

Figure xx. An example of change carrier procedure for single carrier supported AMS

15.2.8.2.11.45 MAC Control Messages for Carrier Management

15.2.8.2.11. 45.1 Carrier Management Command (AAI_CM-CMD) MAC control message

Table 726—AAI_CM-CMD MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management Command message format(){ |  |  |
| Control Message Type | 8 | AAI_CM-CMD |
| Action code | 2 | 00: Secondary carrier management 01: Primary carrier change 10: Carrier switching 11: reservedCarrier Change for single carrier supported AMS |
| If (Action code == 00){ |  | This message is for secondary carrier activation and/or deactivation |
| Indication Type | 2 | For Activation and/or Deactivation Bit #0: ‘0’: No action, ‘1’: Activation Bit #1: ‘0’: No action, ‘1’: Deactivation |
| If (Indication Type #0 == 1) |  |  |
| Num of target carrier | [TBD] | The number of newly activated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for activation |
| Activation of DL/UL | 1 | ‘0’: Both DL/UL are activated ‘1’: DL is activated but UL is not activated |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |
| If (Indication Type #1 == 1){ |  |  |
| Num of target carrier | [TBD] | The number of deactivated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for deactivation |
| Deactivation of DL/UL | 1 | ‘0’: Both DL/UL are deactivated ‘1’: UL is deactivated but DL is kept active |
| } |  |  |
| } |  |  |
| } |  |  |
| If (Action code == 01){ |  | This message is for primary carrier change. |
| Target carrier index | [TBD] | Target carrier index for primary carrier change |
| Action Time | [TBD] | LSB bits of Superframe number at the time to switch to the target carrier. |
| Next state of serving primary carrier | 1 | ‘0’: Serving carrier will be deactivated after primary carrier change ‘1’: Serving carrier is kept active after primary carrier change |
| } |  |  |
| If (Action code == 10){ |  | This message is for secondary carrier switching. |
| Target carrier index | [TBD] | Target carrier index for carrier switching |
| Switching Mode | 1 | Periodic or event-triggered switching 0: Periodic switching 1: Event-triggered switching |
| Start Time | [TBD] | TBD LSB bits of Superframe number at the start time of carrier switching |
| Switching Interval | [TBD] | Number of superframes during which an AMS stays at the target secondary carrier before returning to the primary carrier |
| If (Switching Mode == 0){ |  |  |
| Switching Period | [TBD] | Number of superframes between consecutive switching interval |
| } |  |  |
| } |  |  |
| If (Action code == 11){ |  | This message is for carrier change for single carrier supported AMS. |
| Physical carrier index of target carrier | [TBD] | Physical carrier index of target carrier |
| Action Time | [TBD] | LSB bits of Superframe number at the time to switch to the target carrier. AMS shall disconnect connection with serving carrier at this time. |
| CDMA_RNG_FLAG | 1 | CDMA Ranging flag for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Dedicated HO ranging is required for the target carrier |
| If (CDMA_RNG_FLAG == 1){ |  |  |
| Dedicated CDMA ranging code/opportunity | [TBD] |  |
| } |  |  |
| } |  |  |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------