| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Dynamic Allocation of E-MBS Region (15.3.6.3.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak LG Electronics | chegal@lge.com, sixs@lge.com, emptylie@lge.com, ksryu@lge.com, ronnykim@lge.com, samji@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | Proposed text change for dynamic allocation of E-MBS region in non-macro diversity mode |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Dynamic Allocation of E-MBS Region (15.3.6.3.3)

Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak

- LG Electronics

# Introduction

According to IEEE P802.16m/D2, the size of E-MBS MAP is of variable because an E-MBS-DATA IE contains resource information for multiple MBS bursts.

This contribution proposes the E-MBS Region A-MAP to inform the location of E-MBS region with dynamic multiplexing of E-MBS region with normal data region in case of non-macro diversity mode.

# Dynamic Allocation of E-MBS Region in non-macro diversity mode

Without macro-diversity, the DRU resource region should be used for obtaining diversity gain. Because DRU region is hardly used with fixed allocation, the dynamic allocation should be applied.

Figure 1 shows the proposed scheme. By using E-MBS zone STID which is given during service creation, an AMS may find E-MBS Region A-MAP.

The proposed scheme has following advantages.

- Diversity gain without MBSFN

- Less impact for scheduling of Unicast Data

- DRU and/or CRU can be used for E-MBS region

- No restriction for E-MBS data allocation

Though the drawback of the proposed scheme is additional effort to decode E-MBS Region A-MAP at every MSI, it’s not a big problem considering the gains.

Figure 1. Dynamic allocation of E-MBS region

# Text proposal for the IEEE 802.16m amendment

- --------------Proposed Text #1 ---------------------------------------------------------------------------

15.2.3.41 AAI_E-MBS-CFG Message

An AAI_E-MBS-CFG message is broadcast by the ABS to inform

Table AAI_E-MBS-CFG message

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| AAI_MZ_ID () { | - | - |
| Number of E-MBS zones (z) | [2] | The number of E-MBS zones the ABS is associated. |
| For (i=0; i<z; i++) { | - |  |
| E-MBS zone identifier | 7 | An MBS zone identifier with which ABS is associated. |
| E-MBS Zone STID | 12 | It is used for AMS to decode the E-MBS Region A-MAP IE. |
| } |  |  |
| E-MBS Region Configuration Indicator | [3] | The mapping between value of this index and E-MBS Region configuration is listed in Table X. |
| } |  |  |

E-MBS Region Configuration Indicator

Table x shows of the description of E-MBS Region configuration indicator.

- Table X— E-MBS Region Configuration Indicator

| - E-MBS Region Configuration Indicator | - Notes |
| --- | --- |
| - 000 | - No E-MBS region |
| - 001 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 010 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 011 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 100 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 101 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 110 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 111 | - Dynamic Allocation |

- --------------Proposed Text #1 ---------------------------------------------------------------------------

--------------Proposed Text #2 ---------------------------------------------------------------------------

- 15.3.6.3.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

A 16-bit CRC is generated based on the contents of the E-MBS MAP.  Following randomization, the resulting sequence of bits shall be encoded by the CTC described in <<15.3.12. 2. >>

If the dynamic allocation is used, the E-MBS Region A-MAP IE is used for indicating the E-MBS region in the subframe which contains E-MBS MAP. An E-MBS Region A-MAP IE is a type of the unicast control Assignment A-MAP IEs and contains the information to decode an E-MBS MAP (e.g., Resource allocation size, MCS, etc.). E-MBS Region A-MAP IE is transmitted at the Assignment A-MAP region of the first frame of MSI and CRC of the A-MAP IE is masked by the E-MBS Zone STID.

--------------Proposed Text #2 ---------------------------------------------------------------------------

--------------Proposed Text #3 ---------------------------------------------------------------------------

- 15.3.6.5.2.15 E-MBS Region A-MAP IE

Table xxx1 shows E-MBS Region A-MAP IE format which contains the information to decode the E-MBS MAP IE for the corresponding E-MBS Zone.

- Table xxx1. E-MBS Region A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS Region A-MAP IE () { | - | - |
| E-MBS IE Type | [4] | E-MBS Region A-MAP IE |
| Resource Indexing for E-MBS Region Size | 11 | Indicates the size of E-MBS region |
| Isizeoffset | [5] | MCS of E-MBS MAP |
| Resource Indexing for E-MBS MAP | [7] |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | CRC which is masked by E-MBS Zone STID |
| } | - | - |

A 16 bit CRC is generated based on the contents of the E-MBS Region A-MAP IE. The CRC is masked by the E-MBS Zone Station ID.

--------------Proposed Text #3 ---------------------------------------------------------------------------