| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the message for AMS scanning of neighbor Femto ABSs |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon | E-mail: sjjung@etri.re.kr |
| Re: | IEEE 802.16m-09/0057: IEEE 802.16 Working Group Letter Ballot Recirc #30a:Announcement |  |
| Abstract | Proposed text to modify the scanning related messages |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the messages for AMS scanning of neighbor Femto ABSs

Soojung Jung, Eunkyung Kim, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

# 1. Introduction

According to the section “15.4.8.1.2 AMS scanning of neighbor Femto ABSs” of “IEEE P802.16m/D2” [1], an AMS may report preamble indices or BS IDs and request the AAI_NBR-ADV message by sending AAI_NBR-REQ or AAI_SCN-REP with neighbor request indication. Upon reception of these messages, the ABS may unicast an AAI_NBR-ADV that includes a list of Femto ABSs which is formed based on the reported preamble index or any other recommended Femto ABSs based on location information. And the AMS may request additional scanning opportunity by sending AAI_SCN-REQ including the detected SA-preamble index which may not be provided in the broadcast AAI_NBR-ADV message.

However, the neighbor request indication is not currently included in the AAI_SCN-REP message format of the current “IEEE P802.16m/D2” document. In addition, there is no mention for the SA-preamble index in the scanning related messages format. Therefore, we propose to add the following text for AAI_SCN-REQ/RSP/REP messages.

# 2. Text Proposal

------------------------------------------------------------Start of the Text--------------------------------------------------------

[add the following rows to Table 681]

| Name | value | Usage |
| --- | --- | --- |
| N_Recommended_Preamble_Index_Indication | Indicated whether N_Recommended_Preamble_Index field is included in this message |  |
| N_Recommended_Preamble_Index | Number of SA- preamble index to be scanned | Shall be included when N_Recommended_Preamble _Index_Indication is set to 1 |
| Recommended_Preamble_Index | SA- preamble index for scanning | Shall be included when N_Recommended_Preamble_Index > 0 |

[add the following rows to Table 682]

| Name | value | Usage |
| --- | --- | --- |
| N_Recommended_Preamble_Index_Indication | Indicated whether N_Recommended_Preamble_Index field is included in this message |  |
| N_Recommended_Preamble_Index | Number of SA- preamble index to be scanned | Shall be included when N_Recommended_Preamble _Index_Indication is set to 1 |
| Recommended_Preamble_Index | SA- preamble index for scanning | Shall be included when N_Recommended_Preamble_Index > 0 |

[add the following rows to Table 683]

| Name | value | Usage |
| --- | --- | --- |
| N_Recommended_Preamble_Index_Indication | Indicated whether N_Recommended_Preamble_Index field is included in this message |  |
| N_Recommended_Preamble_Index | Number of SA preamble index to be scanned | Shall be included when N_Recommended_Preamble _Index_Indication is set to 1 |
| Recommended_Preamble_Index | SA preamble index for scanning | Shall be included when N_Recommended_Preamble_Index > 0 |
| Neighbor_request_indication | Request indication for system information and the list of the neighbor Femto ABSs |  |

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE P802.16m/D2, “Advanced Air Interface(Draft),” Oct. 2009.