| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Clarifications for the Usage of MaxMt (15.3.6.5.2.11) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics Wookbong Lee, Bin-Chul Ihm LG Electronics | E-mail: d.mazzarese@samsung.com wbong@lge.com |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | The contribution proposes clarifications for the usage of MaxMt in the Feedback Allocation A-MAP IE (15.3.6.5.2.11) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Clarifications for the Usage of MaxMt (15.3.6.5.2.11)

David Mazzarese, Hokyu Choi

Samsung Electronics Co., Ltd.

Wookbong Lee, Bin-Chul Ihm
LG Electronics

- Introduction

This contribution provides clarifications for the usage of MaxMt in the Feedback Allocation A-MAP IE. Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D2, Oct 2009.

- Text Proposal

< Insert the following text on page 395 line 31 in section 15.3.6.5.1.11 (Feedback Allocation A-MAP IE)>

----------------------------------------- Start of the Proposed Text --------------------------------------------

For SU MIMO feedback modes, the AMS should apply an additional constraint in the choice of its STC rate feedback: Mt ≤ min(Nt, Nr, MaxMt), where Nt is the number of transmit antennas at the ABS, Nr is the number of receive antennas at the AMS, and MaxMt is indicated in the Feedback Allocation A-MAP IE.

The DL Basic Assignment A-MAP IE sent by the ABS can indicate any value of Mt as long as it complies with Mt ≤ min(Nt, Nr). There is no constraint on the assigned Mt relative to MaxMt for an allocation outside the open-loop region. Mt shall be equal to MaxMt for an allocation in the open-loop region with MaxMt streams.

MaxMt should be interpreted as a function of Measurement Method Indication and the MIMO Feedback Mode.

If Measurement Method Indication == 0b0:

  - With an MFM supporting MU MIMO operation, MaxMt indicates the maximum number of users the AMS should assume will be scheduled in future MU MIMO allocations. Based on this information, the AMS may make a better estimation of the multiuser interference in the CQI. MaxMt is set to one, then the MS shall assume it will be paired with no other AMSs when it calculates CQI. If MFM 5 is indicated in the Feedback Allocation A-MAP IE, MaxMt indicates the unitary matrix from which the AMS should feedback its preferred stream index on a given physical subband.

  - With an MFM supporting SU MIMO operation, MaxMt indicates the maximum STC rate allowed in the feedback report of the AMS. The ABS also uses MaxMt to make sure that the STC rate feedback is in accordance to STC rate ≤ min(Nt,Nr) for the AMS. For an MFM requesting OL MIMO feedback for operation with a diversity allocation, setting MaxMt=2 enforces feedback for DLRU, otherwise the AMS may feedback STC rate up to 4.

If Measurement Method Indication == 0b1:

- MaxMt indicates the rank of the open-loop region for which the feedback shall be provided. The open-loop region type can be deduced from MaxMt and MFM. MaxMt indicates the pilot pattern used in the OL region in which CQI measurements should be taken by the AMS.

----------------------------------------- End of the Proposed Text --------------------------------------------