| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Advanced Preamble design for supporting tone dropping (15.3.6.1) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Chiwoo Lim, Sung-Eun Park, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. Jinsoo Choi, HanGyu Cho, Jin Sam Kwak LG Electronics Yang-Seok Choi, Jiacheng Wang Intel Corporation | chiwoo.lim@samsung.com, se.park@samsung.com {emptylie, hgcho, samji}@lge.com yang-seok.choi@intel.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 Section: 15.3.6.1 Advanced Preamble |  |
| Abstract | The contribution proposes the text changes to Advanced Preamble design of the 802.16m draft amendment, IEEE P802.16m/D2. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to Advanced Preamble design 
for supporting tone dropping (15.3.6.1)

Chiwoo Lim, Sung-Eun Park, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi
Samsung Electronics Co., Ltd.

Jinsoo Choi, HanGyu Cho, Jin Sam Kwak
LG Electronics

Yang-Seok Choi, Jiacheng Wang

Intel Corporation

- Introduction

The contribution proposes the text of Advanced Preamble with tone dropping support to be included in the 802.16m amendment. The current draft amendment [1] has tone dropping description as “Tone dropping based on 10 and 20 MHz systems can be used to support other various bandwidths”. However, the Advanced Preamble design only supports regular nominal channel bandwidth such as 5,10 and 20MHz，so there is need of Advanced Preamble design to support irregular nominal channel bandwidth with tone dropping.

- Proposal for Advanced Preamble for supporting tone dropping

Assumption

1) Irregular nominal channel bandwidth in 5~20 MHz uses the same subcarrier spacing of 10MHz or 20MHz.

2) Irregular nominal channel bandwidth in 5~10 MHz uses 1024-FFT, and Irregular nominal channel bandwidth in 10~20MHz uses 2048-FFT.

3) PA-Preamble carries irregular nominal channel bandwidth information.

4) SA-Preamble occupies the available bandwidth as large as possible, while maintaining the scalable structure.

Advanced Preamble design concept

For regular nominal channel bandwidth, the SA-Preamble has a scalable structure as shown in Figure 1. That is, the 5MHz sequence is reused for 10 and 20MHz bandwidth by repetition of 8 sub-blocks {A, B, C, D, E, F, G, and H}. Each sub-block consists of 18 bits deployed over 54 sub-carriers.

Figure 1─SA-Preamble structure

For irregular nominal channel bandwidth, the SA-Preamble is obtained by dropping the farthest sub-blocks of the regular bandwidth from DC subcarrier on both sides.

Base on the sub-block dropping design, the irregular nominal channel bandwidth range and corresponding SA-Preamble are listed in Table 1.

Table 1—SA-Preamble based on sub-block dropping

| Irregular Nominal Channel Bandwidth Range, x (MHz) | Sub-blocks of SA-Preamble after tone dropping |
| --- | --- |
| 5 < x < 6.25 | ABCD EFGH |
| 6.25 ≤ x < 7.5 | H ABCD EFGH A |
| 7.5 ≤ x < 8.75 | GH ABCD EFGH AB |
| 8.75 ≤ x < 10 | FGH ABCD EFGH ABC |
| 10 < x < 11.25 | EFGH ABCD EFGH ABCD |
| 11.25 ≤ x < 12.5 | D EFGH ABCD EFGH ABCD E |
| 12.5 ≤ x < 13.75 | CD EFGH ABCD EFGH ABCD EF |
| 13.75 ≤ x < 15 | BCD EFGH ABCD EFGH ABCD EFG |
| 15 ≤ x < 16.25 | ABCD EFGH ABCD EFGH ABCD EFGH |
| 16.25 ≤ x < 17.5 | H ABCD EFGH ABCD EFGH ABCD EFGH A |
| 17.5 ≤ x < 18.75 | GH ABCD EFGH ABCD EFGH ABCD EFGH AB |
| 18.75 ≤ x < 20 | FGH ABCD EFGH ABCD EFGH ABCD EFGH ABC |

- References

[1] IEEE P802.16m/D2, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” October 2009.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Insert the following text under the Table 780 in page 314]

The sequences of indexes from 3 to 9 in Table 780 are reserved for the irregular nominal channel bandwidth to support tone dropping.

[Remedy-2: Modify line 36-38 in page 319 as follows]

Let “block” denote 8 consecutive sub-blocks {E, F, G, H, A, B, C, D}. The algorithm to assign the preamble blocks to multiple transmit antennas where the tone dropping is not applied and the number of antennas is power of 2 can be described as follows. Let:

[Remedy-3: Insert the following text under the Table 783 in page 321]

SA-Preamble sequences for tone dropping support are obtained by dropping the farthest sub-blocks of the reference bandwidth from DC subcarrier on both sides.

For support of irregular nominal channel bandwidth by tone dropping, the dropping unit of SA-Preamble is two sub-blocks. The granularity of nominal channel bandwidth is 1.25MHz. Table x shows the allocation of sequence sub-blocks for SA-Preamble according to the different range of available nominal channel bandwidth where Nsi is the total number of sub-blocks in irregular nominal channel bandwidth.

Table x—Allocation of sequence sub-blocks for tone dropping support

| Irregular Nominal Channel Bandwidth Range, x (MHz) | Nsi | Allocation of sequence sub-blocks |
| --- | --- | --- |
| 5 < x < 6.25 | 8 | ABCD EFGH |
| 6.25 ≤ x < 7.5 | 10 | H ABCD EFGH A |
| 7.5 ≤ x < 8.75 | 12 | GH ABCD EFGH AB |
| 8.75 ≤ x < 10 | 14 | FGH ABCD EFGH ABC |
| 10 < x < 11.25 | 16 | EFGH ABCD EFGH ABCD |
| 11.25 ≤ x < 12.5 | 18 | D EFGH ABCD EFGH ABCD E |
| 12.5 ≤ x < 13.75 | 20 | CD EFGH ABCD EFGH ABCD EF |
| 13.75 ≤ x < 15 | 22 | BCD EFGH ABCD EFGH ABCD EFG |
| 15 ≤ x < 16.25 | 24 | ABCD EFGH ABCD EFGH ABCD EFGH |
| 16.25 ≤ x < 17.5 | 26 | H ABCD EFGH ABCD EFGH ABCD EFGH A |
| 17.5 ≤ x < 18.75 | 28 | GH ABCD EFGH ABCD EFGH ABCD EFGH AB |
| 18.75 ≤ x < 20 | 30 | FGH ABCD EFGH ABCD EFGH ABCD EFGH ABC |

---------------------------------------------------  Text End  ---------------------------------------------------