| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Constellation rearrangement (15.3.11.4.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. | se.park@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 Section: 15.3.6.1 Advanced Preamble |  |
| Abstract | The contribution proposes the text changes to the constellation rearrangement of the 802.16m draft amendment, IEEE P802.16m/D2. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to Constellation Rearrangement (15.3.11.4.2)

Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the clean-up text of the constellation rearrangement (CoRe) to be included in the 802.16m amendment. In the current draft amendment [1], the constellation rearrangement version (CRV) is changed to the other version of CoRe in both downlink and uplink HARQ operation whenever the transmitted bits wraparound at the end of the circular buffer. There’s an ambiguity to determine the value of CRV by introducing the circular shift and reading represented by Nshift,i in bit selection algorithm. There’s also an ambiguity when we interpret the meaning of “wraparound” in uplink. So this contribution proposes the clear text that can avoid any ambiguities.

- CRV determination when Circular Shift and Reading is applied

When the circular shift and reading (CSR) represented by Nshift,i in bit selection algorithm is applied, there’s an ambiguity to determine the value of CRV. For example, Figure 1 shows the transmit order when SPID=3 in downlink. Let’s assume that CRV is explicitly indicated as zero.

Figure 1 Transmit order at SPID=3 in DL

It is unclear that the indicated CRV represents the version of portion 2 in orange color, or the version of portion 1 in yellow color in Figure 1.

In this contribution, it is proposed that the CRV is determined assuming that Nshift,i is 0 for all subpakcet i. In that case, CRV=0 indicates the version of portion 2 in orange color. Then the CRV of portion 3 and portion 1 will be 1 because the wraparound buffer occurs at the end of the portion 2.

- CRV determination in uplink

In current amendment text, there’s some ambiguity to interpret the starting value of CRV for uplink implicitly.

In uplink HARQ operation, the information on CRV is implicitly known at AMS and ABS, and the CRV of the first HARQ subpacket shall be 0. The CRV will be changed to other version only whenever the transmitted bits wraparound at the end of circular buffer.

Figure 2 IR HARQ transmission example in uplink

Figure 2 shows an example of IR HARQ transmission in uplink. Definitely the CRVs of 0th and 1st subpackets are 0. But the CRV of 2nd subpacket is unclear because this transmission doesn’t meet the end of circular buffer during the transmission. To clarify it, we propose the starting value of CRV for uplink as assuming Nshift,i = 0 for all subpacket i. In other word, we propose to determine that CRV=1 for 2nd subpacket in Figure 2. Then the CRV of the first HARQ subpacket is always 0.

- References

[1] IEEE P802.16m/D2, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” October 2009.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy: Change subsection 15.3.11.4.2 Constellation rearrangement from line 32, page 542 to line 22, page 543 as follows]

15.3.11.4.2 Constellation rearrangement

Two constellation re-arrangement (CoRe) versions shall be supported. CostellationThe constellation rearrangement only applies to 16QAM and 64QAM. In case of QPSK, it is transparent.

Table 916 and Table 917 describes the operations that produce two different CoRe versions for 16QAM and 64QAM according to the number of MIMO streams, respectively, so that the four bits in a 16QAM symbol (the six bits in a 64 QAM) are of the same resilience. In other words, the two bits of higher quality at CoRe-version 0 are of lower quality at CoRe-version 1 while the two bits of lower quality at CoRe-version 0 are of higher quality at CoRe-version 1.

Table 916—Constellation rearrangement version (rankMIMO stream = 1)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - |
| 16 QAM | 4 | 1 | b3 | b2 | b1 | b0 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 |
| 64 QAM | 6 | 1 | b5 | b4 | b3 | b2 | b1 | b0 |

Table 917—Constellation rearrangement version (rankMIMO stream > 1)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FirstEven symbol |  |  |  |  |  | SecondOdd symbol |  |  |  |  |  |  |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - |  | b4 | b5 | b6 | b7 | - | - |
| 16 QAM | 4 | 1 | b1 | b4 | b3 | b6 | - | - |  | b5 | b0 | b7 | b2 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 |  | b6 | b7 | b8 | b9 | b10 | b11 |
| 64 QAM | 6 | 1 | b2 | b7 | b0 | b5 | b10 | b3 |  | b8 | b1 | b6 | b11 | b4 | b9 |

In DL HARQ operation, CRV (Constellation Rearrangement Version) is signaled for each subpacket to an AMS as a starting value for CRV. In UL HARQ operation, the information on CRVs is implicitly known at AMS and ABS and the starting value for CRV of the first HARQ subpacket i shall be 0. The CRV shall be changed to the other version of CoRe in DL and UL HARQ operation whenever the transmitted bits wraparound at the end of circular buffer.

In DL and UL HARQ operation, the CRV is determined assuming that Nshift,i is 0 for all subpakcet i.

---------------------------------------------------  Text End  ---------------------------------------------------