| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change of Broadcast Control Information Elements (Section 15.3.6.5.1.2) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Chao-Wang Huang, Cheng-Ming Chen, Yun-Yi Shih, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI | Voice: +886-3-5914533 E-mail: huangcw@itri.org.tw |
| Re: | P802.16m/D2 / Area : Chapter 15.3.6 Downlink Control Structure |  |
| Abstract | The contribution proposes the modification of table 791 to be included in P802.16m/D2. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change of Broadcast Control Information Elements (Section 15.3.6.5.1.2)

Chao-Wang Huang, Cheng-Ming Chen, Yun-Yi Shih, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

# Introduction

This contribution proposes text modification to the section of broadcast control information elements IEEE P802.16m/D2 [1]. In particular, we propose to modify some entries relating to ranging signal in Table 791 of [1] to make it more accurate and clear. The proposed text is developed in order to be readily combined with IEEE Std 802.16-2009 [2]. It is compliant to the 802.16m SRD [3] and the 802.16m SDD [4].

The suggested modification in Table 791 include following:

- In order to clarify the content of RNG codes information, we propose to replace it by further dividing this entry into “The number of cyclic shifted codes per root index” and “The start root index of ZC code”. According to Table 876 of [1], 2 bits are required to represent the number of cyclic shifted codes per root index. Also, since the definition for the start root index of ZC code (page 486 of [1]) shows that it solely depends on the value of k, where k = 0, 1, 2,…, 15, the representation of such index requires 4 bits.

- According to Table 877 of [1], there are 16 ranging code partitions in total, so the size for ranging code/subset partition configuration shall be 4 instead of 3.

- The current version of Table 791 does not include the information of ranging channel format. Thus, we suggest adding a new IE to indicate whether format 0 or format 1 (in Table 871 of [1]) is employed as the structure of ranging channel.

# Reference

[1] IEEE P802.16m/D2, “IEEE 802.16m DRAFT Amendment Document,” October 2009

[2] IEEE Std 802.16-2009, “IEEE Standard for Local and metropolitan area networks: Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

[3] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements,” January 2009.

[4] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document,” May 2009.

- Text proposal

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: current text in the subclause 15.3.6.5.1.2

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

[  ------------------------------------ Text Proposal  ----------------------------------  ]

[Modify the following text in the 15.3.6.5.1.2 S-SFH SP1 IE format]

- 15.3.6.5.1.2 S-SFH IE

# TableTTable 791 S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| Start superframe offset where new SP1 information is used | 2 |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSB of 48 bit ABS MAC ID | 12 | Specifies the 12 least bit of ABS ID |
| Number of UL ACK/NACK channels per HARQ feedback region | 2 | Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 MHz band, 6, 12, 18, 24 For 10 MHz band, 6, 12, 24, 30 For 20 MHz band, 12, 24, 48, 60 |
| Number of UL ACK/NACK channels per HF-A-MAP region | 2 | Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 MHz band, 4, 8, 12, 16 For 10 MHz band, 8, 16, 24, 32 For 20 MHz band, 16, 32, 48, 64 |
| Power control channel resource size indicator | 2 |  |
| Non-user specific A-MAP location | 1 | Reuse 1 or reuse 3 |
| A-A-MAPMCS selection | 1 |  |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20 MHz, DL_CAS_SB0 (4 bits), DL_CAS_MB0 (6 bits), DL_CAS_SBi (3 bits) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20 MHz, UL_CAS_SB0 (4 bits), UL_CAS_MB0 (6 bits), UL_CAS_SBi (3 bits) |
| Unsync ranging allocation interval channel information (ranging region periodicity) | 3 |  |
| Unsync ranging location in the frame (time and frequency) | 2 |  |
| RNG codes information | 8 |  |
| The number of cyclic shifted codes per root index | 2 |  |
| The start root index of ZC code | 4 |  |
| Ranging code subset/ partition configuration | 3 4 |  |
| Unsync Ranging channel formats | 1 |  |
| ABS EIRP | 7 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for any new initial entry |
| Reserved | TBD |  |
| } |  |  |

[--------------------------------- End of Text Proposal -------------------------------- ]