| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Extended Header Types (15.2.2.2) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30a on P802.16m/D2 |  |
| Abstract | This contribution proposes extended header type value for each extended headers. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Extended Header Types (15.2.2.2)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

The length of ‘Type’ field for extended header is still TBD. In this contribution, we set the length of ‘type’ field to 4 bits and to assign type value for each EHs in order of section number.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 655 as follows]

Table 655 – Extended Header Format

| Syntax | Size | Notes |
| --- | --- | --- |
| Extended Header () { |  |  |
| LAST | 1 | Last Extended Header indication: 0 = one or more extended header follows the current extended header unless specified otherwise; 1 = this extended header is the last extended header unless specified otherwise |
| Type | TBD4 | Type of extended header |
| Body Contents | Variable | Type dependent content |
| } |  |  |

[Modify Table 656 as follows]

Table 656 – Description of Extended Header Types

| Extended Header Types | Names | Description |
| --- | --- | --- |
| - | Fragmentation and packing extended header | See 15.2.2.2.1 |
| - | Fragmentation extended header | See 15.2.2.2.2 |
| TBD0b0000 | Multiplexing extended header | See 15.2.2.2.3 |
| 0b0001 | Message ACK extended header | See 15.2.2.2.4 |
| TBD 0b0010 | Sleep Control extended header | See 15.2.2.2.6 |
| TBD 0b0011 | Correlation matrix feedback extended header | See 15.2.2.2.7 |
| TBD 0b0100 | MIMO feedback extended header | See 15.2.2.2.8 |
| 0b0101 | Piggybacked bandwidth request extended header | See 15.2.2.2.9 |
| TBD 0b0110-0b1111 | Reserved |  |

[Change the size of ‘Type’ field in Table 661, 662, 664, 665, 666 and 667 from ‘TBD’ to ‘4’]

---------------------------------------------------------End of the Text----------------------------------------------------------