| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Packing with Fragmentation (15.2.4.4.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30a on P802.16m/D2 |  |
| Abstract | This contribution contains an example of how to perform packing with fragmentation. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Packing with Fragmentation (15.2.4.4.1)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

Figure 392 shows how SDUs or SDU fragments are packed into a MAC PDU. However, there are two mistakes as follow;

- Number of FPEH: r SDU/SDU fragments are packed into the first MAC PDU in Figure 392. The number of packing information (End/length or End) included in the first MAC PDU shall be same as the number of SDU/SDU fragments included in the first MAC PDU. But they are different from each other.

- Sequence Number: Sequence number is increased by 1 whenever a new MAC PDU is generated, which means that SN is not related with the number of SDUs packed into a MAC PDU. However, the number of SDUs in the first PDU (r) is considered when determining SN value for the subsequent MAC PDUs.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Replace Figure 392 with the following figure]

Figure 392 – Packing with fragmentation

---------------------------------------------------------End of the Text----------------------------------------------------------