| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dedicated Carrier Supporting E-MBS in IEEE 802.16m |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D2,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30a: Announcement, IEEE 802.16-09/0057 |  |
| Abstract | E-MBS Support in detail of dedicated carrier on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Dedicated Carrier Supporting E-MBS in IEEE 802.16m

Eunkyung Kim, Jaesun Cha, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1 Instructions

The draft of IEEE 802.16m System Description Document (SDD) [1] describes broadcast only carrier where E-MBS data can be transmitted. E-MBS service through that carrier (i.e., E-MBS Dedicated carrier) can achieve the power conservation as well as performance enhancement. In order to support the E-MBS for the multi-carrier AMS in spite of limited capability of AMS, carrier switching should be considered and supported regardless of the AMS operating state – Connected State and Idle State. Carrier switching between different carriers will lead to the interruption of E-MBS content or unicast traffic. To avoid those interruptions in spite of limited capability of AMS, the different factor has to be considered according to the state of AMS.

When an AMS is in active mode, the switching to the dedicated carrier according to the E-MBS Scheduling Interval (MSI) for E-MBS service should be provided to avoid interruptions.

In addition, an AMS in sleep mode can avoid those interruptions for unicast traffic through primary carrier by following operation:

- Shifting within listening window

- Deferring till next listening window

- Extending the listening window

However, in IEEE 802.16m for the 802.16m Amendment Draft Standard [2], there is no functionality of E-MBS Dedicated Carrier. Therefore, in this contribution, we describe the dedicated carrier operation supporting E-MBS operation in IEEE 802.16m for the 802.16m Amendment Draft Standard.

# 2. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[In P802.16m/D2, page 146, line 36-42 modify as following]

- multicarrier Switching: The multicarrier mode in which the AMS switches its physical layer connection from the primary to the secondary carrier per ABS' instruction. The AMS connects with the secondary carrier for the specified time period and then returns to the primary carrier. When the AMS is connected to the secondary carrier, the AMS is not required to maintain its transmit or receive through the primary carrier. This mode is used for switching to partially configured carriers for downlink transmission only service such as E-MBS service on the dedicated carrier.

[In P802.16m/D2, page 159, add the following text end of 15.2.8.2.10.1]

In order to provide E-MBS service on the partially configured carrier without any interruption of any transmission through primary carrier, negotiation procedure of sleep mode operation may be performed in consideration of the E-MBS Scheduling Interval.

[Add the following text in the P802.16m/D2]

15.2.8.x E-MBS Support

The multi-carrier AMS which is incapable of processing multiple radio carriers at the same time may perform carrier switching mode. In order to support the E-MBS for the multi-carrier AMS, carrier switching should be considered and supported regardless of the AMS operating state – Connected State and Idle State. The carrier switching for E-MBS service on the dedicated carrier which is partially configured may be operated in consideration of the E-MBS Scheduling Interval.

[Add the following text in the P802.16m/D2]

15.9.x E-MBS Dedicated Carrier

The multi-carrier AMS which is capable of processing multiple radio carriers at the same time may perform normal data communication at one carrier while receiving E-MBS data over another carrier. It may also receive multiple E-MBS streams from multiple carriers simultaneously.

The multi-carrier AMS which is incapable of processing multiple radio carriers at the same time may perform carrier switching mode. The carrier switching for E-MBS service on the dedicated carrier which is partially configured may be operated in consideration of the E-MBS Scheduling Interval.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16m-09/0034r2, “IEEE 802.16m System Description Document,” September 2009

- IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” October 2009