| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Control Message Format for E-MBS in IEEE P802.16m |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D2,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30a: Announcement, IEEE 802.16-09/0057 |  |
| Abstract | MAC control messages to support E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MAC Control Message Format for E-MBS in IEEE 802.16m

Eunkyung Kim, Jaesun Cha, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1 Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

The AMS may continue to receive E-MBS transmissions from any ABS that is part of the E-MBS Zone, regardless of the AMS operating state—Connected State, Idle State.

When an AMS moves across the E-MBS zone boundaries and an ABS in another E-MBS zone is transmitting the same E-MBS flows, it can continue to receive E-MBS data from the ABS in Connected State or Idle State.

According to the IEEE 802.16m Amendment Draft Standard [1], when an AMS moves across the E-MBS boundaries, it can continue to receive E-MBS data from the ABS regardless of its operating state. In Connected State, the AMS performs handover procedure for E-MBS. In Idle State, the AMS may transit to Connected State to perform handover or it may initiate E-MBS location update process for the purpose of E-MBS zone transition unless the AMS already has the MSTID mappings in the target E-MBS zone.

However, the current HO procedure and location update procedure are not sufficient for seamless transition from one E-MBS zone to another without any interruption of E-MBS data service and operation.

Therefore, in this contribution, some MAC management messages (i.e., AAI_NBR-ADV, AAI_HO-CMD, AAI_RNG-REQ, and AAI_RNG-RSP) are modified to support the E-MBS HO and E-MBS location update.

# 2. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Add the following rows into Table 680-parameters for AAI_HO-CMD in 15.2.3.10 of 802.16m/D2]

| Name | Value | Usage |
| --- | --- | --- |
| N_New MBS Zones | Number of target ABS supports E-MBS Zones which is different from those supported by serving ABS | It may be included only if the Mode = 0b00 for each target ABS which supports E-MBS. |
| MBS Zone Identifier(s) | The new MBS Zone Identifier which the AMS shall maintain in any state on being served MBS. |  |
| Multicast STID | The MSTID which the AMS shall maintain in any state on being served E-MBS for each E-MBS Zone. |  |
| Service Flow Encoding(FID, service flow related parameter(s)) | mapping of the flow IDs used in E-MBS Zone supported by the serving ABS to flow IDs to be used in E-MBS Zone supported by the target BS. New Flow IDs are included in the increasing order of the corresponding old Flow IDs. |  |

[Add the following row into Table 674 – parameters for AAI_RNG-REQ in 15.2.3.1 of 802.16m/D2]

| Name | Value | Usage |
| --- | --- | --- |
| E-MBS Zone Change Indicator | Indicates the AMS is currently attempting to perform location update due to E-MBS Zone change. | It shall be included only if the target ABS which supports E-MBS. |
| E-MBS related information | Current MSTID, FID, MBS Zone ID | If the E-MBS Zone Change Indicator is set, it shall be include in the AAI_RNG-REQ. |

[Adopt the following rows into Table 675-parameters for AAI_RNG-RSP in 15.2.3.2 of 802.16m/D2]

| Name | Value | Usage |
| --- | --- | --- |
| Multicast STID | The MSTID which the AMS shall maintain in any state on being served MBS. | It may be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and network re-entry for HO procedure if it needs to update. |
| E-MBS Zone Identifier | The new E-MBS Zone Identifier which the AMS shall maintain in any state on being served MBS. |  |
| Service Flow Encoding(FID, service flow related parameter(s)) | An identifier of a service flow. It pairs with MSTID in the E-MBS MAP message and it is allocated to each E-MBS connections. As a result, an AMS can receive multiple E-MBS traffics for an E-MBS connection with different E-MBS contents distinguished by the FID belonging to an MSTID. |  |

[Modify 15.2.3.11 on page 48 of 802.16m/D2 as follows]

15.2.3.11 AAI_NBR-ADV

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

1) ABS type (macro, micro, femto, relay, TBD)

2) carrier frequency

3) MAC version

4) TDD/FDD and related definitions (expected to be the same given carrier frequency)

5) BW, CP info: may not be required if carried in P/S-SCH channel

6) Multi-carrier capability and configuration

7) E-MBS zone related information (E-MBS Zone ID)

ABS determines and indicates the system configuration information included for each deployment type and their corresponding broadcast frequency.

[Add the following subsection in the end of 15.2.3]

15.2.3.x AAI_E-MBS-CFG

AAI_E-MBS-CFG message includes the E-MBS related configuration information necessary for E-MBS operation.

The following parameters are included according to the usage of the AAI_E-MBS-CFG message:

Table xxx – parameters for AAI_E-MBS-CFG

| Name |  | Value | Usage |
| --- | --- | --- | --- |
| E-MBS Zone IDs of serving ABS |  | The E-MBS Zone Identifier which the AMS shall maintain in any state on being served MBS. |  |
| E-MBS Zone IDs of neighbor ABSs |  | The E-MBS Zone Identifier which the AMS shall maintain in any state on being served same MBS content by neighbor ABSs. |  |
| E-MBS time offset | Superframe number | LSB of the superframe number of the E-MBS MAP | Shall be (m+3) LSB part of superframe number, where length of MSI is 2m+3 |
|  | Frame number | Frame number in the Superframe number of the E-MBS MAP 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super frame |  |
|  | Subframe number | Subframe number in the Frame number of the E-MBS MAP 0b000: 1st subframe of the frame …. 0b111: 8th subframe of the frame |  |
| E-MBS MAP resource index |  | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |  |
| E-MBS MAP Isizeoffset |  | Offset used to compute E-MBS MAP size index |  |
| E-MBS MAP MIMO Mode |  | E-MBS MAP MIMO Encoding Format 0b0: MIMO mode 0 0b1: MIMO mode 1 | When MIMO mode-1 is used for E-MBS, the number of streams shall be no more than 2. |
| MSTID and FID mappings between serving E-MBS Zone and neighbor E-MBS Zone |  | The mappings of MSTID and FID between serving E-MBS Zone and neighboring E-MBS Zone for the same content |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” October 2009