| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Discussion on DCR mode (P802.16m/D2-15.2.18) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Lei Zhang Yang Liu Mary Chion Feng Xie ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB30a / Area: Chapter 15.2.18 (DCR mode) |  |
| Abstract | This contribution proposes changes to the Section 15.2.18 of IEEE P802.16m/D2 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Discussion on DCR mode (P802.16m/D2-15.2.18)

Lei Zhang, Yang Liu, Mary Chion, Feng Xie

ZTE Corporation

- Introduction

In P802.16m/D2, DCR (Deregistration with content retention) mode is proposed to expedite network re-entry when AMS goes into hibernation mode or handover to other RAT. However, the description of DCR mode is quite unclear. For instance, as stated in D2, DID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs, what is the exact network entity be used to identify the AMS? On the other hand, the length of DID is limited to 10 bit in D2, is this enough to identify thousands of AMSs in the network? In addition, the real requirement or benefit of DCR mode still needs further discussion.

This contribution proposes to remove DCR mode from D2 or, as a choice, more reasonable changes may be performed to DCR mode based on further discussion and clarification.

- Proposed text

======================== Start of Proposed Text ========================

[Modify the paragraph from lines 21 on page 241 of P802.16m/D2 as follows.]

15.2.18 Deregistration with content retention (DCR) mode

Deregistration with content retention (DCR) mode is a mode in which an AMS is deregistered from the network while its context is kept in a network entity until the idle mode retention timer is valid.

While the idle mode retention timer is valid, the network retains AMS’s information which is used to expedite AMS’s network re-entry. Deregistration identifier (DID) is assigned to AMS with paging cycle and paging offset value of 0.

DID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs.

15.2.18.1 DCR initiation in connected state

AMS may initiate DCR mode by transmitting an AAI_DREG-REQ message with the De-registration_Request_Code parameter= 0x04; request for AMS deregistration from serving ABS and retention of AMS’s connection information. The AMS may request the network to retain specific AMS service and operational information for DCR mode management purposes through inclusion of the Idle Mode Retain Information element in the AAI_DREG-REQ.  When the ABS decides to allow AMS’s DCR mode request, the ABS shall send an AAI_DREG-CMD with action code 0x08 in response to the AAI-DREG-REQ message. When ABS decides to reject AMS’s DCR mode request, the ABS shall send an AAI_DREG-CMD with action code 0x09 in response to the AAI_DREG-REQ message.  In the AAI-DREG-CMD, DID shall be assigned which is used for network re-entry from DCR mode to identify the AMS.

15.2.18.2 DCR mode initiation from idle mode

AMS may initiate DCR mode in idle mode state by performing the location update with ranging purpose code of 0x05 in AAI_RNG-REQ message for transition to DCR mode.

When the ABS decides to allow AMS’s DCR mode request, the ABS shall send a AAI_RNG-RSP with action code 0x04. When ABS decides to reject AMS’s DCR idle mode request, the ABS shall send an AAI_RNG-RSP with action code 0x05.

Upon successful DCR mode change request, the network shall initiate DCR mode operation by retaining AMS’s information until the idle mode retention timer is valid. At the time of DCR mode change, paging cycle, paging offset, paging controller values shall be updated to be 0 for the AMS and the ID of the network entity that assigns/retains the context  shall be used to uniquely identify DCR mode AMS.

15.2.18.3 DCR mode extension

An AMS in DCR mode can extend its idle mode retention timer by sending a RNG-REQ message to extend the timer before it expires.

15.2.18.4 Network re-entry from DCR mode

For the network re-entry from DCR mode, the AMS shall initiate network re-entry with the ABS by sending an AAI_RNG-REQ message with ranging purpose code of 0x06 and the ID of the network entity that assigns/retains the context. Rest of re-entry procedure shall be performed same as network reentry from idle mode described in section 15.3.12.5.

15.2.18.5 DCR mode termination

DCR mode may only be terminated through:

AMS reentry to the network

Expiration of the idle mode retention timer

[Modify the paragraph from lines 39 on page 64 of P802.16m/D2 as follows.]

15.2.3.20 AAI_DREG-REQ message

The following parameter shall be included in AAI_DREG-REQ message :

Control Message Type

De-registration_Request_Code

0x00: AMS deregistration request from ABS and network

0x01:  request for AMS deregistration from serving ABS and initiation of AMS idle mode

0x02: response for the unsolicited AAI_DREG-CMD message with action code 0x05 by the ABS

0x03: reject for the unsolicited AAI_DREG-CMD message with action code 0x05 by the ABS.

0x04: request for AMS deregistration from serving ABS to enter DCR mode

0x054 – 0x015: reserved

When De-registration_Request_Code is 0x04, the following parameters shall be included in the AAI_DREG-REQ message.

Idle Mode Retain Information element

[Modify the paragraph from lines 31 on page 65 of P802.16m/D2 as follows.]

15.2.3.21 AAI_DREG-RSP message

The following parameter shall be included in AAI_DREG-RSP message:

Control message type

Action code

0x07: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Request Code 0x04: to allow retention of AMS’s connection information

0x09: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Request Code 0x04: to reject retention of AMS’s connection information

When the action code is set to 0x07, the following parameters shall be included in AAI_DREG-CMD message.

Idle Mode Retain Information

Deregistration Identifier (DID)

[Update Table 674 from lines 45 on page 36 of P802.16m/D2 as follows.]

Table 674 – parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Purpose Indication | If bit #5 is set to 1, it indicates that the AMS is initiating location update for transition to DCR mode from idle mode If bit #6 is set to 1, in combination with ID of the network entity that assigns/retains the context, it indicates that the AMS is currently attempting re-entry from DCR mode Bit#3-7 : reserved | It shall be included when the AMS is attempting to perform re-entry, HO, or location update or location update for entering DCR mode. |

[Update Table 675 from lines 63 on page 38 of P802.16m/D2 as follows.]

Table 683 – parameters for AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Location Update Response | 0x00=Success of Location Update 0x01=Failure of Location Update 0x02=Reserved 0x03=Success of location update and DL traffic pending 0x04 = Allow AMS’s DCR mode request 0x05 = Reject AMS’s DCR mode request 0x064~0xFF : Reserved | It shall be included when the ABS sends an AAI_RNG-RSP message in response to an AAI_RNG-REQ message used to perform location update., or location update for entering DCR mode |

======================== End of Proposed Text ========================