| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Discussion on DCR mode (P802.16m/D2-15.2.18) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Lei Zhang Yang Liu Mary Chion Feng Xie ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB30a / Area: Chapter 15.2.18 (DCR mode) |  |
| Abstract | This contribution proposes changes to the Section 15.2.18 of IEEE P802.16m/D2 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Discussion on DCR mode (P802.16m/D2-15.2.18)

Lei Zhang, Yang Liu, Mary Chion, Feng Xie

ZTE Corporation

Shantidev Mohanty

Intel Corporation

- Introduction

In P802.16m/D2, DCR (Deregistration with content retention) mode is proposed to expedite network re-entry when AMS goes into hibernation mode or handover to other RAT. However, the description of DCR mode is quite unclear. For instance, as stated in D2, DID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs, what is the exact network entity be used to identify the AMS? On the other hand, the length of DID is limited to 10 bit in D2, is this enough to identify thousands of AMSs in the network? In addition, the real requirement or benefit of DCR mode still needs further discussion.

This contribution proposes to remove DCR mode from D2 or, as a choice, more reasonable changes may be performed to DCR mode based on further discussion and clarification.

- Proposed text

======================== Start of Proposed Text ========================

[Modify the paragraph from lines 21 on page 241 of P802.16m/D2 as follows.]

15.2.18 Deregistration with content retention (DCR) mode

…

While the idle mode retention timer is valid, the network retains AMS’s information which is used to expedite AMS’s network re-entry. Retention identifier (RID) Deregistration identifier (DID) is assigned to AMS during DCR mode initiation from connected state or idle mode. with paging cycle and paging offset value of 0. RID DID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs.

[Modify the paragraph from lines 47 on page 241 of P802.16m/D2 as follows.]

15.2.18.1 DCR initiation in connected state

…

In the AAI-DREG-CMD, DID RID shall be assigned which is used for network re-entry from DCR mode to identify the AMS.

…

[Update Table-674 on page 36 of P802.16m/D2 as follows.]

Table 674—Parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform HO reentry |
| Retention Identifier (RID) | The ID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs. | It shall be included when the AMS is attempting to perform DCR mode extension or network reentry from DCR mode. |

[Modify the paragraph from lines 65 on page 65 of P802.16m/D2 as follows.]

When the action code is set to 0x07, the following parameters shall be included in AAI_DREG-RSP message.

Idle Mode Retain Information

Retention identifier (RID) Deregistration identifier (DID)

======================== End of Proposed Text ========================