| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC support for Multi-BS operation for the IEEE 802.16m (section 15.2.3.30) |  |
| Date Submitted | 2008-11-18 |  |
| Source(s) | Inuk Jung, Wookbong Lee, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1856 [cooper, wbong, ksryu, ronnykim, samji] @lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #30a |  |
| Abstract | Text proposal for Multi-BS MIMO section |  |
| Purpose | For member’s review and adoption into P802.16m_D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC support for Multi-BS operation for the IEEE 802.16m (section 15.2.3.30)

Inuk Jung, Wookbong Lee, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to provide MAC support for Multi-BS Co-MIMO in the 802.16m D2 document [1].

- Rationale/Motivation of modifications

To perform Downlink Multi-BS MIMO, an AMS is required to know certain parameters, such as temporary BSID, ABS’s that take part in downlink multi-BS operation. In the current D2 document, the provision of such information is not specified. The information of Multi-BS Joint MIMO processing or Single BS precoding with multi-BS coordination is broadcasted within the AAI_DL_IM message (defined in section 15.2.3.30).

Thus we propose to provide such information using AAI_DL_IM MAC message..

Also, to keep the system information of the ABSs in the diversity set up-to-date, the ABS should indicate the change count of the AAI_NBR-ADV in respect to the the Multi-BS participating ABSs (Temp_BSID).

Within the AAI_DL_IM message, the BS_index relates to the BS index of the AAI_NBR-ADV broadcast message. The Temp_BSID is autonomously derived from the order of the included BSs. Hence, the Temp_BSID of the first BS in the AAI_DL_IM message will be 0 and so forth.

- References

[1] IEEE P802.16m/D2, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

On the 80216m_D2, page 78, add current text in 15.2.3.30 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

15.2.3.30 AAI_DL_IM Message

An AAI_DL_IM broadcast DL FFR partition resource metric, BC_SI, NIP_threshold, power level for each frequency partition.

-- ASN1START

AAI_DL_IM Message ::= SEQUENCE {

FFR partition resource metric :: = SEQUENCE {

Resource_Metric_FP2		INTEGER 	(TBD) 	OPTIONAL

Resource_Metric_FP3		INTEGER 	(TBD) 	OPTIONAL

}

BC_SI		INTEGER 	(0..15) 	OPTIONAL

NIP_th_1	INTEGER 	(TBD) 	OPTIONAL

NIP_th_2	INTEGER 	(TBD) 	OPTIONAL

FP1_power	INTEGER 	(TBD) 	OPTIONAL

FP2_power	INTEGER 	(TBD) 	OPTIONAL

FP3_power	INTEGER 	(TBD) 	OPTIONAL

Diversity_Set_NUM		INTEGER	(0..15)		OPTIONAL

Change_count 		INTEGER	(0..255)	OPTIONAL

Temp_BSID :: = SEQUENCE OF {

BS_index		INTEGER	(0..255)	OPTIONAL

}

}

-- ASN1STOP

Table X—Parameters for AAI_DL_IM Message

| Name | Value | Usage |
| --- | --- | --- |
| Resource_Metric_FP2 | TBD |  |
| Resource_Metric_FP3 | TBD |  |
| BC_SI | 4 |  |
| NIP_th_1 | TBD |  |
| NIP_th_2 | TBD |  |
| FP1_power | TBD |  |
| FP2_power | TBD |  |
| FP3_power | TBD |  |
| Diversity_Set_NUM | 4 | The number of ABSs taking part in downlink single BS precoding with Multi-BS Coordination |
| Change_count | 8 | Change count in AAI_NBR-ADV for the following BS_index |
| BS_index | 8 | Indicates diversity set members Refers to the ABS index of the AAI_NBR_ADV. The Temp_BSID refers to the ‘i’ value. |

--------------------------------------------------------Text End -----------------------------------------------------