| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Security Support for Zone switch procedure in IEEE802.16m systems (sections 15.2.3.1, 15.2.3.2) |  |
| Date Submitted | 2008-11-06 |  |
| Source(s) | Inuk Jung, Gene Beck Hahn, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1856 [cooper, gbhahn, ksryu, ronnykim, samji] @lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #30a / Topic: HO |  |
| Abstract | Text proposal for Handover |  |
| Purpose | For member’s review and adoption into P802.16m_D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Security Support for Zone switch procedure in IEEE802.16m systems (sections 15.2.3.1, 15.2.3.2)

Inuk Jung, Gene Beck Hahn, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to provide Security support Zone Switching procedure in the 802.16m D2 document [1].

- Rationale/Motivation of modifications

1. MAC message and data protection during Zone Switch

To perform provide security service, such as encryption/decryption of data or MAC management messages, during zone switch, the AMS and ABS has to know each others security parameters. During a Zone Switch, the AMS and ABS cannot follow the conventional 16m Security Key Exchange procedure. Hence, ABS sends its NONCE_BS via the ZS_TLV. However, NONCE_MS must also be known by ABS, which AMS shall include in the AAI_RNG-REQ message after entering MZone.

After ABS receives the NONCE_MS, it may successfully generate PMK, AK and CMAC, which then it also may authenticate the AAI_RNG-REQ message. From this point on, the AMS and ABS may encrypt any MAC or data message.

2. Correct context mapping

The context mapping of service flow ID’s are defined to be done automatically in the ascending order from the first transport Connection CID. The definition SAID of 16 and 16m has changed slightly, which is an attribute of a service flow. Hence, this shall also be reflected while doing the context mapping.

Since basic service flow ID are mapped automatically, the re-mapping SAID per service flow ID shall be explicitly informed to the AMS by the ABS in the MZone. This may be done through any MAC message, but it may seem natural to include it in the AAI_RNG-RSP message.

- References

[1] IEEE P802.16m/D2, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy #1

On the 80216m_D1, page 35, table 674, modify text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

15.2.3.1 AAI_RNG-REQ

An AAI_ RNG-REQ message is transmitted by AMS at network entry, to which HARQ operation is applied. An AMS shall generate AAI_ RNG-REQ message containing parameters according to the usage of the AAI_RNG-REQ message:

…

Table 674—Parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |
| NONCE_MS | A freshly generated random number of 64 bits | ABS generates PMK, AK and CMAC keys based on this parameter |
| … | … | … |

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #2

On the 80216m_D1, page 38, table 675, modify text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

15.2.3.2 AAI_RNG-RSP

An AAI_RNG-RSP, to which HARQ operation is applied, shall be transmitted by the ABS in response to a received AAI_RNG-REQ. In addition, it may also be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. As a result, the AMS shall be prepared to receive an AAI_RNG-RSP at any time, not just following an AAI_ RNG-REQ transmission.

…

In case of Zone Switch procedure from LZone to MZone, the AAI_RNG-RSP message shall also include the SAID information which should be correctly mapped to the corresponding service flow ID.

Table 675—parameters for AAI_ RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| Neighbor station measurement report indicator | Perform Neighbor station measurement report if set to '1' | Identifies Neighbor station measurement report is required during current network entry |
| N_SF_update | 8 bit | Number of service flow that requires update |
| Service_flow_bitmap | 16 bit | Bitmap for indicating the specific service flows that are being updated. The MS will map them autonomously by referring to this bitmap |
| for(i=0; i< N_SF_update; i++) { |  |  |
| SAID |  | The updated SAID value |
| } |  |  |
| … | … | … |

--------------------------------------------------------Text End -----------------------------------------------------