| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Paging Carrier Information in Idle Mode |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Yuqin Chen, Nan Li, Hongyun Qu ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: chen.yuqin@zte.com.cn li.nan25@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Letter Ballot #30a |  |
| Abstract | This contribution proposes suggested amendment text for paging carrier information in idle mode (Chapter 15.2.17). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for Paging Carrier Information in Idle Mode

Yuqin Chen，Nan Li, Hongyun Qu

ZTE Corporation

# Introduction

According to AWD, each ABS may support multiple carriers. Fig 1 shows an example of an ABS with multiple carriers and multiple PGIDs. For paging operation in idle mode, how to transmit paging messages on multiple carriers is a problem to solve. A simple way is transmitting the whole paging messages on each carrier, an AMS can monitor any carrier to receive. The overhead of this method is large due to the duplicated transmission. In this proposal, three ways to solve the problem are discussed.

Fig 1. An example of an ABS with multiple carriers

# Proposed Solutions

## Solution 1: Paging messages transmitted on all fully configured carriers

In solution 1, it is proposed to transmit all paging messages on all fully configured carriers.

In this case, after entering idle mode, AMS works as in single carrier deployment. As shown in Fig 1, ABS transmits all paging messages for PGID 1, PGID 2 and PGID3 on all fully configured carriers. Obviously, the overhead for transmitting identical paging message on all carriers is heavy. To reduce the overhead, solution 2 and 3 are proposed as following.

## Solution 2: Partial paging messages transmitted on each fully configured carrier

It is proposed that partial paging messages are transmitted on each fully configured carrier in solution 2.

For the case shown in Fig 1, ABS could transmit paging message for PGID1 on Carrier 1, and transmit paging messages for PGID 2 and PGID 3 on Carrier 2. The paging carrier information for each PGID may be broadcasted in SFH IE along with PGID information to AMSs.

When an AMS enters idle mode, the paging carrier for the PGID the AMS belongs to can be indicated by AAI-DREG-CMD message. Then AMS can receive paging message by monitoring the corresponding paging carrier.

If the AMS performs location update, the paging carrier information on the current ABS is indicated by Location Update TLV in AAI_RNG-RSP message. In case the paging carrier information is referred by physical carrier index, the AMS needs to receive multi-carrier configuration information first.

When an AMS moves to the coverage of the ABS in Figure 1, there may be three possible cases:

Case 1: AMS {PGID2, working on Carrier 1}: AMS gets the paging carrier information from SFH IE, and then switch to Carrier 2 to keep monitoring;

Before Carrier Switching, AMS receive the multi-carrier configuration information at the ABS .

Case 2: AMS {PGID2, working on Carrier 2}: AMS gets information that PGID 2 is supported on Carrier 2 from SFH IE, and keep monitoring on the paging message during listening interval.

Case 3: AMS {PGID 4}: AMS finds out the PGID 4 is not support on current ABS, then performs location update or network re-entry.

The corresponding S-SFH IE field for paging is listed in Table 1.

Table 1 S-SFH SPx IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SPx IE format () { |  |  |
| N_PGIDs |  | Number of paging groups in the ABS |
| For (i=0; i<N_PGIDs; i++){ PGID; Paging carrier information (physical carrier index) } |  | Identifier of paging group and carriers belong to the corresponding Identifier. |
| } |  |  |

The paging information TLV for AAI_DREG-CMD, and location update TLV for AAI_RNG-RSP are proposed as listed in Table 2 and Table 3.

Table 2 PG Info TLV

| Name | Type | Length(Bytes) | Value |
| --- | --- | --- | --- |
| PG Info | TBD | TBD | Bits #xx-yy: PGID assigned for AMS Bits #yy+1-zz: paging carrier for PGID above (physical carrier index) |

Table 3 Location Update TLV

| Name | Type | Length(Bytes) | Value |
| --- | --- | --- | --- |
| Location Update Response | TBD | TBD | Bits #xx-yy: PGID assigned for AMS Bits #yy+1-zz: paging carrier for PGID above (physical carrier index) |

## Solution 3: Paging Messages transmitted only on one fully configured carrier

In this solution, it is proposed to transmit all paging message only on one fully configured carrier.

In the example shown in Fig 1, ABS transmits all paging messages only on Carrier1. However, the PGID information shall be transmitted on all fully configured carriers. To reduce the carrier switching delay, the paging carrier information is included in the PGID information.

When an AMS enters idle mode, the paging carrier for the PGID the AMS belongs to can be indicated by AAI-DREG-CMD message. Then AMS only monitors the paging carrier to listen to the paging message.

If the AMS performs location update, the paging carrier information on the current ABS is indicated by Location Update TLV in AAI_RNG-RSP message. In case the paging carrier information is referred by physical carrier index, the AMS needs to receive multi-carrier configuration information first.

## Summary

Among the three solutions mentioned above, the 1st solution bears heavy overhead due to the identical paging messages transmitted on all carriers. In the 3rd solution, the paging carrier selected also suffers loading problem.

The 2nd solution distributes the overhead of paging messages to multiple carriers, which is better for the ABS to control load balancing. And from AMS’s point of view, paging procedure is not changed compared to single carrier mode. Therefore, we propose the 2nd solution to TGm.

# Text proposal in the 802.16m amendment

---------------------------------------------------------- Text Start ----------------------------------------------------------------

# 15.2.3.2 AAI_RNG-RSP

…

Table 675- Parameters for AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| … |  |  |
| Paging Cycle | New Paging Cycle of the AMS 0b0000-0b1111 | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed. |
| Paging Group ID | New PGID of the AMS |  |
| Paging Carrier | The paging carrier (physical carrier index) carrying the paging message for the new PGID. |  |
| Paging Offset | New PAGING_OFFSET of AMS |  |

# 15.2.3.21 AAI_DREG-RSP message

... (P65, L38)

When the action code is set to 0x05, the following parameters shall be included in AAI_DREG-RSP message.

Paging information

Paging cycle

Paging offset

Paging group ID

Paging Carrier (physical carrier index): the carrier carrying the paging message for PGID

Paging controller ID

…

# 15.2.17 Idle mode

15.2.17.2 Operation during Idle mode

15.2.17.2.1 Broadcast paging message

A Paging message is an AMS notification message which either indicates the presence of DL traffic pending for the specified AMS or it is intended to poll an AMS and request a location update without requiring a full network entry or to request an AMS to perform LBS measurement. In addition, an emergency alert indicator is included in the paging message to notify the idle AMSs reception of the emergency information. Upon reception of the emergency alert indicator that is set to “1”, the AMS shall decode the A-A-MAP to obtain the emergency information.

In multi-carrier deployment, ABS may transmit partial paging messages for a specified group PGIDs on each fully configured carrier. In this case, ABS indicates the paging carrier for AMS to monitor by AAI_DREG-RSP message. If the paging carrier is not current serving carrier at AMS, AMS switches to the paging carrier indicated by AAI_DREG-RSP message when enters idle mode.

…

15.2.17.2.3 Operation during paging listening interval

Page 237, line 53:

The ABS shall transmit the PGID Info and the paging carrier information at a predetermined location in the paging listening interval in order to advertise the paging group(s) that is supported by the ABS. The PGID info and the paging carrier information shall be transmitted by the ABS regardless of whether or not there any notifications for AMSs.

Page 238, line 41:

Table 751—PGID Info Format

| Name | Value |
| --- | --- |
| Num_PGIDs | Number of paging groups in the ABS |
| PGID | Identifier of paging group (0~216) |
| Paging carrier information | physical carrier index |
| m | Time domain hash parameter (1~4) used to determine the frame number of a superframe for paging message transmission of an idle mode AMS. |

# 15.2.17.4.2 Location update process

Page 240, line 45:

If the ABS evaluates the CMAC Tuple as valid and supplies a corresponding authenticating CMAC Tuple,

then the ABS shall reply with an encrypted AAI_RNG-RSP message including the Location Update Response

TLV completing the location update process. If paging group has changed, then the ABS shall include Pag-

ing Group ID in the AAI_RNG-RSP message. The paging carrier information is included in the location update TLV.

------------------------------- ----------------------------Text End ------------------------------------------------------------

# References

[1] P802.16m/D2 October 2009