| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of mapping between LRU index and physical index (15.3.6.5.2.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Taeyoung Kim, Hyunkyu Yu, Jaeweon Cho, Hokyu Choi Samsung Electronics | E-mail: ty33.kim@samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a” Target topic: “IEEE P802.16m/D2 section 15.3.6.5.2.2”. |  |
| Abstract | The contribution proposes modifications for tone dropping support in the current draft standard 802.16m/D2. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of mapping between LRU index and physical index (15.3.6.5.2.2)

Taeyoung Kim, Hyunkyu Yu, Jaeweon Cho, Hokyu Choi

Samsung Electronics

# Motivation

In 802.16m/D2, the equation for mapping between LRU index and physical PRU index is not correct and the order of distributed resource and contiguous resource is not reasonable. Therefore, in this contribution, we propose to revise the current equations and change the order that the LRU index for distributed resource should be started from index 0.

# Proposed Text

Adopt the following text changes for [1]:

Original Amendment text is shown in black. New proposed text is in blue. Removed Amendment text is shown in strike out red text.

------------------------------------------- Text Start ---------------------------------------------------------

[Change the text and equations from line 54 to 65 on the page 361, in 15.3.6.5.2.2, as follows:]

Si=m=0m=iFPSm, 0≤i<3 Si=m=0iFPSm, 0≤i<FPCT

LRU[k]=CRUFPik−Si−1for Si−1≤k≤LCRU,FPi+Si−1DRUFPik−LCRU,FPi+Si−1for LCRU,FPi+Si−1≤K≤Si,for 0≤i≤3

LRU[k]=DRUFPik−Si−1Si−1≤k<LDRU,FPi+Si−1CRUFPik−LDRU,FPi+Si−1LDRU,FPi+Si−1≤k<Si,for 0≤i<FPCT

with 0≤k<m=0m=3FPSm0≤k<m=0FPCT−1FPSm and S-1=0.

------------------------------------------- Text End -----------------------------------------------------------

# Reference

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface”.