| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of P-SFH IE and S-SFH IE for IEEE 802.16m D2 (Section 15.3.6.5.1.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Mihyun Lee, Jaeweon Cho, Hokyu Choi Samsung Electronics | mihyun.mac.lee@samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a |  |
| Abstract | The contribution proposes to modify P-SFH IE and S-SFH IE in order to clarify the description of parameters carried in P-SFH IE and S-SFH IE. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of P-SFH IE and S-SFH IE for IEEE 802.16m D2 (Section 15.3.6.5.1.1)

Mihyun Lee, Jaeweon Cho, Hokyu Choi

Samsung Electronics

- Introduction

The contribution proposes to modify P-SFH IE and S-SFH IE in order to clarify the parameters carried in P-SFH IE and S-SFH IE.

1) Each S-SFH subpacket IE is of a fixed size and the number of repetition used for S-SFH is indicated in P-SFH IE. So, the indication of S-SFH Size is redundant.

2) It is not realistic that multiple SPx IE are scheduled in same superframe since TBCC specified in D2[1] supports the maximum information as 128 bits. The direct indication is better than the bitmap indication.

- Proposed text with markups

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

- References

[1] IEEE P802.16m/D2, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2009-10-14

- Text proposals for P802.16m D3

Remedy #1: Modify the text in Table 789, line 42, page 340, section 15.3.6.5.1.1 as follows,

-----------------------------------------  Text Start  ---------------------------------------------------

Table 789– P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSBs of superframe number | 4 | Part of superframe number |
| S-SFH change count | 4 | Describe the S-SFH SPx IE that apply to this superframe. Indicates the value of S-SFH change count associated with the S-SFH SPx IE(s) transmitted in this superframe |
| S-SFH Size | 4 | The units of LRU |
| S-SFH Number of Repetitions | 2 1 | Indicate transmission format (repetition) the number of repetitions used for S-SFH. 0b0: 6 0b1: 3 |
| S-SFH Scheduling information bitmap | 3 | Indicates the presence of S-SFH SPx IE(s) in this superframe. 0b000: no S-SFH SPx IE 0b001: S-SFH SP1 IE 0b010: S-SFH SP2 IE 0b011: S-SFH SP1 IE 0b100 ~ 0b111: reserved 0b000: no S-SFH If 1st bit = 1, S-SFH includes SP1 otherwise no SP1 If 2nd bit = 1, S-SFH includes SP2 otherwise no SP2 If 3rd bit = 1, S-SFH includes SP3 otherwise no SP3 |
| S-SFH SP change bitmap | 3 | Indicate the change of S-SFH SPx IE. The bit #0 to bit#2 is mapped to S-SFH SP1 IE to S-SFH SP3 IE, respectively. |
| Configuration Transition Flag | 1 | Indicates the associated S-SFH change count in this superfame 0b0: Use S-SFH SPx IEs associated with the current S-SFH change count 0b1: Use S-SFH SPx IEs associated with (the current S-SFH change count – 1) modulo 16 |
| Reserved | 5 | The reserved bits are for future extension. |
| } |  |  |

Remedy #2: Modify the text in Table 790, line 46, page 341, section 15.3.6.5.1.2 as follows,

Table 790 – S-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH IE format () { |  |  |
| If (1st bit of S-SFH Scheduling information bitmap == 1) { If (S-SFH Scheduling information ==0b001) { |  |  |
| S-SFH SP1 IE () |  | Includes S-SFH SP1 IE in Table 791 |
| } |  |  |
| if (2nd bit of S-SFH Scheduling information bitmap == 1) { else if (S-SFH Scheduling information ==0b010) { |  |  |
| S-SFH SP2 IE () |  | Includes S-SFH SP2 IE in Table 792 |
| } |  |  |
| if (3rd bit of S-SFH Scheduling information bitmap == 1) { else if (S-SFH Scheduling information ==0b011) { |  |  |
| S-SFH SP3 IE () |  | Includes S-SFH SP3 IE in Table 793 |
| } |  |  |
| } |  |  |

---------------------------------------------  Text End  ---------------------------------------------------