| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed texts for HARQ signaling (Section 15.2.14.2.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Mihyun Lee, Jaeweon Cho, Hokyu Choi, Jaehee Cho Samsung Electronics | mihyun.mac.lee@samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a |  |
| Abstract | The contribution proposes to specify the HARQ signaling parameters. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed texts for HARQ signaling and timing (Section 15.2.14.2.1)

Mihyun Lee, Jaeweon Cho, Hokyu Choi, Jaehee Cho

Samsung Electronics

- Introduction

The 802.16m system provides asynchronous HARQ for DL and synchronous HARQ for UL. To support these HARQ operations, the HARQ signaling such as the maximum number of retransmissions of the same DL/UL data bursts specified in section 15.2.14.2.1[1] are required. The last HARQ DG discussion, most members preferred these parameters shall be configurable according to cell types.

In this contribution, we propose that the ABS broadcasts these HARQ signaling parameters because MS shall know these parameters before making a unicast connection.

- References

[1] IEEE P802.16m/D2, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2009-10-14

- Proposed Text Changes for the 802.16m/D1

Remedy #1: Modify the text in page 198, line 34, section 15.2.14.2.1.1 as follows,

The number of retransmissions of the same data burst shall not exceed the maximum [DL N_MAX_ReTx]. DL N_MAX_ReTx is carried in AAI_SCD message.

Remedy #2: Add the following parameters into AAI_SCD message specified in page 77, line 33, section 15.2.3.28 as follows,

AAI_SCDMessage :: SEQUENCE {

DL N Max_ReTx		INTEGER (4, 8)

}

Remedy #3: Modify the text in page 198, line 64, section 15.2.14.2.1.2 as follows,

In UL, the maximum number of total HARQ channels per AMS is 16. The number of retransmissions of the same data burst shall not exceed the maximum [ UL N_MAX_ReTx]. UL N_MAX_ReTx is carried in S-SFH SP1 IE.

Remedy #4: Add the following parameter into Table 791 in page 342, section 15.3.6.5.1.2

Table 791 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| UL N_Max_ReTx | 1 | Specifies the maximum number of retransmissions of the same data burst for UL. 0b0: 4 0b1: 8 |
| } |  |  |