| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Operation in Dedicated Carrier (15.9.2.4) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Qi Wu, Kaushik Josiam, Jaejeong Shim Samsung Electronics | Email : qi.wu@samsung.com k.josiam@sta.samsung.com brian.shim@samsung.com |
| Re: |  |  |
| Abstract |  |  |
| Purpose | For review and discussion by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS Operation in Dedicated Carrier

Qi Wu, Kaushik Josiam, Jaejeong Shim

Samsung Electronics

1. Introduction

The idle mode AMS loses its synchronization with its unicast-only carriers while still keeps synchronization with its E-MBS carriers. For the E-MBS dedicated carrier, if the unicast paging is also transmitted in the carrier as one of the E-MBS services, the E-MBS AMSs in the carrier do not need to synchronize with its unicast carrier each time before receiving paging. Another benefit is that the multicarrier switching AMSs will not miss the E-MBS content during synchronizing and receiving paging.

2. Proposed text

The following text is proposed to add to IEEE P802.16m/D2.

---------------------------------------------------- Proposed Text #1  ------------------------------------------------

15.9.2.4 E-MBS Operation in Dedicated Carrier

When E-MBS is deployed in dedicated carrier, the paging information for the AMSs subscribing E-MBS contents in this carrier will also be transmitted in the carrier using a well-known E-MBS STID and FID.

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------