| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS MAP and E-MBS Data Information Elements (15.3.6.2.3, 15.3.6.5.3) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Qi Wu, Kaushik Josiam, Jaejeong Shim, TaeYoung Kim Samsung Electronics | Email : qi.wu@samsung.com k.josiam@sta.samsung.com brian.shim@samsung.com ty33.kim@samsung.com |
| Re: | Contribution in support of a comment for LB #30a |  |
| Abstract | Proposes amendment text for E-MBS MAP and E-MBS Data Information Elements |  |
| Purpose | For review and discussion by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for E-MBS MAP and E-MBS Data Information Elements

Qi Wu, Kaushik Josiam, TaeYoung Kim, Jaejeong Shim

Samsung Electronics

The following text is proposed to modify IEEE P802.16m/D2.

---------------------------------------------------- Proposed Text #1  ------------------------------------------------

Modify Table 788 as follows

Table 788: E-MBS MAP Structure

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| E-MBS_Zone_ID EMBS-Zone-ID | 7 | The E-MBS Zone ID to which this E-MBS MAP apply |
| MSI Length (m) | [TBD]2 | The length of an MSI is 2m+3 superframes 0b00: 2 superframes, 40 ms- 0b01: 4 superframes, 80 ms 0b10: 8 superframes, 160 ms 0b11: 16 superframes, 320 ms |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

---------------------------------------------------- Proposed Text #1  --------------------------------------------

---------------------------------------------------- Proposed Text #2 ------------------------------------------------

Modify Table 822 as follows

Table 822: E-MBS DATA Information Elements

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-_DATA_IE () { | - | - |
| No. of Multicast STIDs+FIDs | 8 | Total number of E-MBS streams in the IE |
| For (i=0; i< No. of Multicast STID +FID’s, i++){ | - |  |
| MSTID+FID | 16 | Multicast STID + Flow ID of an E-MBS sStream |
| Isize-offset | 5 | Depends on supported modes, 32modes assumed as baseline |
| E-MBS MIMO Mode MEF | [TBD] 1 | E-MBS MIMO Encoding Format 0b0: SFBC 0b1: Vertical Encoding |
| If (MEF == 0b1){ |  | Parameters of Vertical Encoding |
| Mt | 1 | Number of streams for transmission for Nt =2, Nt =4, and Nt =8 (Mt <= Nt) 0b0: 1 stream 0b1: 2 streams |
| } |  |  |
| E-MBS Frame/AAI subframe offset | [TBD] | Includes the location of the frame/subframe where the E-MBS data burst begins and Allocation size |
| E-MBS Resource Indexing | [TBD] | Includes the location of the subcarrier index where the E-MBS data burst begins and allocation size |
| Allocation Period | Variable | Inter-arrival intervals of E-MBS data bursts for an E-MBS streamFrame offset for multiple transmission instances (in number of frames) Depends on the MSI Length |
| } | - | - |
| } |  |  |

Allocation Period: its length depends on the MSI Length as follows.

| MSI Length | Size of Allocation Period |
| --- | --- |
| 0b00 | 3 bits |
| 0b01 | 4 bits |
| 0b02 | 5 bits |
| 0b03 | 6 bits |

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------