| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Zone Configuration Information Transmission (15.9.3.2) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Chun-Yen Hsu, Chiu-Wen Chen, Kanchei(Ken) Loa Institute for Information Industry (III) Kaiying Lv, Bo Sun ZTE Linghang Fan NEC | hcy@iii.org.tw |
| Re: | In response to IEEE 802.16 Working Group Working Group Letter Ballot Recirc #30a |  |
| Abstract | This contribution proposes text for E-MBS Zone Configuration Information Transmission |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS Zone Configuration Information Transmission (15.9.3.2)

Chun-Yen Hsu, Chiu-Wen Chen, Kanchei(Ken) Loa
Institute for Information Industry (III)

Kaiying Lv, Bo Sun
ZTE

Linghang Fan
NEC

Introduction

E-MBS configuration information that can facilitate continuous reception of E-MBS data or seamless inter-MBS zone transition is carried in AAI_NBR-ADV or AAI_E-MBS-CFG message. However, idle AMSs have to interpret each A-A-MAP to receive the E-MBS-related configuration information or they will miss them. In order to facilitate E-MBS reception for idle AMSs, E-MBS configuration information shall be advertised at a pre-determined location.

Proposed text for the P80216m/D2:

----------------------------------------------------Start of the Text---------------------------------------------------------

[Modify the following text in Line 63, p.584 to Line 6, p.585 as indicated:]

#### 15.9.3.2 E-MBS Zone Configuration

##### When the E-MBS Zone identifier list appears in AAI_NBR_ADV message with only one value of '0' for some of ABS's, it indicates that the corresponding neighbor ABSs are not affiliated with any E-MBS zone.

The ABS sends AAI_DSA for establishment of connection for E-MBS, E-MBS ZONE shall be encoded in the AAI-DSA message. One ABS may have multiple E-MBS Zone IDs for different E-MBS services.

To support inter-MBS zone transition, AMSs need to get E-MBS_Zone_IDs to which the neighboring ABS's belong as well as any flow continuity across such neighboring zone. E-MBS configuration information that facilitates continuous reception of E-MBS data or seamless inter-MBS zone transition is carried in AAI_E-MBS-CFG message. AAI_E-MBS-CFG is transmitted by ABS periodically. In order to facilitate E-MBS reception for idle AMSs, AAI_E-MBS-CFG shall be advertised at a pre-determined subframe/frame offset in a superframe. AAI_E-MBS-CFG message includes the AAI_E-MBS-CFG message transmission interval for AMSs to locate and decode the next AAI_E-MBS-CFG message.

An AAI_E-MBS-CFG message transmitted on a carrier shall include the E-MBS-Zone_IDs of E-MBS Zones supported by the serving ABS on the corresponding carrier.

----------------------------------------------------End of the Text---------------------------------------------------------

[1] IEEE P80216m/D2, “P802.16m/D2: Advanced Air Interface”