| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on parameters to be neogtiated by AAI_SBC-REQ/RSP (15.2.3.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Youngkyo Baek Hyunjeong Kang Yeongmoon Son Jaehyuk Jang JaeJeong Shim Mihyun Lee Rakesh Taori Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.3.3” |  |
| Abstract | This contribution provides message format for AAI_SBC-REQ/RSP. |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on parameters to be neogtiated by AAI_SBC-REQ/RSP (15.2.3.3)

Youngkyo Baek,Hyunjeong Kang,Yeongmoon Son,Jaehyuk Jang,JaeJeong Shim, Mihyun Lee, Rakesh Taori

Samsung Electronics

# Introduction

During initial network entry capability negotiation is done by AAI_SBC-REQ/RSP messages. We need to list up all parameters to be negotiated and map parametersets to capability index. At first this contribution suggests parameters to be neogotiated by AAI_SBC-REQ/RSP.

# Proposed Text

Modify the subsections 15.2.3.3 AAI_SBC-REQ and 15.2.3.4 AAI_SBC-RSP as follows.(page 39 line42)

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2.3.3 AAI_SBC-REQ

An AAI_SBC-REQ message, to which HARQ operation is applied, is transmitted by AMS to negotiate pre-authentication capability during network entry.

The AAI_SBC-REQ message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed:

The following parameters may be included and parameter sets are mapped to cabability index( the mapping is TBD):

Table xxx— parameters for AAI_ SBC-REQ

| Name |  |  | Value |
| --- | --- | --- | --- |
| Security negotiation parameters | Authorization policy support |  | If Bit #0=0, EAP-based authorization is not supported; If Bit #0=1: EAP-based authorization is supported |
|  | PN Window Size |  | Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks; Bit #0= 1 Bit #1 = 8 Bit #2 = 16 Bit #3 = 32 Bit #4 = 64 Bit #5 = 128 |
|  | Auth type for EAP |  | Auth Type for EAP shall only be included when EAP-based authorization is supported. If supported, either bit #0 or bit#1 is set to 1. Default (bit#0,bit#1)=(0,1) bit #0: device authentication bit #1: user authentication bit #2 -7: reserved |
| Physical parameters supported | OFDMA MAP capability | Bit 0: HARQ MAP Capability Bit 1: Extended HARQ IE capability Bit 2: Sub MAP capability for first zone Bit 3: Sub MAP capability for other zones Bit 4: DL region definition support Bits 5–7: Reserved |  |
|  | UL control channel support | Bit 0: 3-bit MIMO fast feedback Bit 1: Enhanced fast feedback Under negotiation for SBC fast feedback, if enhanced feature is enabled, the SS should use only the enhanced fast-feedback channel in the CQICH allocation IE (see 8.4.5.4.13 and 8.4.5.4.14). Bit 2: UL ACK Bit 3: Reserved. Shall be set to zero. Bit 4: UEP fast-feedback Bit 5: A measurement report shall be performed on the last DL burst, as described in 8.4.11.1 Bit 6: Primary/Secondary fast-feedback Bit 7: DIUC-CQI fast-feedback |  |
|  | OFDMA SS UL power control support | Bit 0: UL open-loop power control support Bit 1: UL AAS preamble power control support. Bit 2–7: Reserved; shall be set to zero |  |
|  | Delay of power control switch over | The minimum number of frames that SS takes to switch from the open-loop power control scheme to the closed-loop power control scheme or vice versa |  |
|  | Number of UL HARQ channels | If Bit#0= 0, number of UL H-ARQ channels = 4 If Bit#0= 1, number of UL H-ARQ channels = 8 |  |
|  | Number of DL HARQ channels | If Bit#0= 0, number of DL H-ARQ channels = 4 If Bit#0= 1, number of DL H-ARQ channels = 16 |  |
|  | HARQ incremental redundancy buffer capability | Bits 0–3: NEP value indicating DL HARQ buffering capability for incremental redundancy CTC. Bit 4: Aggregation flag for DL Bits 5–7: Reserved Bits 8–11: NEP value indicating UL HARQ buffering capability for incremental redundancy CTC. Bit 12: Aggregation flag for UL Bits 13–15: Reserved |  |
|  | HARQ chase combining and CC-IR buffer capability | Bits 0–5: DL HARQ buffering capability for chase combining (K) Bit 6: Aggregation flag for DL Bit 7: Reserved. Bits 8–13: UL HARQ buffering capability for chase combining (K) Bit 14: Aggregation flag for UL Bit 15: Reserved |  |
|  | Maximum number of bursts per frame capability in HARQ | Bits 0–2: One less than the maximum number of UL HARQ bursts per HARQ-enabled MS per frame. (0b000 = 1, default) Bit 3: Indicates whether the maximum number of UL HARQ bursts per frame (i.e., bits 0–2) includes the one non-HARQ burst. (0 = not included, default) Bits 4–7: One less than the maximum number of DL HARQ bursts per HARQ-enabled MS per frame. (0b0000 = 1, default) |  |
|  | OFDMA MS FFT size | Bit 0: reserved, set to zero Bit 1: 2048-FFT Bit 2: 128-FFT Bit 3: 512-FFT Bit 4: 1024-FFT Bits 5–7: Reserved, set to zero |  |
|  | OFDMA SS demodulator | Bit 0: 64-QAM Bit 1: BTC Bit 2: CTC Bit 3: STC Bit 4: CC with optional interleaver Bit 5: HARQ Chase Bit 6: HARQ CTC_IR Bit 7: Reserved; shall be set to zero Bit 8: HARQ CC IR Bit 9: LDPC Bit 10 Dedicated pilots Bits 11–15: Reserved; shall be set to zero. |  |
|  | OFDMA SS modulator | Bit# 0: 64-QAM Bit# 1: BTC Bit# 2: CTC Bit# 3: STC Bit# 4: HARQ chase Bit# 5: CTC_IR Bit# 6: CC_IR Bit# 7: LDPC |  |
|  | OFDMA SS CINR measurement capability | Bit 0: Physical CINR measurement from the preamble Bit 1: Physical CINR measurement for a permutation zone from pilot subcarriers Bit 2: Physical CINR measurement for a permutation zone from data subcarriers Bit 3: Effective CINR measurement from the preamble Bit 4: Effective CINR measurement for a permutation zone from pilot subcarriers Bit 5: Effective CINR measurement for a permutation zone from data subcarriers Bit 6: Support for 2 concurrent CQI channels Bit 7: Frequency selectivity characterization report |  |
|  | OFDMA MS CSIT capability | Bit 0: CSIT compatibility type A. Bit 1: CSIT compatibility type B. Bit 2: Power assignment capability (indicates support for nonequal power assignment) Bits 3–5: Sounding response time capability Bits 6–9: Maximum number of simultaneous sounding instructions (0 = unlimited) Bit 10: SS does not support P values of 9 and 18 when supporting CSIT type A Bits 11–15: Reserved |  |
|  | OFDMA SS demodulator for MIMO support | Bit #0 2-antenna STC matrix A Bit #1 2-antenna STC matrix B, vertical coding Bit #2 2-antenna STC matrix B, horizontal coding Bit #3: 4-antenna STC matrix A Bit #4: 4-antenna STC matrix B, vertical coding Bit #5: 4-antenna STC matrix B, horizontal coding Bit #6: 4-antenna STC matrix C, vertical coding Bit #7: 4-antenna STC matrix C, horizontal coding Bit #8: 3-antenna STC matrix A Bit #9: 3-antenna STC matrix B Bit #10: 3-antenna STC matrix C, vertical coding Bit #11: 3-antenna STC matrix C, horizontal coding Bit #12: Capable of calculating precoding weight Bit #13: Capable of adaptive rate control Bit #14: Capable of calculating channel matrix Bit #15: Capable of antenna grouping Bit #16: Capable of antenna selection Bit #17: Capable of codebook based precoding Bit #18: Capable of long-term precoding Bit #19: Capable of MIMO Midamble Bit #20: Allocation granularity in a DL PUSC STC zone with dedicated pilots Bit #21: Concurrent allocation support in a DL PUSC STC zone with dedicated pilots Bit #22: Allocation granularity in a DL AMC STC zone with dedicated pilots for Matrix B Bit #23: Concurrent allocation support in a DL AMC STC zone with dedicated pilots for bursts with different ranks |  |
|  | OFDMA SS modulator for MIMO support | Bit #0: Capable of 2-antenna STC Matrix A Bit #1: Capable of 2-antenna STC Matrix B, Vertical coding Bit #2: Capable of 2-antenna STC Matrix B, Horizontalcoding Bit #3: Capable of beamforming Bit #4: Capable of adaptive rate control Bit #5: Capable of single antenna transmission Bit #6: Capable of collaborative SM with one antenna Bit #7: collaborative SM with two antennas Bit #8: Capable of disabling UL subchannel rotation Bit #9-15: Reserved |  |
|  | SDMA pilot capability | Bits 0–1: SDMA pilot pattern support for AMC zone: 0b00 – No support 0b01 – Support SDMA pilot patterns #A and #B 0b11 – Support all SDMA pilot patterns 0b10 – Reserved Bits 2–7: Reserved |  |
|  | OFDMA multiple DL burst profile capability | Bit 0: DL burst profile for multiple FEC types (Table 428) Bit 1: UL burst profile for multiple FEC types (Table 429) Bits 2–7: Reserved; shall be set to 0 |  |
| Maximum TX power | Bit #0-#7: Maximum transmitted power for BPSK. Bit #8-#15: Maximum transmitted power for QPSK. Bit #15-#23: Maximum transmitted power for 16-QAM. Bit #24-#31: Maximum transmitted power for 64-QAM. AMSs that do not support 64-QAM shall report the value 0x00. |  |  |
| current TX power | Current transmit power |  |  |

15.2.3.4 AAI_SBC-RSP

An AAI_SBC-RSP message, to which HARQ operation is applied, is transmitted by ABS in response to a received AAI_SBC-REQ during initialization.

The AAI_SBC-RSP message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed.

The following parameters may be included and parameter sets are mapped to cabability index( the mapping is TBD):

Table yyy— parameters for AAI_ SBC-RSP

| Name |  |  | Value |
| --- | --- | --- | --- |
| Security negotiation parameters | Authorization policy support |  | Bit #0: EAP-based authorization is supported; default =1(supported) Bit#1-7 : reserved |
|  | PN Window Size |  | Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks; default =128 |
| Physical parameters supported | OFDMA MAP capability | Bit 0: HARQ MAP Capability Bit 1: Extended HARQ IE capability Bit 2: Sub MAP capability for first zone Bit 3: Sub MAP capability for other zones Bit 4: DL region definition support Bits 5–7: Reserved |  |
|  | UL control channel support | Bit 0: 3-bit MIMO fast feedback Bit 1: Enhanced fast feedback Under negotiation for SBC fast feedback, if enhanced feature is enabled, the SS should use only the enhanced fast-feedback channel in the CQICH allocation IE (see 8.4.5.4.13 and 8.4.5.4.14). Bit 2: UL ACK Bit 3: Reserved. Shall be set to zero. Bit 4: UEP fast-feedback Bit 5: A measurement report shall be performed on the last DL burst, as described in 8.4.11.1 Bit 6: Primary/Secondary fast-feedback Bit 7: DIUC-CQI fast-feedback |  |
|  | OFDMA SS UL power control support | Bit 0: UL open-loop power control support Bit 1: UL AAS preamble power control support. Bit 2–7: Reserved; shall be set to zero |  |
|  | Delay of power control switch over | The minimum number of frames that SS takes to switch from the open-loop power control scheme to the closed-loop power control scheme or vice versa |  |
|  | Number of UL HARQ channels | If Bit#0= 0, number of UL H-ARQ channels = 4 If Bit#0= 1, number of UL H-ARQ channels = 8 |  |
|  | Number of DL HARQ channels | If Bit#0= 0, number of DL H-ARQ channels = 4 If Bit#0= 1, number of DL H-ARQ channels = 16 |  |
|  | HARQ incremental redundancy buffer capability | Bits 0–3: NEP value indicating DL HARQ buffering capability for incremental redundancy CTC. Bit 4: Aggregation flag for DL Bits 5–7: Reserved Bits 8–11: NEP value indicating UL HARQ buffering capability for incremental redundancy CTC. Bit 12: Aggregation flag for UL Bits 13–15: Reserved |  |
|  | HARQ chase combining and CC-IR buffer capability | Bits 0–5: DL HARQ buffering capability for chase combining (K) Bit 6: Aggregation flag for DL Bit 7: Reserved. Bits 8–13: UL HARQ buffering capability for chase combining (K) Bit 14: Aggregation flag for UL Bit 15: Reserved |  |
|  | Maximum number of bursts per frame capability in HARQ | Bits 0–2: One less than the maximum number of UL HARQ bursts per HARQ-enabled MS per frame. (0b000 = 1, default) Bit 3: Indicates whether the maximum number of UL HARQ bursts per frame (i.e., bits 0–2) includes the one non-HARQ burst. (0 = not included, default) Bits 4–7: One less than the maximum number of DL HARQ bursts per HARQ-enabled MS per frame. (0b0000 = 1, default) |  |
|  | OFDMA MS FFT size | Bit 0: reserved, set to zero Bit 1: 2048-FFT Bit 2: 128-FFT Bit 3: 512-FFT Bit 4: 1024-FFT Bits 5–7: Reserved, set to zero |  |
|  | OFDMA SS demodulator | Bit 0: 64-QAM Bit 1: BTC Bit 2: CTC Bit 3: STC Bit 4: CC with optional interleaver Bit 5: HARQ Chase Bit 6: HARQ CTC_IR Bit 7: Reserved; shall be set to zero Bit 8: HARQ CC IR Bit 9: LDPC Bit 10 Dedicated pilots Bits 11–15: Reserved; shall be set to zero. |  |
|  | OFDMA SS modulator | Bit# 0: 64-QAM Bit# 1: BTC Bit# 2: CTC Bit# 3: STC Bit# 4: HARQ chase Bit# 5: CTC_IR Bit# 6: CC_IR Bit# 7: LDPC |  |
|  | OFDMA SS CINR measurement capability | Bit 0: Physical CINR measurement from the preamble Bit 1: Physical CINR measurement for a permutation zone from pilot subcarriers Bit 2: Physical CINR measurement for a permutation zone from data subcarriers Bit 3: Effective CINR measurement from the preamble Bit 4: Effective CINR measurement for a permutation zone from pilot subcarriers Bit 5: Effective CINR measurement for a permutation zone from data subcarriers Bit 6: Support for 2 concurrent CQI channels Bit 7: Frequency selectivity characterization report |  |
|  | OFDMA MS CSIT capability | Bit 0: CSIT compatibility type A. Bit 1: CSIT compatibility type B. Bit 2: Power assignment capability (indicates support for nonequal power assignment) Bits 3–5: Sounding response time capability Bits 6–9: Maximum number of simultaneous sounding instructions (0 = unlimited) Bit 10: SS does not support P values of 9 and 18 when supporting CSIT type A Bits 11–15: Reserved |  |
|  | OFDMA SS demodulator for MIMO support | Bit #0 2-antenna STC matrix A Bit #1 2-antenna STC matrix B, vertical coding Bit #2 2-antenna STC matrix B, horizontal coding Bit #3: 4-antenna STC matrix A Bit #4: 4-antenna STC matrix B, vertical coding Bit #5: 4-antenna STC matrix B, horizontal coding Bit #6: 4-antenna STC matrix C, vertical coding Bit #7: 4-antenna STC matrix C, horizontal coding Bit #8: 3-antenna STC matrix A Bit #9: 3-antenna STC matrix B Bit #10: 3-antenna STC matrix C, vertical coding Bit #11: 3-antenna STC matrix C, horizontal coding Bit #12: Capable of calculating precoding weight Bit #13: Capable of adaptive rate control Bit #14: Capable of calculating channel matrix Bit #15: Capable of antenna grouping Bit #16: Capable of antenna selection Bit #17: Capable of codebook based precoding Bit #18: Capable of long-term precoding Bit #19: Capable of MIMO Midamble Bit #20: Allocation granularity in a DL PUSC STC zone with dedicated pilots Bit #21: Concurrent allocation support in a DL PUSC STC zone with dedicated pilots Bit #22: Allocation granularity in a DL AMC STC zone with dedicated pilots for Matrix B Bit #23: Concurrent allocation support in a DL AMC STC zone with dedicated pilots for bursts with different ranks |  |
|  | OFDMA SS modulator for MIMO support | Bit #0: Capable of 2-antenna STC Matrix A Bit #1: Capable of 2-antenna STC Matrix B, Vertical coding Bit #2: Capable of 2-antenna STC Matrix B, Horizontalcoding Bit #3: Capable of beamforming Bit #4: Capable of adaptive rate control Bit #5: Capable of single antenna transmission Bit #6: Capable of collaborative SM with one antenna Bit #7: collaborative SM with two antennas Bit #8: Capable of disabling UL subchannel rotation Bit #9-15: Reserved |  |
|  | SDMA pilot capability | Bits 0–1: SDMA pilot pattern support for AMC zone: 0b00 – No support 0b01 – Support SDMA pilot patterns #A and #B 0b11 – Support all SDMA pilot patterns 0b10 – Reserved Bits 2–7: Reserved |  |

--------------------------------------------------- End of Proposed Text------------------------------------------------------