| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message format of AAI_RES-CMD and AAI_SII-ADV (15.2.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Youngkyo Baek Hyunjeong Kang Jungje Son Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.3” |  |
| Abstract | This contribution proposes the message format about AAI_RES-CMD and AAI_SII-ADV to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Message format of AAI_RES-CMD and AAI_SII-ADV (15.2.3)

Youngkyo Baek, Hyunjeong Kang, Jungje Son

Samsung Electronics

# 1. Introduction

AAI_RES-CMD and AAI_SII-ADV are listed as MAC control messages, but they have no definition yet.

AAI_RES-CMD is used for ABS to force a AMS to reset itself, reinitialize its MAC and initial system access.

AAI_SII-ADV is used to broadcast a list of Network Service Provider (NSP) Identifiers.

# 2. Text Proposal

Inset the proposed subclauses at page 93,line 35 as follows

======================== Start of Proposed Text =====================

15.2.3 MAC control messages

……………

15.2.3.41AAI_RES-CMD( reset command)

The AAI_RES-CMD message shall be transmitted by the ABS to force the AMS to reset itself, reinitialize its MAC, and repeat initial system access. This message may be used if an AMS is unresponsive to the ABS or if the ABS detects continued abnormalities in the UL transmission from the AMS. It contains no information except MAC message name.

15.2.3.42 AAI_SII-ADV(Service Identity Information)

An ABS may use the AAI_SII-ADV message to broadcast a list of Network Service Provider (NSP) Identifiers. Assignment method, administration, and usage of NSP IDs are outside the scope of this standard. The list of NSP IDs to be included in this message and the message transmission frequency are programmable. The following parameters may be included in the AAI_SII-ADV message; at least one shall be included in an AAI_SII-ADV message:

| Name | Value | Usage |
| --- | --- | --- |
| NSP List | List of one or more Network Service Provider 24 bit Identifiers. |  |
| Verbose NSP Name List | List of the verbose names of the NSPs. The value of Verbose NSP Name List is a compound list of verbose NSP name lengths and verbose NSP names. The order of the Verbose NSP Name Lengths and Verbose NSP Names presented shall be in the same order as the NSP IDs presented in the NSP List. | It shall only be included in the message if NSP List is included in the message |

============================== End of Proposed Text ===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”