| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Key usage during 16m-16e zone switching (15.2.5.2.4) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.5.2.4” |  |
| Abstract | This contribution proposes modification on key update section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Key usage during 16m-16e zone switching (15.2.5.2.4)

Youngkyo Baek, Jicheol Lee

Samsung Electronics

# 1. Introduction

Zone switching procedure requires security update to achieve seamless operation. Due to difference between key derivation of 16e and that of 16m, we need a mechanism to share security keys(e.g. PMK, AK, CMAC, TEK, etc).

We suggest deriving new PMK in case of zone switch procedure from Lzone to Mzone, and reusing PMK which is used the previous Mzone in case of zone switch procedure from Mzone to Lzone.

# 2. Text Proposal

Inset the proposed text just before the subclause ‘15.2.5.2.5 SA Management’ (page 111,line 10) as follows

======================== Start of Proposed Text #1=====================

15.2.5.2.4.5 Key usage during Location Update and Network re-entry from Idle mode

…………..

15.2.5.2.4.6 Key update during zone switching from LZone to MZone

ABS shall include Nonce_BS in the zone switch information.

AMS shall perform key agreement and network reentry procedure in MZone to derive new PMK, AK, CMAC keys and TEKs to be used in MZone as follows.

- AMS derives new PMK, the NONCE_BS, transmitted by zone switching information, and NONCE_MS.( key agreement MSG#1 is omitted since NONCE_BS is already shared.)

- AMS derives new AK and its CMAC key based on the new PMK.

- AMS sends AAI_RNG-REQ message containing key agreement MSG#2 attributes (e.g. NONCE_BS, NONCE_MS and CMAC digest, which is based on new CMAC key.)

- New PMK, AK, CMAC keys and TEKs are derived at the network side. ABS validates the AAI_RNG-REQ.

- ABS responses with AAI_RNG-RSP message containing key agreement MSG#3 attributes(e.g. NONCE_BS and NONCE_MS) and the AAI_RNG-RSP is encrypted by new TEK.

15.2.5.2.4.7 Key update during zone switch from MZone to LZone

AMS shall increment CMAC_KEY_COUNT and, based on the current active PMK, derive new AK, KEK, CMAC keys according to Section 7.2.2.2. New TEKs are derived according to Section 7.2.2.2 if in AAI_HO-CMD message HO process optimization bit #2 = 1 (Seamless handover). Otherwise TEKs to be used in LZone are obtained via TEK transfer encrypted by KEK. If Zone-Switching-Mode=1, the AMS shall also manage the old security context used to maintain communications in MZone before zone switching to LZone finishes.

15.2.5.2.5 SA Management

…………..

============================== End of Proposed Text #1===============

Modify the table 674 Parameters for AAI_ RNG-REQ (page 37,line 43) as follows

======================== Start of Proposed Text #2=====================

Table 674—Parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| …….. | ……….. | ……….. |
| Power Down Indicator | ……….. | ……….. |
| NONCE_MS | A freshly generated 32-bit random number used for PMK derivation | It shall be included when PMK is updated. |
| NONCE_BS | A 32-bit number transferred from ABS and used for PMK derivation |  |
| CMAC_KEY_COUNT | ……….. | ……….. |
| …….. | ……….. | ……….. |

============================== End of Proposed Text #2===============

Modify the table 675 Parameters for AAI_ RNG-RSP (page 39,line 39) as follows

======================== Start of Proposed Text #3=====================

Table 675—Parameters for AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| …….. | ……….. | ……….. |
| Neighbor station measurement report indicator | ……….. | ……….. |
| NONCE_MS | The 32-bit NONCE_MS transferred by AAI_RNG-REQ | It shall be included when PMK is updated. |
| NONCE_BS | The 32-bit NONCE_BS transferred by AAI_RNG-REQ |  |

============================== End of Proposed Text #3===============

Modify the sentence (page 134,line 58) as follows

======================== Start of Proposed Text #4=====================

The Zone Switch TLV shall include the following:

- MZone A-Preamble index.

- Time offset between LZone and Mzone

- Action Time: Action time of zone switch from LZone to MZone. AMS performs zone switch at Action Time. If HO_Reentry_Mode=0, ABS stops all resource allocation for the AMS at LZone.

- Zone Switch Mode: If set to 1, the AMS maintains its data communication with the ABS in LZone while performing network reentry in MZone; otherwise it breaks data communication in LZone before performing network reentry in MZone.

- NONCE_BS: to derive AMSID*. PMK is shared between LZone and MZone during zone switch. it is used to derive new PMK to be used in Mzone

============================== End of Proposed Text #4===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”