| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to Table 832 Section 15.3.7.2.5.5 (DL MIMO Control Parameters) for the IEEE 802.16m/D2 |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Alexei Davydov, Huaning Niu, Yuan Zhu Intel | alexei.davydov@intel.com |
| Re: | Category: LB30a / Area: Chapter 15.3.7 (DL MIMO) |  |
| Abstract | The contribution proposes correction to the Table 832 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Corrections to Table 832 Section 15.3.7 (DL MIMO Control Parameters) for the IEEE 802.16m/D2

Alexei Davydov, Huaning Niu, Yuan Zhu

Intel Corporation

# Text proposal

<Modify contents of Table 832 DL MIMO Control Parameters on page 416 line 52>

--------------------------------------------------  Text Start  ------------------------------------------------

| Feedback Allocation IE |  |  |  |  |
| --- | --- | --- | --- | --- |
| MFM | MIMO Feedback mode | Refer to Table 691 |  | To decide the feedback content and related MS processing |
| MaxMt | Maximum Number of MIMO streams | If MFM indicates a SU feedback mode: 1, 2, 4 or 8 0, 1, 2, 3 or 4. If MFM indicates a MU feedback mode: 1, 2, 3, or 48 4, 5, 6 or 7. |  | If MFM indicates a SU feedback mode: the maximum number of MIMO streams STC rate scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| ... |  |  |  |  |

---------------------------------------------------  Text End  ---------------------------------------------------