| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the Ranging Channel for Non-Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.2.4.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon LG Electronics | Voice: +82-31-450-7902 E-mail: camille@lge.com, samji@lge.com, hgcho@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a P802.16m/D2 : 15.3.9.2.4.1 |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE P802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The clarification of the Ranging Channel for Non-Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.2.4.1)

HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, and Young-Hyoun Kwon

LG Electronics

# 1. Introduction

This contribution proposes the modified text of ranging channel section in the IEEE P802.16m/D2 [1].

# 2. Proposed Changes

This contribution proposes the text of ranging channel code and configuration for clarification.

# Reference

[1] IEEE P802.16m/D2, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” October 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 15.3.9.2.4.1 of P802.16m/D2

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

# Ranging channel for non-synchronized AMSs

# Ranging Preamble codes

The ranging preamble codes are classified into initial ranging and handover ranging preamble codes. The initial ranging preamble codes shall be used for initial network entry and association. Handover ranging preamble codes shall be used for ranging against a target ABS during handover. For a ranging code opportunity, each AMS randomly chooses one of the ranging preamble codes from the available ranging preamble codes set in a cell, except that in the handover ranging case where a dedicated ranging code is assigned, the AMS shall use the assigned dedicated preamble code.

The Zadoff-Chu sequences with cyclic shifts are used for the ranging preamble codes. The pth ranging preamble code xp(k) is defined by

(254)

where

p	is the index for pth ranging preamble code which is made as the spth cyclic shifted sequence from the root index rp of Zadoff-Chu sequence.

(255)

(256)

where rns0 is broadcasted in the S-SFH and Mns is the number of cyclic shifted codes per ZC root index according to Table 876. NTOTAL is the total number of initial (0~NIN -1) and handover preamble codes (NIN ~NIN +NHO -1) per sector.

NCS	is the unit of cyclic shift according to the cell size. It is defined by  where Mns is the number of cyclic shifted codes per ZC root index according to Table 876.

NRP	is the length of ranging preamble codes defined as NRP =139 for ranging channel Format 0 in Table 871 and NRP =557 for ranging channel Format 1 in Table 871.

The number of cyclic shifted codes per root index (MMns), the start root index of ZC code (rns0), and the ranging preamble code partition information are broadcasted by S-SFH. The number of cyclic shifted codes per root index is defined in Table 876. The start root index of ZC code is defined by rns0=4×kns+1 (=1, 5, 9, …, or 6561) for Format 0 and rns0=16×kns+1 (=1, 17, 33, …, or 241) for Format 1 where kns is a cell specific value broadcasted by S-SFH. The ranging preamble code partition information indicate the number of initial and handover ranging preamble codes and is defined in Table 877.

Table 876 - The number of cyclic shifted codes per ZC root index, Mns

| index | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Mns | 1 | 2 | 4 | 8 |

Table 877 - Ranging preamble code partition information table, NIN and NHO

| Partition Index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of initial ranging preamble codes, NIN | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 32 | 32 | 32 | 32 |
| Number of handover ranging preamble codes, NHO | 8 | 16 | 24 | 32 | 8 | 16 | 24 | 32 | 8 | 16 | 24 | 32 | 8 | 16 | 24 | 32 |

# Ranging channel configurations

The information for ranging time resource allocation is indicated by the S-SFH in a regular allocation. The information of ranging channel allocation consists of the ranging configuration with AAI subframe-offset (OSF) for ranging resource allocation in the time domain. The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDCell and the allocated number of subbands KSB according to the Equation (264) where IDCell is defined in 15.3.6.1.2 and KSB is defined in 15.3.8.2.1.

ISB = mod(IDCell, KSB) 			                  	(257)

where ISB denotes the subband index (0,…, KSB-1) for ranging resource allocation among KSB subbands.

Table 878 shows the information of ranging channel allocation in a regular allocation, which is indicated by the S-SFH.

Table 878 - Ranging channel allocations by S-SFH

| Configurations | The AAI subframe allocating Ranging channel (1) |
| --- | --- |
| 0 | mod(OSF, NUL(2))th UL AAI subframe in every frame |
| 1 | mod (OSF, NUL)th UL AAI subframes in the first frame in every superframe |
| 2 | mod (OSF, NUL)th UL AAI subframe in the first frame in every even numbered superframe, i.e., mod(superframe number, 2)=0 |
| 3 | mod (OSF, NUL)th UL AAI subframe of the first frame in every 4th superframes, i.e., mod(superframe number, 4)=0 |

- It indicates the AAI subframe allocating ranging channel for Format 0 and 1. For Format 1, it indicates the starting AAI subframe for allocating ranging channel AAI subframe in contiguous 3 AAI subframes.

- NUL is the number of UL AAI subframes per frame.

The ranging channel for handover ranging can also be allocated by AMAP based on ABS scheduling decision in any subframe, except the subframe that has already been used for a regular allocation.

# Ranging signal transmission

Equation (265) specifies the transmitted signal voltage to the antenna, as a function of time, during ranging channel format 0 or 1.

(265)

where

t		is the elapsed time since the beginning of the subject ranging channel., with 0<t<(TRCP+2×TRP) for Format 0 or 0<t<(TRCP+TRP) for Format 1.

NRP		is the length of ranging preamble code in frequency domain.

xp(n)		is the p-th ranging preamble code with length NRP.

Koffset		is the parameter related to the frequency position and is defined by

NPRU	is the total number of PRUs as defined <<15.3.8.2.1 Subband partitioning>>.

k0	is the lowest PRU index of the assigned ranging channel.

Psc	is the number of the consecutive subcarriers within a PRU in frequency domain as defined in <<15.3.8.1 Physical and logical resource unit>>.

is the ranging subcarrier spacing.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------