| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Ranging Channel Structure for Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.1.4.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon LG Electronics Xinrong Wang and Yang-Seok Choi Intel Corporation Pei-Kai Liao, Yih-Shen Chen, Paul Cheng MediaTek Inc. Zheng Yan-Xiu ITRI Hwasun Yoo and Chiwoo Lim Samsung Electronics Co., Ltd. | Voice: +82-31-450-7902 E-mail: camille@lge.com, samji@lge.com, hgcho@lge.com {xinrong.wang; yang-seok.choi}@intel.com pk.liao@mediatek.com zhengyanxiu@itri.org.tw hwasun.yoo@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a P802.16m/D2 : 15.3.9.1.4.2 |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE P802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Ranging Channel Structure for Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.1.4.2)

HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, and Young-Hyoun Kwon

LG Electronics

Xinrong Wang and Yang-Seok Choi

Intel Corporation

Pei-Kai Liao, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

Zheng Yan-Xiu

ITRI

Hwasun Yoo and Chiwoo Lim

Samsung Electronics Co., Ltd.

# 1. Introduction

This contribution proposes the modified text of ranging channel section in the IEEE P802.16m/D2 [1].

# 2. Proposed Changes

This contribution proposes the text of ranging channel structure for synchronized AMSs.

# Reference

[1] IEEE P802.16m/D2, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” October 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 15.3.9.1.4.2 of P802.16m/D2

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

# Ranging Channel Structure for synchronized AMSs

The ranging channel for synchronized AMSs is used for periodic ranging. The AMSs that are already synchronized to the target ABS are allowed to transmit the periodic ranging signal. ForIn a femtocell, the ranging channel for synchronized AMSs shall perform can be used for initial ranging, handover ranging, and periodic ranging by using the ranging channel for synchronized AMSs.

The physical structure in the ranging channel for synchronized AMSs occupies 72 subcarriers by K6 OFDMA symbols time. starting from the first OFDMA symbol within a subframe, where there are two repeated signal waveforms and each signal waveform as a basic unit is generated by the ranging preamble code over 72 subcarriers by 3 OFDMA symbols.

Figure 551552 illustrates the basic unitphysical structure with a basic unit of ranging channel structure in the time domain where Tg and Tb are defined in <<<15.3.2.4.>>>.

Figure 552 – Ranging channel structure for synchronized AMSs in the time domain

Power control operation described in <<<15.3.9.4.3>>> applies to ranging signal transmission.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------