| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Ranging Channel Code and Configuration for Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.2.4.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon LG Electronics Xinrong Wang and Yang-Seok Choi Intel Corporation Pei-Kai Liao, Yih-Shen Chen, Paul Cheng MediaTek Inc. Zheng Yan-Xiu ITRI Hwasun Yoo and Chiwoo Lim Samsung Electronics Co., Ltd. | Voice: +82-31-450-7902 E-mail: camille@lge.com, samji@lge.com, hgcho@lge.com {xinrong.wang; yang-seok.choi}@intel.com pk.liao@mediatek.com zhengyanxiu@itri.org.tw hwasun.yoo@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a P802.16m/D2 : 15.3.9.2.4.2 |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE P802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Ranging Channel Code and Configuration for Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.2.4.2)

HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, and Young-Hyoun Kwon

LG Electronics

Xinrong Wang and Yang-Seok Choi

Intel Corporation

Pei-Kai Liao, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

Zheng Yan-Xiu

ITRI

Hwasun Yoo and Chiwoo Lim

Samsung Electronics Co., Ltd.

# 1. Introduction

This contribution proposes the modified text of ranging channel section in the IEEE P802.16m/D2 [1].

# 2. Proposed Changes

This contribution proposes the text of ranging channel code and configuration for synchronized AMSs.

# Reference

[1] IEEE P802.16m/D2, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” October 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 15.3.9.2.4.2 of P802.16m/D2

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

# 15.3.9.2.4.2.        Ranging channel for synchronized AMSs

# Ranging Preamble Codes

The Padded Zadoff-Chu codes with cyclic shifts are used for the ranging preamble basic codes. The pth ranging preamble code xp(n, k) for nth OFDMA symbol within a basic unit is defined by

(259)

where

p	is the index for pth ranging preamble code for nth OFDMA symbols within a basic unit which is made as the sp th cyclic shifted sequence from the root index rp of Zadoff-Chu sequence.

(e.1)

sp=mod(p,Ms),p=0,1,...,NTOTAL−1                                         (e.2)

The start root index, rs0, is broadcasted. Ms is the number of cyclic shift per ZC root index and defined by Ms=1G. NTOTAL is the number of periodic ranging preamble codes (0~NPE -1) per sector, which is defined by Table (t.1). For femtocell, NTOTAL is the total number of initial (0~NIN -1), handover (NIN ~NIN +NHO -1) and periodic ranging preamble codes (NIN +NHO ~ NIN +NHO+NPE -1) per sector which is defined by Table 738 and Table (t.12).

m	is the unit of cyclic shift.

NTCS	is the unit of time domain cyclic shift per OFDMA symbol according to the CP length and defined by , where G and NFFT are the CP ratio and FFT size defined in Table 762, respectively.

NRP	is the length of ranging preamble codes per OFDMA symbol, i.e., NRP =72.

Cq(n)	is [TBD]

The start root index of ZC code (rs0rs0), and the ranging preamble code information are broadcasted by additional broadcast information. The start root index of ZC code is defined by rs0=6×ks+1 where ks is a cell specific value. The ranging preamble code information indicates the number of periodic ranging preamble codes. and is defined by Table (t.1). For femtocell, the ranging preamble code partition information indicate the number of initial, handover and periodic ranging preamble codes and is defined in Table (t.2).

Table (t.1) - Ranging preamble code information table, NPE

| index | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Number of periodic ranging preamble codes, NPE | 8 | 16 | 24 | 32 |

Table (t.2) - Ranging preamble code partition information table for Femtocell, NIN, NHO and NPE

| Partition Index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of initial ranging preamble codes, NIN | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 |
| Number of handover ranging preamble codes, NHO | 4 | 8 | 16 | 24 | 4 | 8 | 16 | 24 | 4 | 8 | 16 | 24 | 4 | 8 | 16 | 24 |
| Number of periodic ranging preamble codes, NPE | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 |

Ranging channel configurations

The information of ranging channel allocation consists of the ranging configuration with AAI subframe-offset (OSF) for ranging resource allocation in the time domain where OSFOSF is same AAI subframe-offset of ranging channel for non-synchronized AMSs defined in 15.3.9.2.4.1. The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDCell and the allocated number of subbands KSB according to the Equation (e.3) where IDCell is defined in 15.3.6.1.2 and KSB is defined in 15.3.8.2.1.

ISB, s = mod( IDCell+1, KSB) 			                               	(e.3)

where ISB, s denotes the subband index (0,…, KSB-1) for ranging resource allocation among KSB subbands.

Table 879 shows the information of ranging channel allocation when number of UL AAI subframe per frame (NUL) is larger than 1where NUL is the number of UL AAI subframe per frame.

Table 879 - Ranging channel allocations for synchronized AMSs

| Configurations | The subframe allocating Ranging channel |
| --- | --- |
| 0 | mod(OSF+1, NUL)th UL AAI subframe in every frame |
| 1 | mod(OSF+1, NUL)th UL AAI subframes in the firstsecond frame in every superframe |
| 2 | mod(OSF+1, NUL)th UL AAI subframe in the firstsecond frame in every [TBD]4th superframe, i.e., mod(superframe number, 4)=0 |
| 3 | mod(OSF+1, NUL)th UL AAI subframe of the firstsecond frame in every [TBD]8th superframe, i.e., mod(superframe number, 8)=0 |

When KSB=1 and NUL=1, the configuration 0 shall be not used for ranging channel allocation for synchronized AMSs.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------