| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification on CMAC_KEY_COUNT management (15.2.5.2.1.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Samsung Electronics Avishay Shraga Xiangying Yang Intel | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.5.2.1.3” |  |
| Abstract | This contribution proposes the texts for CMAC_KEY_COUNT management subclause to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CMAC_KEY_COUNT management (15.2.5.2.1.3)

Youngkyo Baek, Jicheol Lee

Samsung Electronics

Avishay Shraga, Xiangying Yang

Intel

# 1. Introduction

Per current 16mD2, 16m’s CMAC_KEY_COUNT management follows 16e. ( see 7.2.2.2.6.1 and 7.2.2.2.9.1 in [1].)

However, the PKM messages and key derivation is changed from 16e. Especially PMK and AK are derived during key agreement procedure and the AK depends on the CMAC_KEY_COUNT.

Especially, we can expect two options as follows.

- When ABS receives AAI_RNG-REQ containing CMAC_KEY_COUNT and CMAC tuple, if it has no AK context, it asks authenticator for the AK context based on AK_SN and CMAC_KEY_COUNTM. (cf. in 16e, AK is derived based on the AK_SN independently of CMAC_KEY_COUNTM)  The authenticator has to compare CMAC_KEY_COUNTM with CMAC_KEY_COUNTN.  If the counter is available, the authenticator responds with AK context. If not, the authenticator shall let the ABS send an AAI_RNG-RSP message requesting re-authentication.

- In another way, when ABS receives AAI_RNG-REQ containing CMAC_KEY_COUNT and CMAC tuple, if it has no AK context, it asks authenticator for the AK context and the authenticator derives AK based on AK_SN and CMAC_KEY_COUNTN.  But, if CMAC_KEY_COUNTN is not equal to CMAC_KEY_COUNTM, the ABS has to request AK context again based on CMAC_KEY_COUNTM. So after obtaining AK context, the ABS can validate CMAC. It seems not better than the first way.

Therefore, we suggest that CMAC_KEY_COUNT management reflect 16m key derivation and AK context include CMAC_KEY_COUNT used to derive the AK as the following text proposals.

Additionally, CMAC KEY COUNT takes effects on the AK , CMAC key and TEK. So we suggest replacing CMAC_KEY_COUNT with AK _COUNT.

For comparison, refer to Figure 1 for 16e scheme and Figure 2 for proposed 16m scheme.

Figure 1. CMAC KEY COUNT management in 16e

Figure 2. proposed CMAC KEY COUNT management in 16m

# 2. Text Proposal

Modify the texts (page 104,line 6) as the following proposed text.

======================== Start of Proposed Text #1=====================

15.2.5.2.1.3 AK derivation

……………….

The AK derivation is done:

AK = Dot16KDF (PMK, MSID*|BSID| AKCMAC _KEY_COUNT|”AK”, 160)

……………….

•AKCMAC_KEY_COUNT – a counter which is used to ensure different AKs for the same BS-MS pairs across handovers, the counter is managed as described in 7.2.2.2.9.1 and 7.2.2.2.6.1.

……………….

The AK lifetime is identical to the PMK lifetime.

15.2.5.2.1.3.1 AK_COUNT_ management

The AMS shall maintain an AK_COUNT counter for each PMK context, and the Authenticator is assumed to maintain an AK_ COUNT counter for each PMK context, which is normally kept synchronized with the corresponding counter at the AMS.

Key count management (i.e CMAC_KEY_COUNT and AK_COUNT) during zone switching is TBD.

The value of this counter maintained by the AMS is denoted as AK_ COUNTM and the value maintained by the Authenticator is denoted as AK_ COUNTN. Each AK context that an ABS maintains has an AK_COUNT value, which is denoted AK_COUNTB.

15.2.5.2.1.3.1.1 Maintenance of AK_COUNTM by the AMS

Upon successful completion of the key agreement, the AMS shall derive a new PMK and initiate a new AK_COUNT counter and set its value to zero. In particular, this shall occur upon reception of the key_agreement-MSG#1 message. The AMS shall initiate either re-authentication or new key agreement before the AK_COUNTM reaches its maximum value of 65535. The AMS shall manage a separate AK_COUNTM counter for every active PMK context.

Specifically, during re-authentication or key agreement, the old AK_COUNTM (corresponding to the old PMK) shall be used for CMAC generation of MAC control messages before the new PMK and AK are activated, while the new AK_COUNTM shall be used for CMAC generation for key_agreement-MSG#2 and key_agreement-MSG #3 messages.

15.2.5.2.1.3.1.1.1 AK_COUNT LOCK state

When the AMS decides to reenter the network or perform Secure Location Update (immediately prior to transmitting an AAI_RNG-REQ for re-entry or Secure Location Update to a first preferred ABS), or handover to a target ABS (immediately prior to transmitting AAI_RNG-REQ for handover to a first target ABS. in case of seamless HO, immediately prior to access the target ABS), the AMS shall perform the following steps in the stated order:

1) If the AMS is handing over to a target ABS, it shall cache the current AK context and SA context used at the serving BS.

2) The AMS shall increment once the AK_COUNTM counter.

3) The AMS shall enter the AK_COUNT LOCK state.

For each ABS to which it sends an AAI_RNG-REQ message for the first time while in the AK_COUNT LOCK state, the AMS shall derive new AK context and SA context based on the AK_COUNTM value.

While in the AK_COUNT LOCK state, the AMS shall cache the AK context and SA context corresponding to each preferred or target ABS to which it has sent an AAI_RNG-REQ message. The AMS shall update and use these cached values for any subsequent message exchange with the same target or preferred ABS while in the AK_COUNT LOCK state.

When the AMS has completed network reentry at a preferred ABS or has completed handover to a target ABS (in either case establishing the preferred ABS or target ABS as the new serving ABS) or the AMS has completed Secure Location Update, or the AMS cancels handover and remains connected to its current serving ABS, the AMS shall exit the AK_COUNT LOCK state.

Upon exit of the AK_COUNT LOCK state, the AMS may purge the cached AK context and SA context for all ABSs other than the serving ABS.

15.2.5.2.1.3.1.2 Processing of AK_COUNTB by the ABS (informative)

The ABS may possess one or more AK contexts associated with the AMS, each of which includes the value of AK_COUNTB. This value shall be maintained as specified in subsequent paragraphs of this section.

Upon successful completion of the (re)authentication, the ABS shall obtain new AK during key agreement procedure. In particular, this shall occur upon reception of the key_agreement-MSG#2 message. The ABS shall manage a separate AK_COUNTB for every AK context it is maintaining.

Specifically, during re-authentication or PMK update, the old AK_COUNTB (corresponding to the old PMK) shall be used for CMAC generation of MAC control messages before the new AK is activated, while the new AK_COUNTB shall be used for CMAC generation for key_agreement-MSG#2 and key_agreement-MSG #3 messages.

Upon receiving the AAI_RNG-REQ message from the AMS containing the AK_COUNT, the ABS shall compare the received AK_COUNT value, which is AK_COUNTM, with AK_COUNTB, if ABS has AK context.

- If AK_COUNTM < AK_COUNTB, the ABS shall process the message as having an invalid CMAC tuple and send an AAI_RNG-RSP message requesting re-authentication; see subclauses 6.3.23.8.2.1 and 6.3.21.2.7.

- If AK_COUNTB < AK_COUNTM, the ABS shall request and get an AK context, which corresponds to both AK SN in the CMAC tuple and AK_COUNT M, from the authenticator. ABS shall generate the CMAC_KEY_* and validate the received AAI_RNG-REQ message. If it is valid, the ABS shall set AK_COUNTB = AK_COUNTM, update AK context and SA context based on the AK, and send the AMS an AAI_RNG-RSP message encrypted by the newly generated TEK.

- If the CMAC value is not valid, the ABS shall send an AAI_RNG-RSP message requesting re-authentication; refer to subclauses 6.3.23.8.2.1 and 6.3.21.2.7.

- If AK_COUNTB = AK_COUNTM, the ABS shall validate the received AAI_RNG-REQ using the cached AK context. If the CMAC value is valid, the ABS shall send the encrypted AAI_RNG-RSP message to the AMS allowing legitimate entry. If the CMAC value is invalid, the ABS shall send an AAI_RNG-RSP message requesting re-authentication; refer to subclauses 6.3.23.8.2.1 and 6.3.21.2.7.

Upon receiving the AAI_RNG-REQ message from the AMS containing the AK_COUNT, if ABS has no AK context, it shall request an AK context, which corresponds to both AK SN in the CMAC tuple and AK_COUNT M, to the authenticator.

- If AK_COUNTM < AK_COUNTN, the authenticator shall let the ABS send an AAI_RNG-RSP message requesting re-authentication; see subclauses 6.3.23.8.2.1 and 6.3.21.2.7.

- If AK_COUNTM ≥AK_COUNTN, the authenticator shall reply AK context to the ABS. ABS shall generate the CMAC_KEY_* and validate the received AAI_RNG-REQ message. If it is valid, the ABS shall set AK_COUNTB = AK_COUNTM, update AK context and SA context based on the AK, and send the AMS an AAI_RNG-RSP message encrypted by the newly generated TEK. If the CMAC value is not valid, the ABS shall send an AAI_RNG-RSP message requesting re-authentication; refer to subclauses 6.3.23.8.2.1 and 6.3.21.2.7.

During seamless HO preparation, ABS obtains AK context based on AK_COUNTN from the authenticator and the ABS shall set AK_COUNTB = AK_COUNTN.

Once the AMS has completed network re-entry, cancelled handover, or completed Secure Location Update, the ABS is assumed to inform the Authenticator and send to it the value of AK_COUNTM.

The ABS shall cache the AK context in case it receives subsequent MAC management messages from the AMS. When the ABS can determine that the AMS has exited the AK_COUNT LOCK state associated with AK_COUNTM and if it is not serving the AMS, it may purge the cached AK context.

Figure xxx. AK_ COUNT management

============================== End of Proposed Text #1===============

Modify the texts (page 110, line 34) as the following proposed text.

======================== Start of Proposed Text #2=====================

•In AK derivation, the AKCMAC_KEY_COUNT is managed on AMS and target ABS sides as described in Section 7.2.2.2.6.1 and 7.2.2.2.9.1 15.2.5.2.1.3.1.

============================== End of Proposed Text #2===============

Modify the texts (page 111, line 5) as the following proposed text.

======================== Start of Proposed Text #3=====================

•In AK derivation, the AKCMAC_KEY_COUNT is managed on AMS and target ABS sides as described in Section 7.2.2.2.9.1 15.2.5.2.1.3.1.

============================== End of Proposed Text #3===============

Modify the table 718 (page 119, line 5) as the following proposed text.

======================== Start of Proposed Text #4=====================

Table 718—The AK context

| Parameter | Size(bits) | Usage |
| --- | --- | --- |
| ………….. | ………….. | ………….. |
| CMAC_PN_U | 24 | Used to avoid UL replay attack on the management connection before this expires, reauthorization is needed. The initial value of CMAC_PN_U is zero and the value of CMAC_PN_U is reset to zero whenever AKCMAC _KEY _COUNT is increased. |
| AK_COUNT | 16 | A value used to derive the AK |
| CMAC_PN_D | 24 | Used to avoid DL replay attack on the management connection before this expires, reauthorization is needed. The initial value of CMAC_PN_D is zero and the value of CMAC_PN_D is reset to zero whenever AKCMAC _KEY _COUNT is increased. |
| ………….. | ………….. | ………….. |

============================== End of Proposed Text #4===============

Modify the table 674 (page 37, line 45) as the following proposed text.

======================== Start of Proposed Text #5=====================

Table 674—Parameters for AAI_ RNG-REQ

| Name | value | Usage |
| --- | --- | --- |
| ………….. | ………….. | ………….. |
| AKCMAC _KEY _COUNT | The AMS’s current value of the AKCMAC _KEY _COUNT, which is used to generate the security keys in the target ABS. | It shall be included during re-entry, secure Location Update or HO |
| ………….. | ………….. | ………….. |

============================== End of Proposed Text #5===============

Modify the text in the figure396 (page 105, line 44) as the following proposed text.

======================== Start of Proposed Text #6=====================

Dot16KDF (PMK, MSID*|BSID| AKCMAC _KEY _COUNT |”AK”, 160)

============================== End of Proposed Text #6===============

Modify the sentence (page 106, line 47) as the following proposed text.

======================== Start of Proposed Text #7=====================

If CMAC_PN or AKCMAC _KEY _COUNT reach their maximum value, the associated AK as well as PMK becomes permanently deactivated. The ABS and AMS shall maintain the AK context as long as they retain the AK.

============================== End of Proposed Text #7===============

Modify the table 717 (page 118, line 32) as the following proposed text.

======================== Start of Proposed Text #8=====================

Table 717—The PMK context

| Parameter | Size(bits) | Usage |
| --- | --- | --- |
| ………….. | ………….. | ………….. |
| AKCMAC _KEY _COUNT | 16 | Value of the Entry Counter that is used to guarantee freshness of computed CMAC_KEY_* with every entry and provide replay protection. This counter is maintained across zone switch (LZONE and MZONE) to ensure replay protection when switching to LZONE multiple times within the validly period of MSK |

============================== End of Proposed Text #8===============

Modify the sentence (page 131, line 22) as the following proposed text.

======================== Start of Proposed Text #9=====================

The AAI_RNG-REQ message shall include STID, AKCMAC _KEY _COUNT and a CMAC tuple, but not include AMS MAC address or previous serving ABSID.

============================== End of Proposed Text #9===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks : Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems-Advanced Air Interface”