| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Usage of CSGID in AAI_SCN-REQ and AAI_SCN-REP (15.2.3.12 and 15.2.3.14) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Anshuman Nigam, Jung Je Son, Ying Li and Mihyun Lee Samsung Electronics | E-mail: jungje.son@samsung.com anshun@samsung.com ying.l@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0044: IEEE 802.16 Working Group Working Group Letter Ballot #30 Section 15.2.3.12 and 15.2.3.14 AAI_SCN-REQ and AAI_SCN-REP |  |
| Abstract | This contribution provides format of the AAI_SCN-REQ and AAI_SCN-REP message |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Usage of CSGID in AAI_SCN-REQ and AAI_SCN-REP

Anshuman Nigam, Jung Je Son, Ying Li and Mihyun Lee 
Samsung Electronics

# Introduction

When the AMS has to request for information of neighboring Femto ABSs belonging to a CSG then the AMS can provide the desired CSGID(s) in the AAI_SCB-REQ message to the serving ABS. The ABS can respond with a list of ABS, addressed by BSID belonging to the requested CSGID(s) in AAI_SCN-RSP message.

The AAI_SCN-REQ message needs to be enhanced to support this.

When the AMS has to report information of multiple neighboring Femto ABSs belonging to the same CSG then the overhead in AAI_SCN-REP can be reduced by indicating the CSGID once and then listing the ABSs addressed by reduced BSID which is formed by removing the CSGID part from the BSID.

The AAI_SCN-REP message needs to be enhanced to support this.

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2.3.12 AAI_SCN-REQ

Add the following in the Table 681

| Name | Value | Usage |
| --- | --- | --- |
| N_CSGID_Indication | 1 bit | Indicates if N_CSGID is included in the message |
| N_CSGID | 4 bits | Number of CSGIDs for which the information is requested |
| CSGID | 36 bits | CSGID list |

15.2.3.14 AAI_SCN-REP

Add the following in the Table 683

| Name | Value | Usage |
| --- | --- | --- |
| N_CSGID_Indication | 1 bit | Indicates if N_CSGID is included in the message |
| N_CSGID | 4 bits | Number of CSGIDs for which the information is reported |
| CSGID | 36 bits | CSGID list |
| Num_BS | 12 | Number of BSs in each CSGID |
| Reduced_BSID | 12 | Reduced BSID for each BS where Reduced BSID is formed by removing the CSGID part from BSID. |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------