| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Zone switching using location update in idle mode (15.2.17) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Naehyun Lim, Hyunjeong Kang, Youngkyo Baek Samsung Electronics | hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16m-09/0057, “IEEE 802.16 Working Group Letter Ballot #30a: Announcement” |  |
| Abstract | The contribution proposes a zone switching operation in idle mode in 16m/D3. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Zone switching using location update in idle mode

Naehyun Lim, Hyunjeong Kang, Youngkyo Baek

Samsung Electronics

- Introduction

When an AMS operates in legacy idle mode, it may perform location update to LZone of a mixed mode ABS based on the location update trigger condition as in 6.2.23.8.1[1]. During the location update in LZone, the mixed mode ABS may instruct the AMS to switch over Mzone using a location update procedure of Mzone. The ABS provides the parameters NONCE_BS and Deregistration Identifier that are included in AAI_RNG-REQ for the location update in Mzone through a RNG-RSP message. Here, the NONCE_BS is used to create a security context (e.g., PMK, AK) in Mzone.

When an AMS receives the instruction to switch over Mzone by a mixed mode ABS through the RNG-RSP message, then the AMS may perform a location update in Mzone. The AMS shall include the parameters NONCE_BS, NONCE_MS and Deregistration Identifier in the AAI_RNG-REQ message when it performs the location update in Mzone of the ABS. The location update procedure in Mzone follows the operation as defined in 15.2.17.4.2 [2]. To secure AMS’s Deregistration Identifier, the ABS reassigns a Deregistration Identifier through the AAI_RNG-RSP after the location update in Mzone is completed.

The zone switch procedure during location update is presented in Figure 1.

Figure 1 Zone switching procedure from LZone to MZone of ABS through location update in idle mode

- Text proposal for inclusion in the 802.16m draft

[Note to Editor: Please add the following text after section 11.1.3 on page 15]

[Modify the Table 585 in section 11.6 as indicated]

Table 585 – RNG-RSP message encodings

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| Location Update Response | 23 | 1 | 0x00= Success of Location Update 0x01= Failure of Location Update 0x02 = Reserved Zone Switch 0x03=Success of location update and DL traffic pending 0x04~0xFF: Reserved | All |
| Zone Switch | 41 | Variable | Compound TLV to specify zone switch operation | OFDMA |

The Zone Switch Value field is composed of a number of encapsulated TLV fields as specified in Table XXX.

Table XXX – Zone Switch

| Name | Type (1byte) | Length | Value |
| --- | --- | --- | --- |
| NONCE_BS | 1 | 4 | A freshly generated random number of 32bits |
| Deregistration Identifier | 2 | 2 | The 10bit-ID which the AMS is assigned for idle mode of MZone; the AMS shall take 10MSB of this value as Deregistration Identifier and disregard 6LSB of this value. |

[Add the following parameters in Table 674 on page 37]

| NONCE_BS | A random number generated by ABS of 32bits. | It shall be included when the AMS is attempting to perform location update using the zone switching from LZone to MZone. |
| --- | --- | --- |
| NONCE_MS | A random number generated by AMS of 32 bits. |  |

[Modify the texts from line 64 of page 229 to line 7 of page 230 in section 15.2.17]

15.2.17 Idle Mode

An ABS may be a member of one or more paging groups that may have different cycle and offset. When AMS operating in legacy mode select the mixed ABS as a preferred ABS, AMS may stay in the Lzone and perform the legacy Idle Mode operation or switch over the Mzone directed by the mixed ABS through location update procedure. When the mixed ABS instructs the AMS to switch to Mzone of the ABS through the location update, the AMS may perform the procedure as defined in 15.2.17.7. Otherwise, If an AMS in Idle Mode decides to change its operation mode, the AMS shall perform full network re-entry from Idle Mode in the new operation mode. The change includes the AMS moves between BSs operating in different mode or switches from LZone to MZone of a mixed mode ABS. And the decision may be based on the detection of a new operation mode. When an AMS is paged in the Lzone of a mixed mode ABS, the AMS shall perform the network re-entry in the LZone of the ABS and may switch to the MZone of the ABS using Lzone to Mzone handoff procedures as defined in 15.2.6.4.1.2.1.

[Add the new section right after section 15.2.17.6.1 on page 241]

15.2.17.7 Idle Mode Support for Zone switching

An AMS executing the idle mode entry at LZone performs the location update with the LZone of a mixed mode ABS if any of the location update trigger condition of section 6.3.23.8.1 is met. During the location update, the mixed mode ABS recommends the AMS to switch over the MZone from LZone by sending the RNG-RSP message including Location Update Response set to 0x02 (Zone Switch) and the Zone Switch TLV.

If the AMS decides to switch over the MZone, the AMS performs the location update with the MZone of the mixed mode ABS with an AAI_RNG-REQ message containing the NONCE_MS and NONCE_BS which are required for AK derivation from the network.

During the location update, the AMS updates the Deregistration Identifier, based on information received from AAI_RNG-RSP.

Figure 451- Zone switching procedure from LZone to MZone of ABS in idle mode:

The ABS instructs the AMS to switch zone through the location update

Reference

[1] 802.16-2009, Part 16: Air interface for broadband wireless access systems, May 2009

[2] P802.16m/D2, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, October 2009