| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SMS support in idle state (15.2.24) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Hyunjeong Kang, Jungshin Park, Simon Kong, Sajin Kim, Jonghyun Won Samsung Electronics | hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16m-09/0057, “IEEE 802.16 Working Group Letter Ballot #30a: Announcement” |  |
| Abstract | The contribution proposes a clarification on SMS support in 16m/D2. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SMS support in idle state (15.2.24)

Hyunjeong Kang, Jungshin Park, Simon Kong, Sajin Kim, Jonghyun Won

Samsung Electronics

- Introduction

Current IEEE802.16m draft [1] has a mechanism to support SMS in connected state and idle state as follows:

In connected state, AAI_L2-XFER (L2 transfer) message with Transfer-Type=6 is defined to send/receive SMS. In idle state, AAI_RNG-RSP message is used to transmit SMS in DL.

Here, we propose a mechanism to transmit SMS in UL during idle state as well as a confirmation on the received SMS.

A SMS from an AMS in idle state may be included in the AAI_RNG-REQ message with Ranging Purpose Indication Bit#1=1(“idle mode location update”). If UL resource is not enough to send an AAI_RNG-REQ including the SMS, then the AAI_RNG-REQ may be fragment using RNG-REQ fragmentation method [2]. When an ABS receives a SMS from an AMS in idle state through the AAI_RNG-REQ message, an AAI_RNG-RSP message sent by the ABS is used as a confirmation of the SMS. The figure 1 presents a scenario to transmit/receive UL SMS in idle state.

When an ABS has a DL SMS for an idle mode AMS, the ABS includes the SMS in an AAI_RNG-RSP message after sending an AAI_PAG-ADV indicating location update. After sending the AAI_RNG-RSP with SMS, the ABS shall grant a CDMA Allocation A-MAP IE in unsolicited manner to receive AAI_MSG-ACK as a confirmation on the DL SMS. The MCRC of CDMA Allocation A-MAP IE is masked with a RAID and masking indicator for Ranging, where the RAID is the same as the RAID masked in CDMA Allocation A-MAP IE for the AAI_RNG-REQ message sent by the AMS. After the ABS transmits the AAI_RNG-RSP with SMS, the ABS starts a timer SMS_confirm_resource_alloc_timer and grants the CDMA Allocation A-MAP IE to the AMS. When the AMS receives the AAI_RNG-RSP with SMS, the AMS starts a timer SMS_confirm_resource_waiting_timer and waits for the CDMA Allocation A-MAP IE to send the AAI_MSG-ACK. The figure 2 shows a scenario to send/receive DL SMS in idle state.

Figure 1 UL SMS in idle state

Figure 2 DL SMS in idle state

As a confirmation on the received SMS in connected state, we propose to use Transfer-Type=7 of AAI_L2-XFER as a confirmation of a received SMS in connected state. In this method, when an ABS or an AMS receives an AAI_L2-XFER with Transfer-Type=6, it sends an AAI_L2-XFER with Transfer-Type=7.

Reference

[1] P802.16m/D2, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, October 2009

[2] IEEE C80216m-09_2459, Clarification on fragmentation of AAI_RNG-REQ message

- Suggested remedy

-------------------------------  Text Start  ---------------------------------------------------

[Modify line 51 to line 54 on page 260 in section 15.2.24 Short Message Service:]

In connected mode, AAI_L2-xfer is used to send/receive SMS and SMS confirmation. In idle mode, In case of AAI_RNG-RSP message, Short Message Service information may be included in an AAI_RNG-REQ/RSP message. The maximum size of the short message body is 140 bytes. This parameter may be included only when the action code of AAI_PAG-ADV indicates location update or when the AAI_RNG-REQ with Ranging Purpose Indication Bit#1 is set to 1.

When UL SMS is included in an AAI_RNG-REQ message with Ranging Purpose Indication Bit#1=1, an AAI_RNG-RSP is transmitted as a confirmation of the SMS.

When DL SMS is included in an AAI_RNG-RSP, an AAI_MSG-ACK is transmitted as a confirmation of the SMS. A CDMA Allocation A-MAP IE for the AAI_MSG-ACK is granted in unsolicited manner, where MCRC is masked with RAID and masking indicator for Ranging. The RAID shall be the same as a RAID masked in CDMA Allocation A-MAP IE for the AAI_RNG-REQ message. For the CDMA Allocation A-MAP IE of the AAI_MSG-ACK, a ACK timer is applied at the AMS and at the ABS. After sending the AAI_RNG-RSP message, the ABS shall grant CDMA Allocation A-MAP IE by the ACK timer to receive the AAI_MSG-ACK. When the AMS receives the AAI_RNG-RSP, the AMS starts the ACK timer and waits the CDMA Allocation A-MAP IE.

[Replace line 54 to line 55 on page 76 in section 15.2.3.27 L2 Transfer message (AAI_L2_XFER:

]

f) Transfer-Type = 6: SMS

a) Sub-Type = SMS data

b) Sub-Type = SMS confirmation

-------------------------------  Text End  ---------------------------------------------------