| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for the single-BS MIMO feedback MAC message (15.2.3.33) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | This contribution proposes modifications for the single-BS MIMO feedback MAC message AAI_SingleBS_MIMO_FBK |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed modifications for the single-BS MIMO feedback MAC message (15.2.3.33)

David Mazzarese et Al.

Samsung Electronics Co., Ltd.

# Introduction

Corrections are proposed for the MAC control message that carries single-BS MIMO feedback information:

- For the feedback of the transmit correlation matrix, < is replaced by <=

- References to Tables 713 and 714 are replaced to a reference to section 15.3.7.2.5.6

- The size of the bitmap for indicating the best subbands is changed from Nsub to KSB

- TBD for parameter CM is resolved

# Text proposal for inclusion in the 802.16m amendment

<< Modify section “15.2.3.33 AAI_SingleBS_MIMO_FBK” on page 79 line 1 >>

-------------------------------  Start of Text Proposal ---------------------------------------------------

15.2.3.33 AAI_SingleBS_MIMO_FBK

The AAI_SingleBS_MIMO_FBK message format is defined in Table 700. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 2, 3, 5 or 6. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 8 transmit antennas.

Table 700 - AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type = xx |  |  |
| If (Transmit_Correlation_Matrix == 1){ |  | Only for 8Tx optional |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 15.3.7.2.5.6 <<<Table 713>>> |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 15.3.7.2.5.6 <<<Table 714>>> |
| } |  |  |
| } |  |  |
| }else{ |  |  |
| MFM_bitmap | 4 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested in a by Feedback Polling IE. Only MFM 2, 3, 5 and 6 are allowed. LSB #0: MFM 2 LSB #1: MFM 3 LSB #2: MFM 5 LSB #3: MFM 6 |
| If (LSB #0 MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | KSB Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 2, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband indicated in Best_subbands_bitmap |
| } |  |  |
| } |  |  |
| If (LSB #1 MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | KSB Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 3, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband indicated in Best_subbands_bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband indicated in Best_subbands_bitmap, according to CM and CS |
| } |  |  |
| } |  |  |
| If (LSB #2 MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | KSB Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband indicated in Best_subbands_bitmap |
| Stream index | 1 to 3 | Best stream index of m-th subband indicated in Best_subbands_bitmap, according to CS and MaxMt |
| } |  |  |
| } |  |  |
| If (LSB #3 MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | KSB Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband indicated in Best_subbands_bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband indicated in Best_subbands_bitmap, according to CM and CS |
| } |  |  |
| } |  |  |
| If ((LSB #1 MFM_bitmap == 1) or (LSB #3 MFM_bitmap == 1)){ |  | MFM 3 or MFM 6 |
| If (CM == 0b1){ [TBD] |  |  |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 15.3.7.2.5.6 <<<Table 713>>> |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 15.3.7.2.5.6 <<<Table 714>>> |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal ---------------------------------------------------