| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for the single-BS MIMO feedback MAC message (15.2.3.33) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics Wookbong Lee, Bin-Chul Ihm LG Electronics | d.mazzarese@samsung.com wbong@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | This contribution proposes modifications for the single-BS MIMO feedback MAC message AAI_SingleBS_MIMO_FBK |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed modifications for the single-BS MIMO feedback MAC message (15.2.3.33)

David Mazzarese et Al.

Samsung Electronics Co., Ltd.

Wookbong Lee, Bin-Chul Ihm

LG Electronics

# Introduction

Corrections are proposed for the MAC control message that carries single-BS MIMO feedback information:

- For the feedback of the transmit correlation matrix, < is replaced by <=

- References to Tables 713 and 714 are replaced to a reference to section 15.3.7.2.5.6

- The bitmap for indicating the best subbands is changed to the Best_subbands_index

- The condition for the feedback of the transmit correlation matrix is TBD, according to the TBD in the Feedback Polling A-MAP IE

# Text proposal for inclusion in the 802.16m amendment

<< Modify section “15.2.3.33 AAI_SingleBS_MIMO_FBK” on page 79 line 1 >>

-------------------------------  Start of Text Proposal ---------------------------------------------------

15.2.3.33 AAI_SingleBS_MIMO_FBK

The AAI_SingleBS_MIMO_FBK message format is defined in Table 700. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 2, 3, 5 or 6. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 8 transmit antennas. Variables MaxMt, CS, CM, Num_best_subbands and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. Best_subbands_index is encoded as described in section 15.3.9.3.1.4.

Table 700 - AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type = xx |  |  |
| If (Transmit_Correlation_Matrix == 1 TBD){ |  | Only for 8Tx Optional |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 15.3.7.2.5.6 <<<Table 713>>> |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 15.3.7.2.5.6 <<<Table 714>>> |
| } |  |  |
| } |  |  |
| }else{ |  |  |
| MFM_bitmap | 4 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested in a by Feedback Polling IE. Only MFM 2, 3, 5 and 6 are allowed. LSB #0: MFM 2 LSB #1: MFM 3 LSB #2: MFM 5 LSB #3: MFM 6 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap Best_subbands_index | variable Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 variable | STC_Rate for MFM 2, according to MaxMt If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index in bitmap |
| } |  |  |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap Best_subbands_index | variable Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 variable | STC_Rate for MFM 3, according to MaxMt MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index in bitmap |
| Subband PMI | 3 to 6 variable | PMI of m-th subband indicated by Best_subbands_index in bitmap, according to CM and CS Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #2 in MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap Best_subbands_index | variable Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index in bitmap |
| Stream index | 1 to 3 variable | Best stream index of m-th subband indicated by Best_subbands_index in bitmap, according to CS and MaxMt If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| } |  |  |
| If (LSB #3 in MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap Best_subbands_index | variable Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index in bitmap |
| Subband PMI | 3 to 6 variable | PMI of m-th subband indicated by Best_subbands_index in bitmap, according to CM and CS Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If ((LSB #1 MFM_bitmap == 1) or (LSB #3 MFM_bitmap == 1)){ |  | MFM 3 or MFM 6 |
| If (CM == 0b1){ [TBD] |  |  |
| For (i=1; i Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in <<<Table 713>>> |
| For (j=i+1; j Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in <<<Table 714>>> |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal ---------------------------------------------------