| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on E-MBS protocol features and functions (P80216m/D2 15.9.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Kaiying Lv, Bo Sun, Nan Li， Mary Chion | Voice: [Telephone Number (optional)]] E-mail: lv.kaiying@zte.com.cn |
| Re: | Category: LB30a / Area: Chapter 15.9.3.( E-MBS protocol features and functions) |  |
| Abstract | This contribution proposes text on E-MBS FDM multiplexing and E-MBS configuration message |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on E-MBS protocols and features

Kaiying Lv , Bo Sun, Nan Li, Mary Chion

ZTE Corporation

- Introduction

In order to allow supporting FDM multiplexing of E-MBS and unicast traffic in the downlink frame, this contribution proposed a resource allocation scheme and corresponding indication method. As E-MBS configuration message has not been defined completely in P802.16m/D2, we also proposed a text on E-MBS configuration MAC message.

- Proposed Resource allocation and indication schemes

It is desirable to make flexible and high efficient resource allocation for E-MBS traffic with simple signaling and no impact on subchannelization when FDM multiplexing with unicast in the downlink frame. In a downlink frame, there are two types of resource units in each frequency partitions, which are contiguous resource unit (CRU) and distributed resource unit (DRU). In order to make no impact on subchannelization, CRUs can be considered to be allocated to E-MBS traffic. Since FFR partitioning has no impact on E-MBS traffic, it will be more flexible to allow resource allocation in any frequency partition with subband based CRUs or both subband and miniband based CRUs for E-MBS traffic. Then the CRUs  allocated to E-MBS traffic can be further mapped into distributed logical resource units (DLRUs) by subcarrier permutation in order to acquire frequency diversity gain,.

E-MBS resource allocation indication should be carried in E-MBS configuration information, which can include the number of PRUs or subbands and the allocation ratio among frequency partitions. E-MBS users decode the E-MBS configuration information to get resource allocation information and know how to map the logical resource unit into physical subcarriers.

Table 1 illustrates E-MBS resource allocation indication as an example.

| E-MBS resource allocation indication | LRU_MBS: number of PRUs reserved for E-MBS RATIO： resource partitioning among frequency partitions |
| --- | --- |
| 0000 | LRU_MBS: 4 PRUs reserved for E-MBS RATIO: 1:1:1:1, one PRU in each frequency partition (FP0:FP1:FP2:FP3) |
| 0001 | LRU_MBS: 8 PRUs reserved for E-MBS RATIO: 1:1:1:1, two PRUs in each frequency partition (FP0:FP1:FP2:FP3) |
| 0010 | LRU_MBS: 16 PRUs reserved for E-MBS RATIO: 1:1:1:1, four PRUs in each frequency partition (FP0:FP1:FP2:FP3) |
| 0011 | LRU_MBS: 32 PRUs reserved for E-MBS RATIO: 1:1:1:1, 8 PRUs in each frequency partition (FP0:FP1:FP2:FP3) |
| 0100 | LRU_MBS: 4 PRUs reserved for E-MBS RATIO: 1:0:0:0, 4 PRUs only in FP0 |
| 0101 | LRU_MBS: 8 PRUs reserved for E-MBS RATIO: 1:0:0:0, 8 PRUs only in FP0 |
| 0110 | LRU_MBS: 16 PRUs reserved for E-MBS RATIO: 1:0:0:0, 16 PRUs only in FP0 |
| 0111 | LRU_MBS: 12 PRUs reserved for E-MBS RATIO: 0:1:1:1, 3 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1000 | LRU_MBS: 24 PRUs reserved for E-MBS RATIO: 0:1:1:1, 8 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1001 | LRU_MBS: 12 PRUs reserved for E-MBS RATIO: 3:1:1:1, 6 PRUs in FP0 and 2 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1010 | LRU_MBS: 18 PRUs reserved for E-MBS RATIO: 3:1:1:1, 9 PRUs in FP0 and 3 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1011 | LRU_MBS: 24 PRUs reserved for E-MBS RATIO: 3:1:1:1, 12 PRUs in FP0 and 4 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1100 | LRU_MBS: 8 PRUs reserved for E-MBS RATIO: 5:1:1:1, 5 PRUs in FP0 and 1 PRU in each frequency partition of FP1,FP2,FP3 |
| 1101 | LRU_MBS: 16 PRUs reserved for E-MBS RATIO: 5:1:1:1, 10 PRUs in FP0 and 2 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1110 | LRU_MBS: 32 PRUs reserved for E-MBS RATIO: 5:1:1:1, 20 PRUs in FP0 and 4 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1111 | Reserved |

- Text proposal for the 802.16m D2 draft

1) Proposes the addition of a new Section 15.9.3.4, “FDM multiplexing of E-MBS and unicast”.

=====================  Start of Proposed Text section 15.9.3.4 ==========================================

FDM multiplexing can be applied for E-MBS and unicast traffic. Subband based CRUs or both subband and miniband based CRUs in all frequency partitions are allowed to be allocated to E-MBS traffic. E-MBS resource is assigned from the lowest CRU index of each allocated frequency partition.

The CRUs allocated to E-MBS traffic can be further permutated to map into DRUs and then mapped into distributed logical resource units (DLRUs) by subcarrier permutation across the entire resources allocated to E-MBS. Within an E-MBS Zone supporting macro-diversity, zone specific PRU and subcarrier permutation are applied.

E-MBS resource allocation indicator shall be carried in AAI_E-MBS_CFG message, which can indicate the number of CRUs or subbands allocated to E-MBS traffic and the allocation ratio among frequency partitions. An E-MBS AMS can acquire the physical structure of E-MBS resource region on the FDM multiplexing subframe by decoding E-MBS resource allocation indicator carried in AAI_E-MBS_CFG message.

======================   end of Proposed Text Section 15.9.3.4 ==========================================

- Proposes the modification of Section 15.9.3.1, “E-MBS Configuration Indicators”.

============================  Start of Proposed Text Section 15.3.9.1================================

The information regarding E-MBS configuration is transmitted to the AMSs using E-MBS configuration indicators. While most E-MBS configuration indicators necessary for E-MBS operation are of interest only to the AMSs interested in E-MBS, a few E-MBS related configuration information may be broadcasted to all AMSs.

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isizeoffset, E-MBS MAP MIMO Mode and MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content, resource allocation indicator for FDM multiplexing of E-MBS and unicast, E-MBS configuration information transmission interval. Some of tThese E-MBS configuration informations are transmitted in a new MAC management message called the AAI-E-MBS_CFG message to be read only by the E-MBS AMSs.

An AAI_E-MBS_CFG message is periodically sent by ABS to inform the E-MBS AMSs of E-MBS configuration information and necessary information for AMS to locate and decode E-MBS zone specific E-MBS MAP message in E-MBS region. The transmission interval in included in AAI_E-MBS_CFG message for AMS to locate the next AAI_E-MBS_CFG message with the same frame/subframe index.

============================end of Proposed Text Section 15.9.3.1 =================================

(2) Proposes the addition of a new Section 15.2.3.41." AAI_E-MBS_CFG”

=====================Start of Proposed Text section 15.2.3.41 ==========================================

An AAI_E-MBS_CFG message is periodically sent by ABS to inform the E-MBS AMSs of E-MBS configuration information necessary for E-MBS operation. ABS shall generate AAI_ E-MBS_CFG message, including the following parameters:

- E-MBS-Zone_IDs of serving ABSs

- E-MBS MAP time offset (superframe/frame/subframe index)

- E-MBS MAP resource allocation (number of LRUs)

- E-MBS MAP Isizeoffset

- E-MBS MAP MIMO Mode

- E-MBS-Zone_IDs of neighboring ABSs

- E-MBS MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content

- AAI_E-MBS_CFG message transmission interval (superframe index/offset)

- E-MBS resource allocation indicator for FDM

An AAI_E-MBS_CFG message transmitted on a carrier shall include the E-MBS-Zone_IDs of E-MBS Zones supported by the serving ABS on the corresponding carrier.

AAI_E-MBS_CFG message transmission interval is included in an AAI_E-MBS_CFG message for AMS to locate and decode the next AAI_E-MBS_CFG message with the same frame/subframe index.

An AAI_E-MBS_CFG message shall also include E-MBS resource allocation indicator to indicate the number of CRUs allocated to E-MBS service and resource allocation ratio among frequency partitions on a subframe where FDM multiplexing with unicast is applied. The lookup table for FDM multiplexing resource indicator is as follows:

Table XXX –E-MBS resource indicator lookup table

| E-MBS resource allocation indication | LRU_MBS: number of PRUs reserved for E-MBS RATIO： resource partitioning among frequency partitions |
| --- | --- |
| 0000 | LRU_MBS: 4 PRUs reserved for E-MBS RATIO: 1:1:1:1, one PRU in each frequency partition (FP0:FP1:FP2:FP3) |
| 0001 | LRU_MBS: 8 PRUs reserved for E-MBS RATIO: 1:1:1:1, two PRUs in each frequency partition (FP0:FP1:FP2:FP3) |
| 0010 | LRU_MBS: 16 PRUs reserved for E-MBS RATIO: 1:1:1:1, four PRUs in each frequency partition (FP0:FP1:FP2:FP3) |
| 0011 | LRU_MBS: 32 PRUs reserved for E-MBS RATIO: 1:1:1:1, 8 PRUs in each frequency partition (FP0:FP1:FP2:FP3) |
| 0100 | LRU_MBS: 4 PRUs reserved for E-MBS RATIO: 1:0:0:0, 4 PRUs only in FP0 |
| 0101 | LRU_MBS: 8 PRUs reserved for E-MBS RATIO: 1:0:0:0, 8 PRUs only in FP0 |
| 0110 | LRU_MBS: 16 PRUs reserved for E-MBS RATIO: 1:0:0:0, 16 PRUs only in FP0 |
| 0111 | LRU_MBS: 12 PRUs reserved for E-MBS RATIO: 0:1:1:1, 3 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1000 | LRU_MBS: 24 PRUs reserved for E-MBS RATIO: 0:1:1:1, 8 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1001 | LRU_MBS: 12 PRUs reserved for E-MBS RATIO: 3:1:1:1, 6 PRUs in FP0 and 2 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1010 | LRU_MBS: 18 PRUs reserved for E-MBS RATIO: 3:1:1:1, 9 PRUs in FP0 and 3 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1011 | LRU_MBS: 24 PRUs reserved for E-MBS RATIO: 3:1:1:1, 12 PRUs in FP0 and 4 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1100 | LRU_MBS: 8 PRUs reserved for E-MBS RATIO: 5:1:1:1, 5 PRUs in FP0 and 1 PRU in each frequency partition of FP1,FP2,FP3 |
| 1101 | LRU_MBS: 16 PRUs reserved for E-MBS RATIO: 5:1:1:1, 10 PRUs in FP0 and 2 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1110 | LRU_MBS: 32 PRUs reserved for E-MBS RATIO: 5:1:1:1, 20 PRUs in FP0 and 4 PRUs in each frequency partition of FP1,FP2,FP3 |
| 1111 | Reserved |