| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to the section on OL region (15.3.7.2.5.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | This contribution proposes some corrections to the section on open-loop region |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed modifications to the section on OL region (15.3.7.2.5.1)

David Mazzarese et Al.

Samsung Electronics

# Introduction

The section on OL region has some typos. MIMO mode 2 cannot be allowed, since it is a closed-loop MIMO mode. The open-loop region configuration parameters are sent in the AAI_System Configuration Descriptor (SCD) message, so the TBD can be resolved. Some clarification on the usage of non-adaptive precoding and pilots in the open-loop region are also needed.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D2, Oct 2009.

# Text proposal for inclusion in the 802.16m amendment

< Modify section 15.3.7.2.5.1 Open-Loop Region on line 30 page 411 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2.5.1 Open-Loop Region

An open-loop region with MaxMt MIMO streams is defined as a time-frequency resource using the MaxMt MIMO streams pilot pattern and a given open-loop MIMO mode with Mt = MaxMt without rank adaptation. The open-loop region allows base stations to coordinate their open-loop MIMO transmissions, in order to offer a stable interference environment where the precoders and numbers of MIMO streams are not time-varying. The resource units used for the open-loop region are indicated in the AAI_System Configuration Descriptor (SCD) message a downlink broadcast message [SFH or ABI is TBD]. These resource units shall be aligned across cells.

Only a limited set of open-loop MIMO modes are allowed for transmission in the open-loop region. There is no limitation to the use of any open-loop MIMO mode outside the open-loop region, except for CDR, as specified in Table 827.

An open-loop region is associated with a specific set of parameters:

- Type (number of MIMO streams MaxMt, MIMO mode, MIMO feedback mode, type of permutation)

- Resource unit [TBD]

There are three types of open-loop regions, as specified in Table 829.

Table 829 – Types of open-loop regions

|  | MaxMt | MIMO mode | MIMO feedback mode | Supported permutation |
| --- | --- | --- | --- | --- |
| OL region Type 0 | 2 streams | MIMO Mode 0 MIMO Mode 1 (Mt = 2 streams) | 0 | DRU DLRU |
| OL regioin region Type 1 | 1 stream | MIMO Mode 5 (Mt = 1 streams) | 1 | Miniband based CRU (diversity allocation) NLRU |
|  |  |  | 2 | Subband based CRU (localized allocation) SLRU |
| OL region Type 2 | 2 streams | MIMO Mode 2 1 (Mt = 2 streams) MIMO Mode 3 (Mt = 2 streams) | 5 | Subband based CRU (localized allocation) SLRU |

MIMO mode 1 with Mt = 2 streams can be allocated for downlink transmission to a single AMS in the OL region type 2. Dynamic switching between MIMO mode 1 and MIMO mode 3 in downlink transmissions in OL region type 2 is allowed based on the feedback with MIMO Feedback Mode 5. The precoders for transmission with MIMO mode 1 or MIMO mode 3 in OL region type 2 are the same on a given subband.

The OL region type 0 is present if OL-Region-ON is indicated in a downlink broadcast message [SFH or ABI is TBD].

The presence of OL regions types 0, 1 and 2 is indicated in the AAI_System Configuration Descriptor (SCD) message. The OL region type 0 is present whenever at least one type of OL region is present. OL region type 0 occupies all DLRUs in Frequency Partition 0. The resources occupied by OL region type 1 and OL region type 2 are indicated in the AAI_System Configuration Descriptor (SCD) message.

All base stations that are coordinated over the same open loop region should use the same number of MIMO streams, in order to guarantee low interference fluctuation and thus improve the CQI prediction at the AMS. All pilots are precoded by non-adaptive precoding with MaxMt MIMO streams in the open-loop region. CQI measurements should be taken by the AMS on the precoded demodulation pilots rather than on the downlink reference signals midamble.

The MaxMt precoded pilots streams shall be transmitted in all the LRUs in the OL region even if data is not being transmitted by the ABS on some or all of the LRUs.

-------------------------------  Text End  ---------------------------------------------------