| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to the MIMO feedback extended header (15.2.2.2.8) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics Wookbong Lee, Bin-Chul Ihm LG Electronics | d.mazzarese@samsung.com wbong@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | This contribution proposes modifications to the MIMO feedback extended header |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed modifications to the MIMO feedback extended header (15.2.2.2.8)

David Mazzarese, Hokyu Choi
Samsung Electronics Co., Ltd.

Wookbong Lee, Bin-Chul Ihm
LG Electronics

# Introduction

This contribution proposes modifications to the MIMO feedback extended header section 15.2.2.2.8. The variable bit size of some of the fields is clarified. KSB should be used instead of bandwidth.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D2, Oct 2009.

# Text proposal for inclusion in the 802.16m amendment

<< Modify section “15.2.2.2.8 Correlation matrix feedback extended header (MFEH)” on page 27 line 33 >>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

15.2.2.2.8 MIMO feedback extended header (MFEH)

This MIMO feedback extended header format is defined in Table 666. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for one the best subband for MFM 2, 3, 5 or 6. Variables MaxMt, CS and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. The number of subbands YSB is described in section 15.3.9.3.1.4.

Table 666 – Correlation Matrix feedback extended header format MFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| LAST | 1 | Set to “1” if last EH, else, set to “0” |
| Type | TBD | MFEH type |
| MFM | 3 |  |
| If (MFM == 0 or 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 1 to 2 variable | Depending on MaxMt If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit This field shall be omitted if MaxMt =1. |
| } |  |  |
| If (MFM == 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt =1. |
| } |  |  |
| If (MFM == 4){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | variable | Nt =2: 3 bits Nt =4 and CS=0b0: 6 bits Nt =4 and CS=0b1: 4 bits Nt =8: 4 bits |
| } |  |  |
| If (MFM == 4 or 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 3 to 6 variable | Depending on Nt and CS Nt =2: 3 bits Nt =4 and CS=0b0: 6 bits Nt =4 and CS=0b1: 4 bits Nt =8: 4 bits |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 3 to 5 variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 variable | Depending on MaxMt If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt =1. |
| } |  |  |
| If (MFM == 3){ |  |  |
| Subband index | 3 to 5 variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 variable | Depending on MaxMt MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt =1. |
| Subband PMI | 3 to 6 variable | Depending on Nt and CS Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | 3 to 5 variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband stream index | 1 to 3 variable | Depending on MaxMt If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | 3 to 5 variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband PMI | 3 to 6 variable | Depending on Nt and CS Nt =2: 3 bits Nt =4 and CS=0b0: 6 bits Nt =4 and CS=0b1: 4 bits Nt =8: 4 bits |
| } |  |  |
| Reserved | variable | For byte alignment |
| } |  |  |

-------------------------------  End of Text Proposal  ---------------------------------------------------