| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Differentiated Bandwidth Requests (15.2.11.1.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Stavros Tzavidas, Motorola Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang, Intel Abhay Karandikar, B. Srinadh, Prateek Kapadia, Harshad Maral, Telecom Center of Excellence, Indian Institute of Technology (Bombay) Kiran Kuchi,J. Klutto Milleth, CeWiT, Mary Chion, ZTE Ming-Hung Tao, ITRI Yih-Shen Chen, MediaTek Jaehyuk Jang, Samsung Inuk Jung, LGE | stavros.tzavidas@motorola.com |
| Re: | LB30a |  |
| Abstract | Some details of the prioritization mechanism for bandwidth requests are not described in the current text. This contribution proposes text to complete the description. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Differentiated Bandwidth Requests (15.2.11.1.1)

Stavros Tzavidas

Motorola

Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang

Intel

Abhay Karandikar, B. Srinadh, Prateek Kapadia, Harshad Maral

Telecom Center of Excellence, Indian Institute of Technology (Bombay)

Kiran Kuchi,J. Klutto Milleth

CeWiT

Mary Chion

ZTE

Ming-Hung Tao

ITRI

Yih-Shen Chen

MediaTek

Jaehyuk Jang

Samsung

Inuk Jung

LGE

# Introduction

The current text describes a mechanism for differentiating bandwidth requests based on user class.

This contribution completes some parts missing in the text. More specifically:

- We specify which DL control message should be used by the ABS to communicate the minimum access class that is allowed

- We specify a default value (“everyone allowed”) so that this parameter does not need to be sent in every frame

- We specify how priority access class is assigned to a service flow (via DSx transactions). This contribution explains why the remedy proposed in the comment will have exactly the opposite effect than what the comment intended and proposes alternative solutions.

# Text Proposal

Modify the text in paragraph 15.2.11.1.1 “Contention-based random access bandwidth request” as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

15.2.11.1.1 Contention-based random access bandwidth request

The ABS may advertise a minimum access class in the BR channel configuration within a DL Control message the AAI-SCD. If no minimum access class is advertised in the AAI-SCD that means that all access classes are allowed. An access class is assigned to a service flow via DSx MAC management messages during service flow establishment / modification.. When an AMS has information to send and decides to use the contention-based random access bandwidth request, the AMS shall check if the information the AMS has to send is for an access class with priority higher than or equal to the minimum access class advertised by BR channel configuration within a DL Control message. If it is not (the minimum access class is not sufficiently low such that the AMS access class is allowed), then the AMS shall wait until the BR channel configuration within a DL Control message the AAI-SCD advertises a minimum access class, which is less than or equal to the access class of the data and the AMS. When the AMS access class is allowed, the AMS shall set its internal backoff window equal to the Request Bandwidth request backoff start (or Ranging for initial ranging) and send a bandwidth request when the backoff timer expires.

Modify the description of AAI_SCD message in paragraph 15.2.3.28 as indicated below:

15.2.3.28 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change	INTEGER (0 ~ 15)

BS_Restart_Count			INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ON		INTEGER (0..1) OPTIONAL

OL-Rank1-Configuration	INTEGER (0..7) OPTIONAL

SB-OL-Region-2Size		INTEGER (0..15) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0			INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1			INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2			INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3			INTEGER (0..15) OPTIONAL

Alpha (a)				INTEGER (0.. 7) OPTIONAL

Beta (ß)				INTEGER (0..1) OPTIONAL

SINRmin				INTERGER (0..15) OPTIONAL

BW_REQ Access Class min		INTERGER (0..4) OPTIONAL

}

- ASN1STOP

Insert a new row in Table 739 as indicated below

Table 739—Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
|  | … |  |
| QoS Parameter Set Type | 3 | Refer to 11.13.4 |
| Traffic Priority | 3 | Refer to 11.13.5 |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 12.2.11.1.1 |
| Maximum Sustained Traffic Rate | 6 | Refer to 11.13.6 |

-------------------------------------------------- End of the text proposal --------------------------------------------------