| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to the OL MIMO region indicator parameters in the AAI System Configuration Descriptor (AAI_SCD) message, and associated interpretation rules – Proposed Amendment Text |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Sudhir Ramakrishna, David Mazzarese, Hyunjeong Kang Samsung Electronics | sudhir.r@samsung.com |
| Re: | “Contribution in support of a comment on LB30”: Category – AWD Comments Area – Chapter 15.2.3 (MAC Control Messages) |  |
| Abstract | This contribution proposes some modifications to the parameters indicating the OL MIMO region configuration in the AAI System Configuration Descriptor message in Section 15.2.3.28, and also proposes the interpretation rules for these parameters. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to the OL MIMO region parameters in the AAI System Configuration Description (AAI_SCD) message, and associated interpretation rules– Proposed Amendment Text

Sudhir Ramakrishna, David Mazzarese, Hyunjeong Kang

Samsung Electronics

- Introduction

In [1], the AAI System Configuration Description (AAI_SCD) message, defined in Section 15.2.3.28, carries some parameters that indicate the configuration of the MIMO open-loop region. However, there is no description of how those parameters are to be interpreted.

- In this document, text to be deleted has been struck out and marked with red font. Text to be added is in blue font. Existing text to be retained is in black font.

- References

[1] IEEE P802.16m/D2, “Part 16: Air Interface for Broadband Wireless Access Systems”, October 2009.