| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial changes on S-SFH Update (15.2.23) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk and Kiseon Ryu, LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com, sixs@lge.com , ksryu@lge.com |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | This contribution proposes text for S-SFH Update procedure |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial changes on S-SFH update (15.2.23)

Heejeong Cho, Youngsoo Yuk and Kiseon Ryu

LG Electronics

- Introduction

This contribution includes the proposed text for S-SFH Update in the IEEE 802.16m DRAFT Amendment [1].

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

On the IEEE P802.16m/D2, page 258, line 53, make the following changes to subclause 15.2.23

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.23 Update of S-SFH IEs

The S-SFH SP IEs (i.e., S-SFH SP1 IE, SP 2 IE and SP 3 IE) are transmitted at scheduling periods by the ABS, respectively. This SP scheduling periodicity information is transmitted in the S-SFH SP 3 IE. If AMS has been this information, it shall assume that this information is valid until this information is changed.

Every superframe, the ABS transmits P-SFH IE containing the S-SFH Scheduling information bitmap, SCD S-SFH change count and S-SFH SP change bitmap. The S-SFH Scheduling information bitmap indicates S-SFH SP IE(s) which is transmitted in a superframe. AMS can implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE. The SCD S-SFH change count shall remain unchanged as long as any of the values (except MSB of superframe number in S-SFH SP 1) of S-SFH SP IEs remain unchanged. The SCD S-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number) of S-SFH IEs change. The changed S-SFH SP IE(s) shall be transmitted at scheduling superframes.

Each bit of the S-SFH SP change bitmap indicates the changing status of the corresponding S-SFH SP IE. The bit # 0, bit #1 and bit #2 are mapped to S-SFH SP1 IE, S-SFH SP 2 IE and S-SFH SP3 IE, respectively. If any of the values (except MSBs of superframe number in S-SFH SP 1) of an S-SFH SP IE are changed, the corresponding bit is toggled.

According to the SCD S-SFH change count and S-SFH SP change bitmap, AMS previously decides whether it shall decode S-SFH IE or not.

AMS shall compare values of each SCD S-SFH change count in the last received P-SFH IE and the last stored P-SFH IE whenever it receives P-SFH IE.

a) If there is no difference of two SCD S-SFH change counts, AMS may not decode S-SFH IE in the superframe.

b) Else, AMS shall check that change bits are toggled as the value of the difference.

1)If the number of toggled bits in the last received S-SFH SP change bitmap is the same as the difference, AMS shall update the S-SFH SP IE(s) whose bit of the S-SFH SP change bitmap is toggled. Additionally, AMS shall update SCD S-SFH change count and S-SFH SP change bitmap.

2)Else, AMS shall update all S-SFH SP IEs, SCD S-SFH change count and S-SFH SP change bitmap.

During the listening interval, if an AMS doesn't receive the changed S-SFH SP IE(s), the AMS shall try to update the S-SFH SP IE(s). The AMS may only wake up at the first superframe where the S-SFH SP IE(s) will be scheduled in the sleep interval by using the valid SP scheduling periodicity information or implicit scheduling information. After performing update of the S-SFH SP IE(s), the AMS may power down one or more physical operation components or perform other activities that do not require communication with the ABS during the rest of sleep interval.

Decode P-SFH IE

Difference of SCD S-SFH CCs in the received and the stored P-SFH

No decoding of S-SFH SP IEs

== 0

!= 0

Decode and update S-SFH SP IEs for the corresponding toggled bits

Change bits are toggled as the value of the difference

Save SCD S-SFH change count and S-SFH SP change bitmap

Yes

Decode and update all S-SFH SP IEs

No

Figure 462—S-SFH IE Update

-----------------------------------------------------------End of the Text---------------------------------------------------------